| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | FDD and H-FDD frame structure for transparent RS and MR-BS |  |
| Date Submitted | 2008-11-11 |  |
| Source(s) | Gamini Senarath, Hang Zhang, Israfil Bahceci, Peiying Zhu, Derek Yu, Mark Naden Nortel Kanchei(Ken) Loa, Yi-Hsueh Tsai, Youn-Tai Lee, Institute for Information Industry (III) Yung-Ting Lee, Hua-Chiang Yin Coiler Shiann-Tsong Sheu National Central University (NCU) Yih-Guang Jan, Yang-Han Lee Tamkang University (TKU) | Voice: +613 763 5287 bahcecis@nortel.com Voice: +886-2-66000100 Fax: +886-2-66061007 loa@iii.org.tw |
| Re: | IEEE-SA Sponsor Ballot: P802.16j/D7 |  |
| Abstract | Modifications required for the FDD mode |  |
| Purpose | Text proposal for 802.16j Draft Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- FDD and H-FDD frame structure for transparent RS and MR-BS

Gamini Senarath, Israfil Bahceci, Hang Zhang, Peiying Zhu, Derek Yu, Naden Mark
Kanchei(Ken) Loa, Yi-Hsueh Tsai, Youn-Tai Lee
Yung-Ting Lee, Hua-Chiang Yin
Shiann-Tsong Sheu, Yih-Guang Jan, Yang-Han Lee

Nortel, III, Coiler, NCU, TKU

### Introduction

IEEE 802.16j/D7 supports only TDD frame structure, and therefore, only the TDD mode of operation. However, the IEEE 802.16Rev2 supports FDD operation, which includes half-duplex FDD (H-FDD) and full duplex FDD capable mobiles. The FDD and H-FDD operation has also been included as additional features in WiMAX Profile Release 1.5. Therefore, relay support for FDD operation is very desirable such that the coverage and other performance enhancement of relay systems can be extended to the FDD based 16e systems as well.

We propose FDD and H-FDD mode of operation for transparent RS.

### Transparent Relay Stations in TDD and FDD mode

For FDD operation, DL and UL frequencies are different from each other. An RS can also be designed to operate in FDD mode by using the same carrier for its DL (UL) transmission as its superordinate station does for its DL (UL). Figure 1 presents the DL/UL transmission in FDD mode.

f1

f2

f1

f2

UL

DL

BS

RS

MS

f2

f1

f2

f1

Figure 1 DL and UL transmission in FDD mode with half-duplex FDD relay.
Each hop employs the same carrier pairs in FDD.

The H-FDD frame structure in 802.16Rev2 can be extended to IEEE802.16j with minor modifications.

- The RS may obtain the UL carrier from the UCD during network entry at the first carrier frequency. The second carrier frequency signaled in RS Config CMD message can refer to DL carrier in FDD mode. In FDD mode, an additional TLV to configure the UL carrier for RS to use in communicating with its subordinate stations can be signaled.

- The RSs can operate in full duplex mode, e.g., may be allocated resources at all H-FDD available resources using HDD Paired Allocation IE. The RS can monitor both DL MAP 1 and DL MAP 2, and perform relaying according the forwarding rules. Both RS and SS can be switched between groups.

- Spec changes

[Insert the subclause in line 46 of page 206 as following indicated:]

8.4.4.8 FDD frame structure of MR-BS and RS

8.4.4.8.1 Frame Structure for transparent mode

The transparent RS may be configured to monitor and relay data for both subframe transmissions described in 8.4.4.1. The RS shall not switch between groups. For both MR-BS and RS, each DL subframe is the same as the DL subframe defined in 8.4.7.1.1 except the preamble shall not appear in the DL subframe 2 of the MR-BS. Each UL subframe is the same as the UL subframe defined in 8.4.7.1.1.