| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarify the usage of DCD and UCD in relay zone for the TTR RS |  |
| Date Submitted | 2008-11-11 |  |
| Source(s) | Kanchei(Ken) Loa, Yi-Hsueh Tsai, Youn-Tai Lee, Institute for Information Industry (III) Yung-Ting Lee, Hua-Chiang Yin Coiler Shiann-Tsong Sheu National Central University (NCU) | Voice: +886-2-66000100 Fax: +886-2-66061007 loa@iii.org.tw |
| Re: | Sponsor ballet |  |
| Abstract | REV2 Alignment and Editorial Correction |  |
| Purpose | Text proposal for 802.16j Draft Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarify the usage of DCD and UCD in relay zone for the TTR RS

Kanchei(Ken) Loa, Yi-Hsueh Tsai, Youn-Tai Lee, Yung-Ting Lee, Hua-Chiang Yin, Shiann-Tsong Sheu

III, Coiler, NCU

# Introduction

In the current standard, the DCD and UCD messages are broadcasted in the relay zone periodically. However, the periodical broadcasting DCD/UCD was designed for the SS network entry and MAC synchronization.  Because it is no access in the relay zone, instead of transmitting the DCU/UCD message in the relay zone, the MR-BS shall transmit the RCD message containing relay link and DCD/UCD information, if necessary, to the TTR RS to reduce overheads.

In order to facilitate the incorporation of this proposal into IEEE 802.16j standard, specific changes to the draft standard P802.16j/D7 are listed below.

# Spec changes

[Modified the text in line 35 of page 181 as following indicated:]

For a In relay networks with centralized scheduling mode RS, as shown in Figure 155d, after receiving an acknowledgement header from the RS corresponding to reception of the DCD/UCD, the MR-BS shall periodically allocate unsolicited bandwidth to the RS to enable it to broadcast the DCD/UCD message over the relay link and access link by using an RS_BW-Alloc_IE in the RS-Access-MAP message. To enable the RS to send the DCD/UCD message with fragmentable broadcast CID, if the RS_BW-Alloc_IE is lost, the RS shall request bandwidth by using an RS BR header after the timer T69 expires. For a In relay networks with distributed scheduling mode RS, the RS shall autonomously broadcast DCD/UCD with fragmentable broadcast CID.

[Replace the following figure in line 5 of page 182 as indicated:]

[Editor note: replace “Timer expires” by “T69 timer expires”, replace “RS BW-ALLOC_IE” by “RS BW-ALLOC_IE in RS-Access-MAP”, and replace “RS-DL MAP/DL MAP” by “DL-MAP”]

MS

DL-MAP

DCD/UCD

T69 timer expires

MR-BS

RS

RS BW-ALLOC_IE in RS-Access-MAP

send unsolicited MAP IE Periodically

DL-MAP

DCD/UCD

DL-MAP

DCD/UCD

RS BR header

RS BW-ALLOC_IE in RS-Access-MAP

RS BW-ALLOC_IE in RS-Access-MAP

RS BW-ALLOC_IE in RS-Access-MAP

Figure 155d—DCD/UCD broadcasting procedure with an RS operating in centralized scheduling mode

[Insert the following text in line 13 of page 42 as indicated:]

The RCD message may include the following TLVs for TTR RS channel description:

DCD encodings (see 11.24.8)

UCD encodings (see 11.24.8)

[Insert the subclause 11.24.8 in line 17 of page 285 as following indicated:]

11.24.8 TTR RS channel description

| Name | Type | Length | Value | Scope |
| --- | --- | --- | --- | --- |
| DCD encodings | 9 | variable | DCD TLV items for relay link | RCD |
| UCD encodings | 10 | variable | UCD TLV items for relay link | RCD |