| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Editorial modification on RS_Config-CMD message and associated TLVs |  |
| Date Submitted | 2008-11-10 |  |
| Source(s) | Kanchei(Ken) Loa, Yi-Hsueh Tsai Institute for Information Industry | Voice: +886-2-66000100 Fax: +886-2-66061007 loa@iii.org.tw |
| Re: | Sponsor ballet |  |
| Abstract | Editorial modification on RS_Config-CMD message |  |
| Purpose | Text proposal for 802.16j Draft Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Editorial modification on RS_Config-CMD message and associated TLVs

Kanchei(Ken) Loa, Yi-Hsueh Tsai

Institute for Information Industry

# Introduction

Editorially modify on “RS operational mode TLV” in order to make it consistent between the table the description text. Move “RS DL fixed forwarding delay” and “RS UL fixed forwarding delay” TLVs to a new subsection since they are associated with centralized scheduling. Move “RS second carrier configuration” TLVs to a new subsection since it is associated with STR RS. Merge the CID_ALLOC-REQ message into the RS_Config-CMD message since some of the configurations are duplicated. Modify TLV encoding for REG-REQ message since some feature support are configuring by RS_Config-CMD not REG-RSP

# Spec changes

- [Editor Note: modify on “RS operational mode TLV”]

[Modify the table in 11.25.1 as following indicated:]

| RS operational mode | 1 | 2 | Bit #0: RS scheduling mode, Bit#0=0, centralized scheduling, Bit #0=1, distributed scheduling Bit #1: RS security mode, Bit #1=0, centralized security, Bit #1=1, distributed security Bit #2:0 = shared BSID with other access stations, 1 = unique BSID setting Bit #3: Embedded path management Bit #4: Explicit path management Bit #5: Burst-based forwarding Reserved Bit #6: Tunnel mode Bit #7: Local CID allocation mode Bit #8: Superordinate RS of an RS group Bit #9: Use a different carrier frequency for subordinate station communication at the RS Bit #10: Operate in STR relaying mode Bit #11: Burst-based forwarding Bit #1112: The type of RS group members served by the superordinate station The superordinate station of an RS group serves non-transparent RS members (Bit#12 = 0) or transparent RS members (Bit #12=1). Bit #1213: Frame configuration setting 0 = the MR-BS configures the frame configuration of the RS using the Frame configuration TLV in RCD, 1 = the MR-BS provides the frame configuration of the superordiante station using Frame configuration TLV in RCD. Bit#13 can be set to ‘1’ only when STR relay Bit #1314-15: Reserved | RS_Config-CMD |
| --- | --- | --- | --- | --- |

[Modify the text in line 23 of page 287 as following indicated:]

RS operational mode

When Bit#0 is set to 0, the RS shall operate in centralized scheduling. When Bit#0 is set to 1, the RS shall operate in distributed scheduling.
When Bit#1 is set to 0, the RS shall operate in centralized security. When Bit#1 is set to 1, the RS shall operate in distributed security.
When Bit#2 is set to 0, the RS shall share BSID with other access stations. When Bit#1 is set to 1, the RS shall use unique BSID.
When Bit#3~Bit#7 is set to 0, the RS shall not use the indicated capability. When Bit#3~Bit#7 is set to 1, the RS shall use the indicated capability.
When Bit#8 is set to 0, the RS is not the superordinate RS of an RS group. When Bit#8 is set to 1, the RS is the superordinate RS of an RS group.
When Bit#9 is set to 0, the RS shall use the same carrier frequency for transmission and reception to and from both the superordinate and subordinate station(s). When Bit#9 is set to 1, the RS shall use a different carrier frequency for transmission and reception to and from the subordinate station(s) to that used to transmit and receive to and from the superordinate station.
Bit#10 can be set to either 0 or 1 to indicate that the RS shall operate in either TTR or STR mode respectively. 
When Bit# 10 is set to 1, the interference induced by the transmitter operating in STR mode shall not cause any link adaptation degradation of the link performance of the related STR receiver.
When Bit#11 is set to 0, the superordinate station of an RS group serves non-transparent RS members. When Bit#10 is set to 1, the superordinate station of an RS group serves transparent RS members.
When Bit#12 is set to 0, the MR-BS configures the frame configuration of the RS using the Frame configuration TLV in RCD. When Bit#12 is set to 1, the MR-BS provides the frame configuration of the superordiante station using Frame configuration TLV in RCD. Bit#12 can be set to ‘1’ only when STR relay

- [Editor Note: move “RS DL fixed forwarding delay” and “RS UL fixed forwarding delay” TLVs to a new subsection]

[Insert the following text in line 54 of page 52 as indicated:]

The RS_Config-CMD message may include the following TLV for non-transparent RS operating in centralized scheduling mode:

RS DL fixed forwarding delay (see 11.25.8)

RS UL fixed forwarding delay (see 11.25.8)

[Insert the following subclause as indicated]

11.25.8 Non-transparent RS operating in centralized scheduling mode

[Move all text in line 16 to line 21 of page 287 to here as following indicated:]

| RS DL fixed forwarding delay | 11 | 1 | Fixed forwarding delay to be used by RS on DL (unit: frame) | RS_Config-CMD |
| --- | --- | --- | --- | --- |
| RS UL fixed forwarding delay | 12 | 1 | Fixed forwarding delay to be used by RS on UL (unit: frame) | RS_Config-CMD |

[Move all text in line 50 of page 287 to line 3 of page 288 to here as following indicated:]

RS DL fixed forwarding delay

The RS DL fixed forwarding delay shall be used by an RS in centralized scheduling mode to receive and forward MAC PDUs. RS shall forward each MAC PDU to the MS or sub-ordinate RS at the frame determined by receiving frame of the MAC PDU plus RS DL fixed forwarding delay.

RS UL fixed forwarding delay

The RS UL fixed forwarding delay shall be used by an RS in centralized scheduling mode to receive and forward MAC PDUs. The RS shall forward each MAC PDU to the MR-BR or superordinate RS at the frame determined by receiving frame of the MAC PDU plus RS UL fixed forwarding delay.

- [Editor Note: move “RS second carrier configuration” TLVs to a new subsection]

[Insert the text in line 54 of page 52 as following indicated:]

The RS config CMD message may include the following TLV for STR RS configuration:

[Move all text in line 3 to line 4 of page 52 to here and modify it as following indicated]

RS second carrier configuration for TDD mode (see 11.25.9)

[Insert the following subclause as indicated]

11.25.9 STR RS configuration

[Move all text in line 50 to line 56 of page 286 to here and modify it as following indicated:]

| RS second carrier configuration for TDD mode | 8 | 6 | Bits #0-23: Centre frequency (kHz) Bits #24-38: Bandwidth (kHz) Bits #39-40: FFT size (0b00: 128FFT; 0b01: 512FFT; 0b10: 1024FFT; 0b11: 2048 FFT) Bits #41-48: reserved | RS_Config-CMD |
| --- | --- | --- | --- | --- |

- [Editor Note: merge the CID_ALLOC-REQ message into the RS_Config-CMD message]

[Modify the following text in line 25 of page 52 as indicated]

The RS_Config-CMD message may include the following TLVs for a non-transparent RS operating in local CID allocation mode:

Contiguous CID allocation (see 11.25.4)

Bit partition CID allocation (see 11.25.4)

[Modify the table in 11.25.4 as following indicated]

| Contiguous CID allocation | 16 | 1variable | Compound TLV (non-systematic method) | RS_Config-CMD |
| --- | --- | --- | --- | --- |
| Basic CID | 16.1 | 4 | Bit 0~15:Starting point of the CID number Bit 16~31:End point of the CID number | RS_Config-CMD |
| Primary management CID | 16.2 | 4 | Bit 0~15:Starting point of the CID number Bit 16~31:End point of the CID number | RS_Config-CMD |
| Secondary management CID | 16.3 | 4 | Bit 0~15:Starting point of the CID number Bit 16~31:End point of the CID number | RS_Config-CMD |
| Transport CID | 16.4 | 4 | Bit 0~15:Starting point of the CID number Bit 16~31:End point of the CID number | RS_Config-CMD |
| T-CID | 16.5 | 4 | Bit 0~15:Starting point of the CID number Bit 16~31:End point of the CID number | RS_Config-CMD |
| MT-CID | 16.6 | 5 | Bit 0~15:Starting point of the CID number Bit 16~31:End point of the CID number | RS_Config-CMD |
| Bit partition CID allocation | 25 | variable | Compound TLV (systematic method) | RS_Config-CMD |
| New CID for the RS | 25.1 | 2 | - | RS_Config-CMD |
| Hop count | 25.2 | 1 | The new hop count of the RS to the MR-BS | RS_Config-CMD |
| K_Code | 25.3 | 1 | The maximum number of direct subordinate RSs is 2 to the power of K_Code (2K_Code). | RS_Config-CMD |

[Modify the following text in line 10 of page 175 as indicated]

When the network topology changes, the MR-BS can inform related RSs of updated value of k and n in the CID_ALLOC-REQ RS-Config-CMD message (6.3.2.3.636.3.2.3.64).

[Delete the following subclause as indicated]

6.3.2.3.63 RS CID Allocation Request (CID_ALLOC-REQ) message

[Delete the following text in line 65 of page 72 as indicated]

CID_ALLOC-REQ

[Delete the following text in line 45 of page 249 as indicated]

CID_ALLOC-REQ

[Delete the following text in line 43 of page 250 as indicated]

CID_ALLOC-REQ

- [Editor Note: modify TLV encoding for REG-REQ message since some feature support are configuring by RS_Config-CMD not REG-RSP]

11.7.24 MR-BS and RS MAC feature support

[Insert the following text and table as indicated:]

The follwoingTLV indicates the RS MAC features supported by the RS. The scheduling mode, security mode, local CID allocation mode, path management mode, tunnel mode, ARQ mode, superordinate RS of an RS group are set by the RS_Config-CMD message.

| 50 | 2 | Bit #0: Centralized scheduling mode support Bit #1: Distributed scheduling mode support Bit #2: Centralized security support Bit #3: Distributed security support Bit #4: Local CID allocation support Bit #5: Embedded path management support Bit #6: Explicit path management support Bit #7: Tunnel mode support Bit #8: End-to-end ARQ support Bit #9: Two-link ARQ support Bit #10: Hop-by-hop ARQ support Bit #11: Superordinate RS of an RS group support Bits #12-157: Reserved | REG-REQ |
| --- | --- | --- | --- |

This TLV indicates the MR system features supported by the RS and the MR-BS.

[Modify the table in 11.7.24 as following indicated:]

| 49 | 23 | Bit #0: NBR-ADV generating support Bit #1: Tunnel mode support Bit #2: Superordinate RS of an RS group support Bit #13: RS mobility support Bit #24: Subordinate RS network entry support Bit #35: Location support Bit #46: Multicast management support Bit #57: DL flow control Bit #8: RS centralized security support Bit #9: RS distributed security support Bit #10: Embedded path management support Bit #11: Explicit path management support Bit #12: Local CID allocation support Bit #613: MOB_SLP-RSP support Bit #714: MOB_SCN-RSP support Bit #815: DSx support Bit #16: End-to-end ARQ support Bit #17: Two-link ARQ support Bit #18: Hop-by-hop ARQ support Bit #19-1523 : Reserved | REG-REQ REG-RSP |
| --- | --- | --- | --- |

The RS indicates which features it can support by using this TLV and setting the appropriate bits to 1. The MR-BS uses this TLV to indicate which of the RSs capabilities may be used and thus sets the appropriate mode of operation at the RS. An RS can set any combination of bits in the REG-REQ message.

An MR-BS can only set either bit #1or bit #2 to indicate which mode it is currently supporting. If bit #13 is set to 1, then the RS can be an MRS. If bit #4 is set to 0, the RS cannot be an intermediate RS.

An RS sets bit #57 to 0 to indicate that it cannot perform DL flow control and to 1 to indicate that it can perform DL flow control. The MR-BS sets bit #57 to 0 to indicate that DL flow control shall not be performed and to 1 to indicate that DL flow control shall be performed.

The MR-BS shall only set either bit #8 or bit #9 to indicate the security mode of operation that the RS shall use.

Bit #313 is only applicable for distributed security mode (i.e. if bit #9 is set to 1). If bit #613 is set to 1, the RS gets MS’s sleep information from the MOB_SLP-RSP message, otherwise it gets it from a MR_SLP-INFO message.

Bit #14 is only applicable for distributed security mode (i.e. if bit #9 is set to 1). If bit #14 is set to 1, the RS gets scan information from the MOB_SCN-RSP message, otherwise it gets it from a MS_SCN-INF message.

Bit #15 is only applicable for distributed security mode (i.e. if bit #9 is set to 1). If bit #15 is set to 1, the RS in a two-hop system directly gets SF parameters from the DSx messages exchanged between the MR-BS and an SS.

[Delete 11.7.24.1 as following indicate:]

11.7.24.1 RS MAC feature support