| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modification on MAC messages for relay FDD and H-FDD operations |  |
| Date Submitted | 2008-11-07 |  |
| Source(s) | Kanchei(Ken) Loa, Yi-Hsueh Tsai Institute for Information Industry (III) Gamini Senarath, Hang Zhang, Israfil Bahceci, Peiying Zhu, Derek Yu, Mark Naden Nortel | Voice: +886-2-66000100 Fax: +886-2-66061007 loa@iii.org.tw Voice: +613 763 5287 bahcecis@nortel.com |
| Re: | IEEE-SA Sponsor Ballot: P802.16j/D7 |  |
| Abstract | Modifications required for the FDD mode |  |
| Purpose | Text proposal for 802.16j Draft Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Modification on MAC messages for relay FDD and H-FDD operations

Kanchei(Ken) Loa, Yi-Hsueh Tsai
Gamini Senarath, Israfil Bahceci, Hang Zhang, Peiying Zhu, Derek Yu, Naden Mark

III, Nortel

- Introduction

In order to support H-FDD, some MAC messages related to the centralized scheduling shall include an H-FDD group indicator to indicate the message is associated to the group 1 or the group 2.

- Spec changes

6.3.2.3.6 RNG-RSP (ranging response) message

Table 44—RNG-RSP message format

| H-FDD Group Indicator | 1 bits | 0: Group-1, 1: Group-2 |
| --- | --- | --- |
| Reserved | 87 bits | Shall be set to zero. |

6.3.2.3.62 MR_Ranging-REP message

[Modify the following table as indicated:]

Table 165c—MR Ranging report (MR_RNG-REP) message format

| Size of Remaining CDMA codes | 76 bits | Amount of uplink bandwidth in bytes for CDMA codes remaining at the RS ready to be forwarded |
| --- | --- | --- |
| H-FDD Group Indicator | 1 bits | 0: Group-1, 1: Group-2 |
| Frame Number Index | 8 bits | LSBs of relevant frame number |

6.3.2.3.82 RS access MAP (RS_Access-MAP) message

[Modify the following table as indicated:]

Table 165y—RS Access MAP message format

| Indicator | 812 bits | Bit 0: 0: Parameters of DL_Frame_Prefix remain same with the latest Configuration. 1: The parameters of DL_Frame_Prefix are updated. Bit 1: 0: RS shall use Normal map format, 1: RS shall use Compressed map format Bit 2: 0: DL-MAP not included 1: DL-MAP included Bit 3: 0: UL-MAP not included 1: UL-MAP included Bit 4: 0: SUB-DL-UL-MAP not included 1: SUB-DL-UL-MAP included Bit 5: 0: HARQ-MAP not included 1: HARQ-MAP included Bit 6: 0: RS BW-ALLOC IE not included 1: RS BW-ALLOC IE included Bit 7: 0: DL_Transmit_Reference_IE not included. 1: DL_Transmit_Reference_IE included Bit 8: H-FDD MAP Indicator 0: MAP1 (Group-1), 1: MAP2 (Group-2) Bit 9~11: reserved |
| --- | --- | --- |
| Frame number | 84 bits | 84 LSB of the frame number in which the corresponding message(s) shall be transmitted by the RS on the access link |

6.3.2.3.83 RS relay MAP (RS_Relay-MAP) message

[Modify the following table as indicated:]

Table 165ae—RS relay MAP message format

| Unsolicited DL BW Indication | 1 bit | Unsolicited DL bandwidth allocated in the R-MAP for sending RCD message(0: not includ, 1: include) |
| --- | --- | --- |
| H-FDD R-MAP Indicator | 1 bit | 0: MAP1 (Group-1), 1: MAP2 (Group-2) |
| reserved | 1 bit | Shall be zero |
| If (R_Zone_Prefix Change Indication == 0b1) { | - | - |