| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comment on Location information request and response messages |  |
| Date Submitted | 2008-10-31 |  |
| Source(s) | Kanchei(Ken) Loa, Yi-Hsueh Tsai Institute for Information Industry (III) [Add other coauthor here] | Voice: +886-2-66000100 Fax: +886-2-66061007 loa@iii.org.tw, lucas@iii.org.tw |
| Re: | IEEE-SA Sponsor Ballot: P802.16j/D7 |  |
| Abstract | Modifications required for location information request and response messages |  |
| Purpose | Text proposal for 802.16j Draft Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Comment on Location information request and response messages

Kanchei(Ken) Loa, Yi-Hsueh Tsai

Institute for Information Industry (III)

- Introduction

In IEEE P802.16j/D7, the original LLA_IE() had been replaced by the Position TLV which is defined in P802.16REV2/D7 for LBS-ADV message. As a result, the location information message uses a mixed format consisting of the bit-map and the TLV. This format is complex, unclear and difficult to implement. This contribution proposes to replace the mixed format with a pure TLV format in order to simplify the message format.

- Spec changes

[Replace Table 165g in line 20 of page 54 by the following table as indicated:]

| Syntax | Size | Notes |
| --- | --- | --- |
| MR_LOC-REQ_ Message_Format() { | - | - |
| Management Message Type = 76 | 8 bits | - |
| Report Mode | 2 bits | 0b00: Once 0b01: Periodic report 0b10: End periodic reporting (no LBS-RSP message shall be transmitted in this case) 0b11: reserved |
| Reserved | 6 bits | Shall be zero |
| TLV Encoded Information | variable | TLV specific |
| } | - | - |

[Insert the text in line 24 of page 55 as following indicated:]

When the Report Mode is periodic report, the MR_LOC-REQ message shall contain the following TLV.

Report Period TLV (see 11.27.3)

When the MR_LOC-REQ message is transmitted by an RS to the MR-BS, it shall contain the following TLV.

Neighbor Station BSID TLV (see 11.27.4)

[Replace Table 165h in line 5 of page 56 by the following table as indicated:]

| Syntax | Size | Notes |
| --- | --- | --- |
| MR_LOC-RSP_ Message_Format() { | - | - |
| Management Message Type = 77 | 8 bits | - |
| TLV Encoded Information | variable | TLV specific |
| } | - | - |

[Insert the text in line 45 of page 56 as following indicated:]

When the MR_LOC-RSP message is transmitted by an RS to the MR-BS, it shall contain one of the following TLVs.

Absolute Position (Long Format) TLV (see 11.27.1)

Absolute Position (Short Format) TLV (see 11.27.1)

Relative Position TLV (see 11.27.1)

When the MR_LOC-RSP message is transmitted by the MR-BS to an RS, it shall contain one of the following TLVs.

Neighbor Station BSID and Absolute Position (Long Format) (see 11.27.2)

Neighbor Station BSID and Absolute Position (short Format) (see 11.27.2)

Neighbor Station BSID and Relative Position (see 11.27.2)

[Insert the text in line 59 of page 293 as following indicated:]

11.27 Location information request and response (MR_LOC-REQ/RSP) messages encodings

11.27.1 The position of the RS

This TLV is used to provide the position of the RS.

| Name | Type | Length (bytes) | Value |
| --- | --- | --- | --- |
| Absolute Position (Long Format) | 1 | 15 | See Table 609 |
| Absolute Position (Short Format) | 2 | 6 or 8 | See Table 610 |
| Relative Position | 3 | 4 or 6 | See Table 611 |

11.27.2 Positions of neighbor stations

These TLVs are used to provide BSIDs or positions of neighbor stations (BSs, MR-BSs as well as RS).

| Name | Type | Length (bytes) | Value |
| --- | --- | --- | --- |
| Neighbor Station BSID and Absolute Position (Long Format) | 6 | 21* Nr[Note] | For (i=0;i<Nr;i++) { 48-bit BSID of the neighboring station 120-bit Absolute Position defined (long format) in Table 609 } |
| Neighbor Station BSID and Absolute Position (Short Format) | 7 | 14* Nr[Note] | For (i=0;i<Nr;i++) { 48-bit BSID of the neighboring station 64-bit Absolute Position defined (short format) in Table 610 } |
| Neighbor Station BSID and Relative Position | 8 | 12* Nr[Note] | For (i=0;i<Nr;i++) { 48-bit BSID of the neighboring station 48-bit Relative Position defined in Table 611 } |

[Note] Number of neighboring stations whose BSIDs and locations are included

11.27.3 Report Period

This TLV is used to provide the report period for periodic reporting mode.

| Name | Type | Length (bytes) | Value |
| --- | --- | --- | --- |
| Report Period | 9 | 2 | Report period in units of frame, a value from 0 to 4095 corresponding to a range of 1 frame to 4096 frame. |

11.27.4 BSIDs of neighbor stations

This TLV is used to provide BSIDs of neighbor stations (BSs, MR-BSs as well as RS).

| Name | Type | Length (bytes) | Value |
| --- | --- | --- | --- |
| Neighbor Station BSID | 10 | 6* Nr[Note] | For (i=0;i<Nr;i++) { 48-bit BSID of the neighboring station } |

[Note] Number of neighboring stations whose BSIDs are included