| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comment on DL Burst Transmit IE |  |
| Date Submitted | 2008-11-10 |  |
| Source(s) | Kanchei (Ken) Loa, Yi-Hsueh Tsai, Youn-Tai Lee, Chun-Yen Hsu Institute for Information Industry (III) Hua-Chiang Yin, Yung-Ting Lee Coiler Shiann-Tsong Sheu National Central University (NCU) | Voice: +886-2-66000100 Fax: +886-2-66061007 loa@iii.org.tw hcyin@coiler.com.tw stsheu@ce.ncu.edu.tw |
| Re: | IEEE-SA Sponsor Ballot: P802.16j/D7 |  |
| Abstract | Comment on DL Burst Transmit IE |  |
| Purpose | Text proposal for 802.16j Draft Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comment on DL Burst Transmit IE

Kanchei (Ken) Loa, Yi-Hsueh Tsai, Youn-Tai Lee, Chun-Yen Hsu
III

Hua-Chiang Yin, Yung-Ting Lee
Coiler

Shiann-Tsong Sheu
NCU

Introduction

As defined in current draft 16j/D7, for transparent RS, the MR-BS schedules the downlink access bandwidth in DL-MAP message for its subordinate RS and notifies the subordinate RS the allocated burst by DL Burst Transmit IE in R-MAP message for relaying bursts. To handle this task, the RS has to be capable of handling the inconsistency between the received DL-MAP and R-MAP messages, if any.

In order to simplify the procedure, we suggest moving the DL Burst Transmit IE from the R-MAP message to the DL-MAP message. As a result, the subordinate RS could find the designated burst to relay the traffic data by processing the DL-MAP message only.

In order to facilitate the incorporation of this proposal into IEEE 802.16j standard, specific changes to the draft standard P802.16j/D7 are listed below.

Spec changes

[Modify Table 320a as following indicated:]

Table 320a—Extended-3 DIUC code assignment for Extended-2 DIUC=15

| Extended-3 DIUC (hexadecimal) | Usage |
| --- | --- |
| 00 | Power Boosting IE |
| 01 | MR DL-MAP Monitor IE |
| 02 | DL Burst Transmit IE |
| 0123-0F | Reserved |

[Change the subclause 8.4.5.10.1.14 to 8.4.5.3.32 as following indicated:]

8.4.5.3.32 8.4.5.10.1.14 DL Burst Transmit IE

Table 335b 460p—DL Burst Transmit IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| DL_Burst_Transmit_IE{ | - | - |
| Extended-2 DIUC Type | 4 bits | 0x0F (Extended-3 DIUC) DL_ Burst_Transmit_IE = 0x0D |
| Length | 8 bits |  |
| Extended-3 DIUC | 4 bits | DL_Burst_Transmit_IE = 0x02 |
| If ( included in SUB-DL-UL-MAP) { | - | - |
| RCID_IE() | variable | Reduced RS basic CID |
| } else { | - | - |
| CID | 16 bits | RS basic CID |
| } |  |  |
| } |  |  |
| Ns | 8 bits | The first DL-MAP IE number in DL-MAP the transparent RS shall forward |
| Nr | 8 bits | Indicate next Nr number of DL-MAP IEs following this DL_Burst_Transmit_IE, the transparent RS shall forward Indicate the number of MAP IEs in DL-MAP the transparent RS shall forward |
| for(k=1; k<Num_CID=Nr;k++){ | - | - |
| Lk | 16 bits | Burst length in bytes to be forwarded |
| } |  |  |
| Padding to byte alignment | variable | Shall be set to 0 |
| } |  |  |

[Modify Table 460c as following indicated:]

Table 460c—R-link specific IE types

| Type (hexadecimal | Usage |
| --- | --- |
| 0D | DL Burst Transmit IE |
| 0ED-1F | Reserved |