| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Correct ARQ mechanism and state machine in hop-by-hop mode |  |
| Date Submitted | 2008-10-29 |  |
| Source(s) | Avner Aloush DesignArt Networks | Voice: +972-54-4225583 Fax: +972-9-7421296 avnera@designartnetworks.com |
| Re: | IEEE-SA Sponsor Ballot: P802.16j/D6a |  |
| Abstract | Comment on Correct ARQ mechanism and state machine in hop-by-hop mode |  |
| Purpose | Text proposal for 802.16j Draft Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Correct ARQ mechanism and state machine in hop-by-hop mode

Avner Aloush 
DesignArt Networks

Introduction

As defined in current draft 16j/D7, for non transparent mode in distributed scheduler in ARQ Hop-by-hop mode the current mechanism constrains a waiting time when MRBS should wait to the MS ACK. Since the link is managed hop by hop there is no point that the MRBS waits for an ACK from the MS relayed by the intermediate RSs to clear buffers in progress in the TX Window only after receiving the ACK from the MS.

In order to facilitate the incorporation of this proposal into IEEE 802.16j standard, specific changes to the draft standard P802.16j/D7 are listed below.

Spec changes

Page 92 line 23

In downlink ARQ operation, when MR-BS/super-ordinate RS sends TDU to the subordinate RS, it waits for ARQ feedback IE for tunnel packets. When TDU is corrupted in the relay link, the subordinate RS sends back R-NAK, and the MR-BS/super-ordinate RS schedules the retransmission of TDU to the subordinate RS. When the MR-BS receives RACK from the subordinate RS, it waits for an ACK from the MS relayed by the intermediate RSs. Access RS may modify the ARQ feedback IE received from MS to inform only ACK to MR-BS. When MR-BS receives RACK from MRS, it clears the buffer corresponding to ARQ blocks. When superordinate RS receives R-ACK from the sub-ordinate RS, it clears the buffer corresponding to TDU.

Page 92 line 55

The ARQ state machine operation in access RS and receiver in MR-BS is the same as described in 6.3.4.6.2 and

6.3.4.6.3. In case of transmitter state machine in MR-BS, an ARQ block or TDU may be in one of the following four five states - not sent, outstanding for R-ACK, outstanding for MS-ACK, waiting for retransmission, and data discard. Outstanding for R-ACK is the state waiting for receiving acknowledged from access RS. When R-ACK received, the state transits to done. outstanding for MS-ACK. In this state, MR-BS receives MS-NACK or after ARQ_BLOCK LIFE_TIME, the state transits to discard. If MR-BS receives MS-ACK in the state of outstanding for R-ACK or waiting for retransmission, the state transits to done. Other state transition descriptions are same as transmitter state machine defined in 6.3.4.6.2.

Figure 55a—ARQ Tx block state in MR-BS