| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on MRS handover |  |
| Date Submitted | 2008-11-7 |  |
| Source(s) | Yang Liu, Hongyun Qu, Yuqin Chen Ling Xu, Sean Cai ZTE Corporation 706/4, Liantang Pengji Industrial Park, Shenzhen, P.R.China 518004 | E-mail: [liu.yang8@zte.com.cn] |
| Re: | This is in response to call for comments on P802.16j/D7 |  |
| Abstract | This contribution proposes a further text update for RS handover procedure. |  |
| Purpose | To review and adopt. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on the MRS Handover

Yang Liu, Hongyun Qu, Yuqin Chen, Ling Xu, Sean Cai

ZTE Corporation

# Introduction

Currently, in P802.16j/D7, for MRS handover with preamble change, the MRS has to relay the hand off signaling for its attached MSs first. After all the MSs leave the MRS, the MRS will start its own handover. For fixed RS, the MR_BS may inform the RS to discard the MS context information with MS_INFO-DEL when the MS is attached to the new access station. However, if the MRS performs handover itself, the RS may not receive the message and therefore keeps the MS context information.

Accordingly, when the MRS’s handover begins, the MRS should discard the MS context information unsolicitedly.

# Proposed Changes

[Add a sentence in 6.3.21.4.2 on Page 187, line 27 as indicated:]

6.3.21.4.2 MRS handover with preamble change (Inter MR-BS)

The serving MR-BS exchanges handover decision and initiation stage signaling (6.3.21.2.2) with each MS

before the MRS conducts handover and preamble change. The MOB_BSHO-REQ message is sent to the

subordinate MSs with the “HO operation mode” set to 1. The operation of MS receiving the MOB_BSHOREQ

follows the procedures in 6.3.21.2. After the MRS transmits MOB_HO-IND to start its own HO, the MRS may unsolicitedly discard the remained MS context information.