| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on relay types |  |
| Date Submitted | 2008-11-07 |  |
| Source(s) | Gamini Senarath, et al. 3500 Carling Ave Nepean, ON, Canada Nortel Networks | Voice: +613-763-5972 gamini@nortel.com |
| Re: | Sponsor ballet |  |
| Abstract | This document is to clarify various relaying configurations to facilitate easy understanding for the reader |  |
| Purpose | Text proposal for 802.16j D7 Draft Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Types of Relay Stations and Profiles

Hang Zhang, Mo-Han Fong, G.Q. Wang, Peiying Zhu, Wen Tong,  David Steer, Gamini Senarath, Derek Yu, Mark Naden,

Nortel

# Introduction

The objective of this contribution is to clarify various types of RSs and MR-BS systems so that a reader can locate the descriptions about the various components of the flow of operation.

This needs to be completed if the TG agreed that this information is important for better readability of the document.

# Proposed text change

[Insert the following new section]

[Insert section 12.5 as follows]

12. 5 MR system profiles

Depending on the RS capability, the MR system can support several relaying model options. They are different from each other based on:

- Whether different hops operate on different carriers or the same carrier

- Whether they use STR relaying system (where relays can receive from the super-ordinate station and transmit to the subordinate station (and vice versa) at the same time) or TTR relaying system (where the transmission/reception should be time separated)

- Whether the system use FDD, H-FDD or TDD modes of operation

{Note to Editor: The last bullet should be changed to “This system supports only TDD mode of operation “ if the FDD contributions submitted to meeting #58 are not approved}

In addition, there are two key RS types based on how an RS as appear to an MS:

- a transparent RS which does not transmit the same preamble, FCH and MAP and

- a non-transparent RS which transmits a preamble, FCH and MAP

Transparent RS cannot operate as an STR RS while non-transparent RS can be either a STR or TTR RS.

A slight variant of a non-transparent RS also exist which transmit the same preamble, FCH and MAP as its supeordinate station thus sharing the same identity as the superordinate station.

RSs can also belong to a RS group where the group members share the identity of the superordinate station of the RS group. This can be done using transparent RSs as RS group members Transparent RS group) or non-transparent RSs as RS group members (Non-transpaent RS group).

The operation of different types of RSs and different modes are described in various sections. The main components of these operations are described below in Table x.

Section numbers are TBD

Table x RS types and profiles.

| # Option/ feature |  | # Transparent RS # (include transparent RS as a RS group member) | # STR Non-transparent RS having a unique BSID | # TTR Non-transparent RS |
| --- | --- | --- | --- | --- |
| # Frame Configuration |  | # Section x.x | # Section x.x | # Section x.x |
| # RS properties | # BSID | # No | # Yes | # No (Same as the super-ordinate station) |
|  | # Basic CID/Primary CID | # Yes | # Yes | # Yes |
|  | # Secondary CID | # Yes | # Yes | # Yes |
|  | # Tunnel CID | # No | # Yes for tunnel based forwarding | # No |
|  | # RS <-> MR-BS security | # Yes | # Yes | # Yes |
| # MS connection and security and service flow management | # MS connection management | # No | # No | # No |
|  | # MS security management | # No # (Centralized secusrity) | # Yes # (Distributed security) | # No |
|  | # MS service flow management | # No | # No | # No |
| # R-link functions | # Tunnel based forwarding | # No | # Yes | # No |
|  | # CID based forwarding | # Yes | # Yes | # Yes |
|  | # R-amble Tx/Rx | # No | # Yes | # Yes |
|  | # Preamble/MAP transmission | # No | # Yes | # Yes (Same as the super-ordinate station) |
|  | # MS control message/control header process | # No | # Yes | # No |
|  | # MS traffic scheduling (DL/UL) | # No (Scheduling is centralised and done by the super-ordinate station) | # Yes (for distributed scheduling) # (For centralsed scheduling this is done by MR-BS) # For an RS group, this is done by the superordinate station of the RS group. | # No |
|  | # MS MAC state management | # No | # No | # No |

.