| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Reliability Enhancement With Network Coding for IEEE 802.16n Network |  |
| Date Submitted | 2010-05-05 |  |
| Source(s) | Hung-Yu, Wei Ching-Chun, Chou National Taiwan University Chun-Yen, Wang Yan-Xiu Zheng ITRI | E-mail: hywei@cc.ee.ntu.edu.tw *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Reply to IEEE 802.16gman-10/0022r1, "Call for Contributions the P802.16n System Requirements Document" |  |
| Abstract | This contribution proposes network coding based relay scheme to enhance reliability for IEEE 802.16n. |  |
| Purpose | For discussion and approval by GRIDMAN study group |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Reliability Enhancement With Network Coding for IEEE 802.16n Network

Hung-Yu Wei, Ching-Chun Chou

National Taiwan University

Chun-Yen Wang, Yan-Xiu Zheng

ITRI

# 1. Introduction

The IEEE 802.16n addresses higher reliability requirements that are not supported by IEEE Standard 802.16 presently [1]. Regarding data reliability, network coding is a popular scheme to enhance the reception robustness and reduce link budget. Network coding has also been shown to enhance transmission in relay networks [2]. Network coding based transmission over multi-path routes could enhance service reliability with diversity gain [3]. Network coding mechanism has been shown to improve both unicast traffic and multicast traffic performance in multihop relay network.

Transmission reliability could be improved for unicast/multicast/broadcast flows in various relay scenarios (fixed relay, mobile relay, mobile BS, etc). Especially, in the usage cases where mobile base stations and mobile relay stations are deployed, the higher reliability requirements could be addressed with both radio path redundancy and network coding redundancy.

As a consequence, we propose to include a coding functionality in MAC layer under IEEE 802.16n scope. The functionality enhances both reliability and required link budget. Network coding is a popular candidate to achieve both objectives.

# 2. Proposed Text

<Add the following text into the SRD>

IEEE 802.16n should enhance the transmission reliability for relay network with fixed relay stations and mobile relay stations. MAC layer coding should be supported to enhance radio transmission redundancy in relay network, e.g. network coding.

# References

[1] IEEE C802.16gman-10/0022r1, "Call for Contributions the P802.16n System Requirements Documents," 2010

[2] Sachin Katti, Hariharan Rahul, Wenjun Hu, Dina Katabi, Muriel Medard, and Jon Crowcroft "XORs In The Air: Practical Wireless Network Coding," ACM SIGCOMM, 2006.

[3] Szymon Chachulski, Michael Jennings, Sachin Katti, and Dina Katabi, "Trading Structure for Randomness in Wireless Opportunistic Routing,"  ACM SIGCOMM, 2007.