| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text proposal of 802.16n System Requirements Document (Section 6.2) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Kyungkyu Kim, Jungje Son, Youngbin Chang Samsung Electronics Co., Ltd | Email: kkei.kim@samsung.com jungje.son@samsung.com yb.chang@samsung.com |
| Re: | Call for Contributions for P802.16n System Requirements Document / Area: Section 6.2 |  |
| Abstract | The contribution proposes the change of current 802.16n SRD (C80216gman-10_0022.doc) |  |
| Purpose | To be discussed and adopted by 802.16n SRD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text proposal of 802.16n System Requirements Document

Kyungkyu Kim, Jungje Son, Youngbin Chang

Samsung Electronics

# Introduction

According to IEEE 802.16n PAR and 5C [1], The PPDR (Public protection/Disaster Relief) solution is explicitly considered as one of major issues to be discussed. Generally, PPDR solution has been adopted as the main issue for Police, Fire and Medical area (i.e. Communication between police officer/fireman, Accident report (Criminal/medical) system and Emergency message broadcasting etc.).

LMR (Land Mobile Radio) system has been broadly used as PPDR solution, one of the LMR supporting standards had been developed by the project Terrestrial Trunked Radio (TETRA) technical body of the ETSI (Europe) [2].  However, the recent PPDR system requires broadband system to support real-time information such as video data and location information etc.

In Korea, Police, Fire and Medical area want to use broadband PPDR system for supporting LMR multimedia transmission. The specific service requirement of PPDR can be shown as following Table 1.

Table 1 - the specific service requirement of PPDR

| Service requirement |  | Note |
| --- | --- | --- |
| 1 | Individual/Group Call | Unique address for Individual /Group call Dynamic Group control |
| 2 | Priority Call | Each call has own priority Emergency call has highest priority (ignore other calls) Pre-emption call Call intercepting/waiting |
| 3 | Data communication | Short Data communication: -Less than 140byte -Short message service (SMS) -Piggybacking available |
|  |  | Packet Data Communication: -Greater than 140byte -Multimedia data (e.g. Video) -Adopt QoS level |
| 4 | Reliability and Robustness | Direct communication between MS Standalone BS mode Multi-hop Relay Solve Shadow zone problem (e.g. Underground) |
| 5 | Support expanded user in a group | Support over 10,000 people in one group* Multicasting/Broadcasting (*note: Group means logical area where it covers one or more BS) |

Considering above facts, IEEE 802.16n need to support both legacy LMR and broadband features. In other word, IEEE 802.16n should take over the LMR’s requirement and additionally meet the broadband related demand. Only regarding service aspect, “Reliability and Robustness” can be excluded because it closes to “Construction and Maintenance of Network”. Therefore, Essential requirements which satisfy above condition can be extracted as follow:

- Group Call

- Multicasting/Broadcasting

- VoIP Service (include priority call)

- Data Communication

We propose several requirements related to service provided on network including above essential requirements.

# Text Proposal

Note:

The text in BLACK color: the existing text in the 802.16n SRD Text

The text in RED color: the removal of existing 802.16n SRD Text

The text in BLUE color: the new text added to the 802.16n SRD Text

[Add the new text section 6.2 on page 6 as following:]

--------------------------------Start of the Proposed Text ----------------------------------------------------------------------

6.2. Requirements related to Services provided on network

In this section, descriptions can be adopted when 802.16n system should support PPDR service.

6.2.1. Point to Multipoint data transfer

6.2.1.1 Enhancements to Multicast and Broadcasting

IEEE 802.16n shall provide support for Enhancements to Multicast and Broadcasting. It should cover at least 10000 people in one group.

6.2.1.2 Group call

IEEE 802.16n shall provide support for group call, including two-way communication

Each group shall be identified by the unique address; in addition the group address shall manage independently from individual address. The structure (e.g. length) may be different from individual address.

The group should be organized dynamically. MS can join in or kick out from a group via open or closed system policy and it can be reorganized the group via high priority order without intention of MS.

IEEE 802.16n should support Multi-Group call, which means the call is sent over one or more group at the same time.

Each group has the own priority, which the higher priority group interrupt the lower priority group anytime.

Group Call can be start without receiving ACK message. In one group, several calls occur at the same time

6.2.1.3 Two-way group call (mobile to mobile endpoints: virtual path)

6.2.2 Improved services

6.2.2.1 Enhanced VoIP service such as Push to talk

IEEE 802.16n shall support the VoIP service includes Push to talk.

Each call has own priority level. Based on the priority level, IEEE 802.16n VoIP service shall support pre-emption, interrupt, and waiting call service.

IEEE 802.16n shall support Emergency Call via VoIP which has highest priority among all services even Emergency Call don’t need to press PTT button to call set-up

6.2.2.2 Data services (divided into short packet and normal packet)

IEEE 802.16n shall follow data transmission policy of recent IEEE 802.16, but the specific structure and function may be modified.

Both Point-to-point and Point-to-Multipoint transmission should be supported.

IEEE 802.16n shall support the data services including Short Data Service and Normal Packet Data Service.

- Short Data Service:

  - Short data packet less than 140 byte

  - It may be piggybacked on other packet such as voice

- Normal Packet Data Service:

  - Data packet greater than 140byte

  - Support over 1Mbps for real-time (e.g. Video Streaming) and large size data transmission

  - Adopt QoS classes, an optimal matching of service and applications; Non real-time data service may delay if higher priority service is occurred

--------------------------------End of the Proposed Text ----------------------------------------------------------------------

# References

- [1] IEEE C80216gman-10_0018r2, “Draft PAR and Five Criteria from Greater Reliability In Disrupted Metropolitan Area Networks (GRIDMAN) Study Group”, 18 March, 2010.

- [2] http://www.tetramou.com.