| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text proposal to 802.16n System Requirements Document |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Eldad Zeira, Ron Murias InterDigital Communications LLC | Email: eldad.zeira@interdigital.com |
| Re: | CfC for 802.16n SRD |  |
| Abstract | This document proposes 16n SRD contents according to layout in C80216gman-10/0022. |  |
| Purpose | To be discussed and adopted by 802.16n |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Notes:

Black font is text in the outline SRD, C80216gman-10/0022. Deletions and insertions are marked.

Blue font is text not in the outline. This text has minor changes from that presented in May, mostly to adapt to the agreed layout.

Blue / highlighted is editorial comments

<<<<<<<<<<<<>>>>>>>>>>>>>>>>

# Overview

The 802.16n amendment shall be developed in accordance with the P802.16 project authorization request (PAR) [1] with its accompanying Five Criteria Statement.

The standard shall be developed as an amendment to 802.16. The resulting standard shall fit within the following scope:

This amendment specifies protocol enhancements to the IEEE 802.16 MAC for enabling increased robustness and alternate radio path establishment in degraded network conditions. Limited OFDMA PHY extensions are included for enabling operation with radio path redundancy and direct communication between subscriber stations. Also mobile base stations and mobile relay stations are supported. Support for enabling application specific specialized security suites is also provided.

With the explanatory note:

Operation in licensed, unlicensed and lightly licensed spectrum bands below 6 GHz with means and mechanisms to coexist with other radio access technologies (RATs) is supported.

This document represents the high-level system requirements for the 802.16n amendment.  All content included in any draft of the 802.16n amendment shall meet these requirements. This document, however, shall be maintained and may evolve. These system requirements embodied herein are defined to ensure competitiveness of the amended standard against other mobile broadband radio access technologies in those areas defined by the PAR and Five Criteria Statement. These system requirements also call for significant gains and improvements relative to the preexisting IEEE 802.16 system that would justify the creation of the amendment. To accelerate the completion and evaluation of the standard, to improve the clarity and reduce complexity of the standard specification, and to further facilitate the deployment of new systems, the number of optional features should be minimized.

# References

- IEEE P802.16n PAR and 5C in 80216gman-10_0018r2.doc

# Definitions

|  | The definitions below have been agreed in the PAR |
| --- | --- |
| Degraded Network | The failure of one or more 802.16 network infrastructure nodes or network connectivity |
| Robustness | The capability of the network to withstand and automatically recover from degradation to provide the required availability to support mission critical applications (essential to the core function of society and the economy). E.g. the ability to recover from a single point of failure |
| Mobile Base Station | A base station which is capable of maintaining service while moving |
| Radio Path Redundancy | The ability to provide alternative paths between base stations, relay stations, and subscriber stations |
|  | Other definitions |
| HR-MS | A subscriber station that complies with the requirements for subscriber stations in this amendment |
| HR-BS | A base station that complies with the requirements for base stations in this amendment |
| HR-RS | A relay that complies with the requirements for relays in this amendment |
| HR-network | A network whose nodes comply with their respective HR requirements in this amendment |
| Minimal QoS | The combination of data rate, latency and other relevant parameters that are considered necessary to establish a radio link |
| Radio Connectable | The ability of any two nodes to communicate directly at an acceptable certain minimal QoS |
| Infrastructure node | An HR-BS or HR-RS |

# Abbreviations and Acronyms

| SPOF | Single point of failure |
| --- | --- |

Table of Contents

1 Overview	1

2 References	1

3 Definitions	1

4 Abbreviations and Acronyms	2

5 General Requirements	5

5.1 Compatibility with other 802.16 equipment	5

5.2 Complexity	5

5.3 Services	5

5.4 Operating Frequencies	5

5.5 Operating bandwidths	5

5.6 Duplex schemes	5

5.7 Support for Government mandates and public safety	5

6 Functional Requirements	6

6.1 Requirements related to construction and maintenance of network	6

6.1.1 General	6

6.1.2 Requirements related to infrastructure SPOF immunity	6

6.1.2.1 Multi-Mode MS (able to become a BS or RS)	6

6.1.2.2 Inter-cell relaying	6

6.1.2.2.1 BS-BS direct communication HR-BS reconfigured as relay (case where BS become relay)	6

6.1.2.2.2 Relay between BS’s HR-BS maintains its BS behavior (case where BS relays while continuing to function as BS)	6

6.1.2.2.3 HR-BS and HR-RS behavior in case of failure	6

6.1.2.3 Standalone networks (No connection to core network. Includes both cases: Inter-BS connectivity, and no inter-BS connectivity)	6

6.1.2.4 Multi-hop relay	7

6.1.2.5 Shared relay (a RS is communicating with multiple BS)	7

6.1.2.6 Relay stations must also be able to function as data source/sink (maintain MS functionality while relaying)	7

6.1.3 Requirements related to link reliability	7

6.1.4 Requirements related to link existence	7

6.1.4.1 MS to MS Direct Communication	7

6.1.4.1.1 BS and relay managing of direct communication	7

6.1.4.1.2 Direct communication without BS or relay management	7

6.1.4.1.3 HR-MS behavior in the case of link failure to BS	7

6.1.4.1.4 Route discovery	8

6.1.4.2 MS to multiple-MS (one to many multicast) Direct Communication	8

6.1.4.3 Standalone networks (no BS and no RS – “ad hoc” network of MS devices only)	8

6.1.4.4 Multi Path Routing	8

6.1.4.5 Path Redundancy	8

6.1.4.6 Local Forwarding for RS and BS	8

6.1.4.7 Neighbor Discovery	8

6.1.4.7.1 Autonomous node discovery	8

6.1.4.7.2 Network aided node discovery	8

6.1.4.8 MS-MS association establishment (not including service flows)	8

6.1.4.8.1 BS or relay support for establishing direct communication	8

6.1.4.8.2 When some MS cannot be controlled by the BS	8

6.1.4.9 Multi-hop relay	9

6.1.4.10 Relay stations must also be able to function as data source/sink (maintain MS functionality while relaying)	9

6.1.5 Requirements related to mobility	9

6.1.5.1 Mobile Base Stations	9

6.1.5.2 Mobile relay stations	9

6.1.6 Requirements related to security	9

6.1.6.1 New security procedures for MS-MS direct communication	9

6.1.6.2 Group Key Management	9

6.1.7 Coexistence requirements	9

6.1.7.1 Operation in unlicensed and lightly licensed bands	9

6.1.7.2 Support for Multi-carrier operation in different licensing regimes	9

6.2 Requirements related to Services provided on network	9

6.2.1 Point to Multipoint data transfer	9

6.2.1.1 Enhancements to Multicast and Broadcasting	10

6.2.1.2 Group call	10

6.2.1.3 Two-way group call (mobile to mobile endpoints: virtual path)	10

6.2.2 Improved services	10

6.2.2.1 Enhanced VoIP service such as Push to talk	10

6.2.2.2 Data services (divided into short packet and normal packet)	10

6.2.2.3 Performance requirements for connection setup time and connection maintenance (except as related to network construction and maintenance)	10

7 Performance Requirements	11

7.1 General	11

7.2 Requirements on network robustness	11

7.2.1 Recovery time from HR-BS backhaul failure	11

7.2.2 Recovery time from HR-BS failure	11

7.2.3 Recovery time from HR-RS failure	11

7.3 Mobile to mobile performance requirements	11

7.3.1 Peak throughput	11

7.3.2 Cell-average throughput	11

7.4 Cell coverage	11

7.4.1 Cell coverage reliability with mobile relaying	11

7.5 Cell average throughput	11

7.5.1 Cell average throughput with MS-MS relaying	11

7.5.2 Cell average throughput with local traffic	11

7.6 Cell edge throughput	11

7.6.1 Cell edge throughput with MS-MS relaying	11

7.6.2 Cell edge throughput with local traffic	12

7.7 Node discovery performance	12

7.7.1 Node discovery success rate	12

7.7.1.1 MS-MS node discovery	12

7.7.1.2 Other nodes	12

7.7.2 Node discovery time	12

7.8 Direct connection establishment performance	12

7.8.1 Direct connection establishment time between MS	12

7.8.2 Direct connection establishment time between base stations and relays	12

8 Operational Requirements	12

# General Requirements

## Compatibility with other 802.16 equipment

This section is pending results of ongoing discussion regarding the baseline for the enhancements.

## Complexity

IEEE 802.16n amendment should minimize complexity of the architecture and protocols and avoid excessive system complexity.  It should enable interoperability of access networks, support low cost devices and minimize total cost of ownership.

IEEE 802.16n should only provide enhancements in areas where current 802.16 systems does not meet the requirements in this document. The IEEE 802.16n system shall satisfy the performance requirements in this document.

## Services

## Operating Frequencies

802.16n shall operate in all radio frequencies where 802.16m operates.

802.16n shall also operate in unlicensed and lightly licensed spectrum bands below 6 GHz with means and mechanisms to coexist with other radio access technologies (RATs)

## Operating bandwidths

## Duplex schemes

## Support for Government mandates and public safety

# Functional Requirements

HR-Network devices shall comply with all AAI or 802.16 OFDMA functional requirements unless indicated below. Additional functional requirements are indicated in this section.

Comment: note baseline dependency pending

## Requirements related to construction and maintenance of network

### General

Support for different topologies

### Requirements related to infrastructure SPOF immunity

An HR-network shall be able to recover from any single point failure in any of its infrastructure nodes (i.e. all nodes excluding the subscriber station) or any of its radio links.

#### Multi-Mode MS (able to become a BS or RS)

An HR-MS may be capable of changing its role to become an HR-BS or an HR-RS.

Comment: we already have a requirement for relaying by MS elsewhere in this document. Is this the same requirement? Alternatively this requirement could simply mean that an MS can switch itself off and re-emerge as e.g. a BS. If this is the case, do we need to standardize this feature?

#### Inter-cell relaying

The HR-network shall support inter-cell relaying. In case of a failure of HR-BS backhaul the HR-BS shall be able to maintain service to its HR-MS and HR-RS by conveying user data to and from another HR-BS. Two mechanisms shall be supported:

##### BS-BS direct communication HR-BS reconfigured as relay (case where BS become relay)

HR-Network shall support MAC and PHY mechanisms to support reconfiguration of an HR-BS for relaying to another HR-BS.

##### Relay between BS’s HR-BS maintains its BS behavior (case where BS relays while continuing to function as BS)

HR-Network shall support HR-BS communication to another HR-BS through an HR-RS which relays user data between two HR-BS.

##### HR-BS and HR-RS behavior in case of failure

An HR-BS shall be able to detect the loss of backhaul connectivity. An HR-BS that loses its backhaul shall attempt to continue to serve its attached mobiles by either of the following:

- The HR-BS shall reconfigure itself as an RS. It shall then attempt to attach itself as a relay to another HR-BS or HR-RS. If successful, it shall then serve its mobiles as a relay.

- The HR-BS shall seek a connection through an RS that is attached to another cell while maintaining its behavior as a BS.

#### Standalone networks (No connection to core network. Includes both cases: Inter-BS connectivity, and no inter-BS connectivity)

All functionality in clauses 6.1.2.2 above shall operate even if no connection to core network is available.

Comment: not clear to me if this requirement merely mandates including CN functionality in same site and if this is the case do we need to standardize it?

#### Multi-hop relay

All HR nodes that are capable of relaying shall support multi-hop relaying.

#### Shared relay (a RS is communicating with multiple BS)

HR-RS and HR-BS shall support the HR-RS relaying of user data between HR-MS and two or more HR-BS.

#### Relay stations must also be able to function as data source/sink (maintain MS functionality while relaying)

Comment: this requirement seems to suggest a way to implement the MS relaying functionality already required elsewhere in this document. Suggest removing it.

### Requirements related to link reliability

Improved Link Reliability (MAC coding, etc)

### Requirements related to link existence

#### MS to MS Direct Communication

MS-MS direct communication shall support both relaying and HR-MS to HR-MS communication of user data.

The HR network shall support MS-MS direct communication to relay user data and control signaling between HR nodes. The HR-MS shall support relaying between two HR-MS (MS-MS-MS configuration), an HR-MS and HR-BS (MS-MS-BS configuration), and HR-MS and HR-RS (MS-MS-RS).

Either or all of these HR-MS may not have a connection to an infrastructure node.

##### BS and relay managing of direct communication

HR-BS and HR-RS shall be capable of sending control signaling to support direct communication between two HR-MS associated with it

##### Direct communication without BS or relay management

HR-MS that are within range of an HR-BS or HR-RS shall be able to relay and/or generate control information to other HR-MS. HR-MS that are not within range shall be able to receive and act on that controlling information.

##### HR-MS behavior in the case of link failure to BS

An HR-MS in an HR-network shall be able to detect the failure of its nearby serving infrastructure node or the inability to create or maintain a connection to it. The HR-MS shall attempt to reestablish connection through another infrastructure node. If no other infrastructure node is available the HR-MS shall attempt to connect through another HR-MS.

##### Route discovery

HR-Network shall support route discovery from, to and between HR-MS. HR-BS and HR-RS shall support route discovery for HR-MS within its range. HR-MSs shall be able to perform autonomous route discovery for each of their connections in case they are not within range of a serving HR-BS. The connection could be to another HR-MS, or it could eventually be routed to an infrastructure node.

#### MS to multiple-MS (one to many multicast) Direct Communication

HR-Network shall support a one-to-many MS to MS direct communication.

#### Standalone networks (no BS and no RS – “ad hoc” network of MS devices only)

#### Multi Path Routing

Comment: is this the same as path redundancy?

#### Path Redundancy

Comment: not clear to me if this is an independent requirement or is already detailed in other requirements in this document.

#### Local Forwarding for RS and BS

#### Neighbor Discovery

The HR-Network shall support the HR-MS discovery by other HR-MS, defined here as the capability to establish the presence and capabilities of other HR-MS, including their connectivity status to other nodes.

##### Autonomous node discovery

The HR-Network shall support autonomous discovery of HR-MS by other HR-MS that cannot receive signaling from network infrastructure nodes.

##### Network aided node discovery

The HR-Network shall support discovery of HR-MS by other HR-MS with the aid of network provided information and signaling.

#### MS-MS association establishment (not including service flows)

##### BS or relay support for establishing direct communication

An HR-Network infrastructure node (HR-BS or HR-RS) shall support connection establishment between any two HR-MS associated with it.

##### When some MS cannot be controlled by the BS

Direct communications between MS shall be supported between MS that cannot be directly controlled by a BS or relay [due e.g. to their being out of range from an operational BS or relay].

HR-MS that are within range of HR-BS shall be capable of relaying signaling for the establishment of a connection to HR-MS that are outside range of a HR-BS.

#### Multi-hop relay

Comment: We already have this requirement in 6.1.2.4, no need to duplicate but we should discuss which clause is best for it.

#### Relay stations must also be able to function as data source/sink (maintain MS functionality while relaying)

Comment: this requirement seems to suggest a way to implement the MS relaying functionality already required elsewhere in this document. See also 6.1.2.6. Suggest removing it.

### Requirements related to mobility

#### Mobile Base Stations

#### Mobile relay stations

### Requirements related to security

The HR-Network topology shall not degrade the security performance achieved with AAI in hierarchical network topology.

#### New security procedures for MS-MS direct communication

TBD

#### Group Key Management

### Coexistence requirements

#### Operation in unlicensed and lightly licensed bands

HR devices shall comply with regulators’ respective requirements for operation in unlicensed and lightly licensed spectrum.

#### Support for Multi-carrier operation in different licensing regimes

The HR-Network shall support multicarrier operation in licensed, unlicensed and lightly-licensed licensing environments.

An HR-MS that supports MC operation in different licensing environments shall be able of operating in all three types of spectrum at the same time.

## Requirements related to Services provided on network

### Point to Multipoint data transfer

#### Enhancements to Multicast and Broadcasting

#### Group call

#### Two-way group call (mobile to mobile endpoints: virtual path)

### Improved services

#### Enhanced VoIP service such as Push to talk

#### Data services (divided into short packet and normal packet)

#### Performance requirements for connection setup time and connection maintenance (except as related to network construction and maintenance)

# Performance Requirements

## General

Comment: this section pending baseline discussion

## Requirements on network robustness

A subscriber node shall be considered always serviceable if there exists at least one node that can establish an alternative path of acceptable QoS between that node and a messaging correspondent node.

[Note that the likelihood of nodes to be serviceable according to this definition is a matter of deployment only and isn’t specified here]

### Recovery time from HR-BS backhaul failure

TBD

### Recovery time from HR-BS failure

TBD

### Recovery time from HR-RS failure

TBD

## Mobile to mobile performance requirements

### Peak throughput

TBD

### Cell-average throughput

TBD

## Cell coverage

HR-Network cell coverage shall meet or exceed AAI requirements for same topology.

### Cell coverage reliability with mobile relaying

Cell coverage reliability is defined as the percentage of HR-MS that can be served with sufficient QoS for a given deployment. Cell coverage shall exceed TBD%.

## Cell average throughput

HR-Network cell average throughput shall meet or exceed AAI requirements for same topology.

### Cell average throughput with MS-MS relaying

TBD

### Cell average throughput with local traffic

TBD

## Cell edge throughput

HR-Network cell edge throughput shall meet or exceed AAI requirements for same topology.

### Cell edge throughput with MS-MS relaying

TBD

### Cell edge throughput with local traffic

TBD

## Node discovery performance

### Node discovery success rate

#### MS-MS node discovery

An HR-MS shall be able to autonomously detect a neighbor within TBD-meter range with a TBD% success rate within the node discovery time.

An HR-MS shall be able to detect a neighbor within TBD-meter range with a TBD% success rate within the node discovery time when aided by the network.

#### Other nodes

TBD

### Node discovery time

An HR-MS shall be able to autonomously detect a neighbor within a TBD meter range within TBD seconds.

An HR-MS shall be able to detect a neighbor within a TBD meter range within TBD seconds when aided by the network.

## Direct connection establishment performance

### Direct connection establishment time between MS

TBD

### Direct connection establishment time between base stations and relays

TBD

# Operational Requirements