| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text proposal to 802.16n System Requirement Document (Section 6.1) |  |
| Date Submitted | 2010-07-10 |  |
| Source(s) | Anseok Lee, Wooram Shin, Hyun Jae Kim, Kwang Jae Lim ETRI | Email: alee@etri.re.kr w.shin@etri.re.kr khjgo@etri.re.kr kjlim@etri.re.kr |
| Re: | Call for contribution for P802.16n System Requirement Document |  |
| Abstract | This document proposes contents of 802.16n SRD based on the outline of SRD. |  |
| Purpose | To be discussed and adopted by 802.16n SRD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Preliminary

In this proposal, we provide requirements related to construction and maintenance (Section 6.1) for 802.16n System Requirement Document.

To improve clarity and reduce duplication of requirements, we inserted proposed text based on re-arranged requirements of outline of SRD (C80216gman-10/0022).

Notes

Black font: The existing text in the outline SRD, C80216gman-10/0022.

Black font in strike through: Proposed for deletion in the outline SRD.

Blue font: Proposed text to 802.16n SRD.

Highlighted font: Editorial comments.

---------------------------- Start of ToC (Section 6.1) on page 3 [1] --------------------------------------------------------

Table of Contents

6 Functional Requirements	2

6.1 Requirements related to construction and maintenance of network	2

6.1.1 Requirements related to multi-mode operation	2

6.1.1.1 Relay function for HR-BS	2

6.1.1.2 Multi-mode MS (able to become a BS or RS) Relay function for HR-MS	2

6.1.2 Requirements related to link existences	2

6.1.2.1 HR-BS to HR-BS Direct Communication (case where BS become relay)	2

6.1.2.2 HR-MS to HR-MS Direct Communication	2

6.1.2.3 HR-MS to multiple HR-MS direct communication	3

6.1.2.4 Standalone networks (No connection to core network. Includes both cases: Inter-BS connectivity, and no inter-BS connectivity)	3

6.1.2.5 Standalone networks (no BS and no RS - “ad hoc” network of MS devices only) of HR-MS devices only	3

6.1.2.6 Path Redundancy	3

6.1.2.7 Multi Path Routing	3

6.1.2.8 Local Forwarding for RS and BS	3

6.1.2.9 Neighbor Discovery	3

6.1.2.10 MS-MS association establishment (not including service flows)	3

6.1.2.11 Multi-hop relay	3

6.1.2.12 Relay stations must also be able to function as data source/sink (maintain MS functionality while relaying)	3

6.1.3 Requirements related to infrastructure SPOF immunity	4

6.1.3.1 Recovery of failure on network connectivity	4

6.1.3.2 Recovery of failure on infrastructure	4

6.1.3.3 Relay between BS’s (case where BS relays while continuing to function as BS)	4

6.1.3.4 Multi-hop relay	4

6.1.3.5 Shared relay (a RS is communicating with multiple BS)	4

6.1.3.6 Relay stations must also be able to function as data source/sink (maintain MS functio-nality while relaying)	4

6.1.4 Requirements related to link reliability	4

6.1.5 Requirements related to mobility	4

6.1.5.1 Mobile Base Stations	4

6.1.5.2 Mobile relay stations	4

6.1.6 Requirements related to security	5

6.1.6.1 New security procedures for MS-MS direct communication	5

6.1.6.2 Group Key Management	5

6.1.7 Coexistence requirements	5

6.1.7.1 Operation in unlicensed and lightly licensed bands	5

6.1.7.2 Support for Multi-carrier operation in different licensing regimes	5

---------------------------- End of ToC (Section 6.1) ----------------------------------------------------------------------------

---------------------------- Start of Section 6.1 on page 6 [1] -----------------------------------------------------------------

# Functional Requirements

## Requirements related to construction and maintenance of network

6.1.1 General

Support for different topologies

This section contains requirements for IEEE 802.16n related to construction and maintenance of network. These requirements are intended to address multi-mode operation, link existence, infrastructure SPOF immunity, link reliability, mobility, security, and coexistence.

### Requirements related to multi-mode operation

#### Relay function for HR-BS

IEEE 802.16n shall provide relay function for HR-BS which enables HR-BS to have relay functions including network discovery, connection management, and association to wireless access from either HR-BS or HR-RS.

This shall enable HR-BS to serve wireless access to subscriber stations by using wireless access network provided by neighbor HR-BS or HR-RS.

#### Multi-mode MS (able to become a BS or RS) Relay function for HR-MS

IEEE 802.16n shall provide relay function for HR-MS which enables HR-MS to have relay functions including connection management and data delivery for HR-MS.

This shall enable HR-MS to associate and communicate with HR-MS.

Comment: Is BS function required for MS in 802.16n?

### Requirements related to link existences

#### HR-BS to HR-BS Direct Communication (case where BS become relay)

IEEE 802.16n shall support HR-BS to HR-BS communication to enable HR-BS to communicate with another HR-BS directly or through intermediate RSs.

This shall be achieved by consisting of HR-BS with relay function.

#### HR-MS to HR-MS Direct Communication

IEEE 802.16n shall support HR-MS to HR-MS communication to enable HR-MS to communicate with another HR-MS directly or through intermediate HR-MSs.

This shall be achieved by consisting of HR-MS with relay function.

#### HR-MS to multiple HR-MS direct communication

IEEE 802.16n shall support HR-MS to multiple HR-MS direct communication to allow communicate with neighboring HR-MSs.

This shall enable group call service among nearby HR-MSs without assist of infrastructure.

#### Standalone networks (No connection to core network. Includes both cases: Inter-BS connectivity, and no inter-BS connectivity)

IEEE 802.16n shall support standalone operation which provides connectivity among stations with no network connectivity to core network.

This operation shall enable emergency communication among stations when infrastructure recovery is not possible in degraded network condition.

#### Standalone networks (no BS and no RS - “ad hoc” network of MS devices only) of HR-MS devices only

IEEE 802.16n shall support standalone networks of HR-MSs to allow independent operation without infrastructures. This shall enable communication among HR-MSs in the area where infrastructure installation is difficult or impossible. (E.g. Maritime networks)

This shall be achieved by providing HR-MS to HR-MS communication.

#### Path Redundancy

#### Multi Path Routing

Comment: “Multi Path Routing” is duplicated with “Path Redundancy”. This is merged into “6.1.2.7 Path Redundancy”.

#### Local Forwarding for RS and BS

Comment: This feature is merged into “6.1.1 Requirement related to multi-mode operation”.

#### Neighbor Discovery

#### MS-MS association establishment (not including service flows)

Comment: “Neighbor Discovery” and “MS-MS association establishment” are about multi-mode operation of stations. These features are merged into “6.1.1 Requirement related to multi-mode operation”.

#### Multi-hop relay

Comment: It seems that “Multi-hop relay” can be provided by current 802.16 Std.

#### Relay stations must also be able to function as data source/sink (maintain MS functionality while relaying)

Comment: This feature is about multi-mode operation of MS. This is merged into “6.1.1 Requirement for multi-mode operation”.

### Requirements related to infrastructure SPOF immunity

#### Recovery of failure on network connectivity

IEEE 802.16n shall support recovery of failure on network connectivity by providing degraded HR-BS with wireless access network connectivity from another HR-BS.

This shall be achieved by HR-BS to HR-BS communication including either direct or relayed fashion.

#### Recovery of failure on infrastructure

IEEE 802.16n shall support recovery of failure on infrastructure by providing wireless access to subscriber stations in degraded network without assist of infrastructures.

This shall be achieved by HR-MS to HR-MS communication including either direct or relayed fashion.

#### Relay between BS’s (case where BS relays while continuing to function as BS)

Comment: “Relay between BS’s” is for recovery of failure on network connectivity of BS.

This feature is merged into “6.1.3.1 Recovery of failure on network connectivity of BS”.

#### Multi-hop relay

Comment: It seems that “Multi-hop relay” can be provided by current 802.16 Std.

#### Shared relay (a RS is communicating with multiple BS)

Comment: “Shared relay” is for recovery of failure on network connectivity of BS.

This feature is merged into “6.1.3.1 Recovery of failure on network connectivity of BS”.

#### Relay stations must also be able to function as data source/sink (maintain MS functionality while relaying)

Comment: This feature is about multi-mode operation of station. This is merged into “6.1.1 Requirement for multi-mode operation”.

### Requirements related to link reliability

Improved Link Reliability (MAC coding, etc)

### Requirements related to mobility

#### Mobile Base Stations

#### Mobile relay stations

Comment: It seems that “Mobile relay stations” can be provided by current 802.16 Std.

### Requirements related to security

#### New security procedures for MS-MS direct communication

#### Group Key Management

### Coexistence requirements

#### Operation in unlicensed and lightly licensed bands

#### Support for Multi-carrier operation in different licensing regimes

---------------------------- End of Section 6.1 -------------------------------------------------------------------------------------

References

[1] IEEE C802.16gman-10/0022, “Outline of 802.16n System Requirements Document, ” May 2010.

[2] IEEE 802.16gman-10/0018r2, “Draft PAR and Five Criteria from Greater Reliability in Disrupted Metropolitan Area Networks (GRIDMAN) Study Group, ” March 2010.

[3] IEEE 802.16gman-10/0022r1, “Call for Contributions for P802.16n System Requirements Document, ” April 2010.