| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Fast Base Station Switch |  |
| Date Submitted | 2004-06-07 |  |
| Source(s) | Mary Chion Jing Wang | mchion@ztesandiego.com jwang@ztesandiego.com |
| Re: | Contribution on comments to IEEE P802.16e/D2 |  |
| Abstract | Fast Base Station Switch Process provides diversity gain without increasing much complexity comparing to the standard HO procedure in the current 802.16e system |  |
| Purpose | Adoption |  |
| Notice | This document has been prepared to assist IEEE 802.16. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate text contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy and Procedures | The contributor is familiar with the IEEE 802.16 Patent Policy and Procedures (Version 1.0) <http://ieee802.org/16/ipr/patents/policy.html>, including the statement “IEEE standards may include the known use of patent(s), including patent applications, if there is technical justification in the opinion of the standards-developing committee and provided the IEEE receives assurance from the patent holder that it will license applicants under reasonable terms and conditions for the purpose of implementing the standard.” Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication. Please notify the Chair <mailto:r.b.marks@ieee.org> as early as possible, in written or electronic form, of any patents (granted or under application) that may cover technology that is under consideration by or has been approved by IEEE 802.16. The Chair will disclose this notification via the IEEE 802.16 web site <http://ieee802.org/16/ipr/patents/notices>. |  |

Fast Base Station Switch

Mary Chion and Jing Wang

ZTE San Diego, Inc.

# Introduction

Soft Hand Over (SHO) improves HO quality by means of macro-diversity. However, the potential gain may be marred by the associated complexity and overhead.

# Overview of Proposed Solution

In this contribution we propose an alternative HO solution—fast Base Station switching (FBSS). FBSS utilizes selection diversity and fast switching mechanism to improve link quality with less complexity. Instead of frame-by-frame transmission synchronization required by SHO, a commonly shared MAC among BSs facilities FBSS process. The FBSS offers the following benefits:

- It provides diversity gain by allowing fast switching of data transmission from BS to another BS dynamically.

- It does not require much PHY changes.

- Since only BS is transmitting at any given time, the scheduler can be more flexible and optimized than in SHO (no data synchronization issue to worry about).

- It can easily support data connection with H-ARQ.

- It won't take up additional air link capacity/resource.

Before a detail description can be made for Fast BS Switching HO, a few definitions need to be made:

- Serving BS: A serving BS is a BS that has allocated resources to the MSS, i.e. assigned Basic CID, Primary Management CID, Secondary Management CID and data CIDs to the MSS.

MSS should keep synchronization with a serving BS at all time.

- Target BS: A target BS is a BS that the MSS is intended to HO to. Once the HO process is successfully completed, a target BS becomes a serving BS.

- Transmitting BS: A transmitting BS is the serving BS that is designated to transmit/receive data to/from the MSS at a given frame.

- Active Set: The active set contains a list of serving BSs to the MSS. The active set is maintained at the BS.

The basic concept of VSHO is as follows.

When a MSS is in Fast BS Switching HO, the MSS’s active set contains multiple serving BSs.

The MSS is only transmitting/receiving data to/from one of the serving BS (transmitting BS) at any given frame. The transmitting BS can change from frame to frame depending on the BS allocation scheme. The MSS is also required to process DL_MAP and UL_MAP from all serving BSs at each frame and decides which serving BS is the transmitting BS for the current frame.

The MSS monitors the downlink of all serving BSs in the active set and determines its preferred transmitting BS based on received CINR from all serving BS. The MSS sends its preferred transmitting BS to the current transmitting BS over fast feedback channel. When the BS receives the request, it shall change the transmitting BS to the MSS preferred BS after all H-ARQ (if activated) re-transmissions are completed.

A FBSS message flow is included to serve as an example for adding a serving BS and the basic operation of Fast BS Switching HO.

# Proposed Text

TBD

Fig. 1 FBSS Message Flow

The common shared MAC can be achieved by Level 3 MAC context sharing or transfer.

The switching of transmitting BS can also be done through MAC message. In this case, the MSS does not need to read DL_MAP and UL_MAP from multiple BSs.