| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | BS Architecture Impact on Handoff Consideration |  |
| Date Submitted | 2004-05-04 |  |
| Serving(s) | Dongkie Lee, DongIl Moon, DongRyul Lee, JongKuk Ahn, Sungho Ha SK Telecom 15F, Seoul Finance Center, 84, Taepyungpro 1 ga, Chung-gu, Seoul, 100-768, Korea | Voice: +82-2-6323-3147 Fax: +82-2-6323-4493 [mailto: {galahad,dimoon,drlee,jgahn,ss23}@sktelecom.com] |
| Re: | Response to IEEE 802.16-04/19 (Recirculation Ballot #14a Announcement) |  |
| Abstract | Current BS Architecture discussion is made over BS-BSC hierarchical relationship. In this contribution, another BS-BSC connection method is proposed, where a mobile client is assigned a BSC based on the MAC address. |  |
| Purpose | Discuss and Adopt as the session information management mechanism |  |
| Notice | This document has been prepared to assist IEEE 802.16. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy and Procedures | The contributor is familiar with the IEEE 802.16 Patent Policy and Procedures <http://ieee802.org/16/ipr/patents/policy.html>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard." Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication. Please notify the Chair <mailto:chair@wirelessman.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.16 Working Group. The Chair will disclose this notification via the IEEE 802.16 web site <http://ieee802.org/16/ipr/patents/notices>. |  |

BS Architecture Impact on Handoff Consideration

Dongkie Lee, DongRyul Lee, DongIl Moon, JongKuk Ahn
SK Telecom

# Problem Statements

Current BS Architecture discussion is made based on the assumption that BS is connected to hierarchically higher BSC. Although this kind of connection is widely used, and simple to implement and easyw there is another approach where a MSS is assigned a BSC based on the MAC address.

# Overview of Proposed Solution

BSs are interconnected with BSCs by m X n conncetion, where m is the number of BSs and n is the number of BSCs. This configuration maybe applied to specified management domain and don’t need to be applied to nationwide or operator-wide.

BS Controller

BS Controller

Metro Ethernet

When a MSS is authenticated by a BS, BS decides to which BSC that MSS is served based on the internal policy. MSS maybe assigned a BSC based on the MAC address prefix, suffix or MAC hash. It’s like in cellular world Mobile Station is served by HLRs, where HLR is assigned and customer profile is provisioned a priori  based on the Mobile Station’s NPA-NXX block.

BS Controller

BS Controller

Metro Ethernet

BS Controller

Figure 1 Current BS-BSC Connection Topology

Figure 2 Proposed BS-BSC Connection Topology

Using this method, pseudo Mobile IP service is provided to MSS without using Home Agent, because if a MSS moves in this management domain, MSS uses the same IP address assigned by serving BSC. Additionally, wherever the MSS moves in this connectivity domain, MSS is always served by the same BSC. Therefore IP Address refresh and TFTP, NTP, Key Exchange etc could be skipped.

With this topology, there should be indication in the MAC message from the BS to MSS, which tells MSS to skip some specific exchange.

| Indicator Bit | Meaning |
| --- | --- |
| IP Address Refresh maybe skipped | 0 or 1 |
| Authentication maybe skipped | 0 or 1 |
| TFTP maybe skipped | 0 or 1 |
| NTP maybe skipped | 0 or 1 |
| etc |  |

# Conclusion

In order to incorporate this kind of topology and allow MSS to skip unnecessary exchanges, some indications should be sent to the MSS from the BS using a MAC message.

NPA – Numbering Plan Area, 3 most significant digits out of 10 digits is called NPA.