| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Safety Channel Handover Procedure |  |
| Date Submitted | [2004-06-18] |  |
| Source(s) | Hyunjeong Kang Jungje Son Changhoi Koo PanYuh Joo Sijun Cho SeungJoo Maeng Samsung Elec. 416, Maetan-3dong, Youngtong-gu Suwon-si, Gyeonggi-do Korea | Voice: +82-31-279-5091 Fax:+82-31-279-5130 hyunjeong.kang@samsung.com jungje.son@samsung.com chkoo@samsung.com panyuh@samsung.com sijun.cho@samsung.com sjmaeng@samsung.com |
| Re: | This contribution is for call for contribution IEEEP802.16e Handover Ad hoc |  |
| Abstract | This contribution proposes the safety channel allocation and handover procedure |  |
| Purpose | Adoption as part of Handover Adhoc recommendation to IEEE 802.16e |  |
| Notice | This document has been prepared to assist IEEE 802.16. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy and Procedures | The contributor is familiar with the IEEE 802.16 Patent Policy and Procedures <http://ieee802.org/16/ipr/patents/policy.html>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard." Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication. Please notify the Chair <mailto:chair@wirelessman.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.16 Working Group. The Chair will disclose this notification via the IEEE 802.16 web site <http://ieee802.org/16/ipr/patents/notices>. |  |

Safety Channel Handover Procedure

Hyunjeong Kang, Changhoi Koo, Jungje Son,
Panyuh Joo, Sijun Cho, SeungJoo Maeng

Samsung Electronics

# Introduction

In order to reduce the interference caused by a neighboring BS, an MSS moving toward its Serving cell boundary may be switched from its current data channel to the safety channel (or safety zone) defined for the neighboring BS. Since the safety channel itself shall not be used in the neighboring BS, the MSS using this channel in the Serving BS can be served with less interference. This transition to the safety channel is triggered based on the MSS report, which includes some neighboring BSs with their CINR levels exceeding a predefined SafetyCH_Threshold.  Then the Serving BS chooses a BS with the highest CINR level and requests the safety channel information of the BS. If the corresponding channel in the Serving BS is available, it is allocated to the MSS. Otherwise the BS should force the MSS to perform a handover to the neighbor BS.

# Proposed Mechanism

- Case 1: Serving BS can allocate the channel zone corresponding to the safety channel zone of neighbor BS

- Case 2: Serving BS cannot allocate the channel zone, therefore it forces the MSS to perform Safety Channel Handover to the neighbor BS

# Proposed Text Changes

We propose the following remedies in IEEE P802.16e/D2 to provide the handover enhancement method related with the fast call recovery

[Add the followings after line 8 page 46]

If the received CINRs of neighbor BSs exceed the predefined SafetyCH_Threshold, the MSS reports the scanning result to its Serving BS. Then the Serving BS may try to allocate the channel correspondent to the safety channel of the neighbor BS with the highest CINR. In this case, the Serving BS and the neighbor BS exchange the safety channel information over the backbone (see section Backbone network HO procedures).

[Change the followings in line 36-40 page 50]

If Handover mode is set to “01” in MOB-BSHO-REQ message, MSS may perform a hand-over to any BS among the recommended BSs in MOB-BSHO-REQ without notifying the Serving BS of a selected Target BS. As an acknowledgement to the MOB-BSHO-REQ message, the MSS may send a MOB-HO-IND message with its Target BS ID set to a pre-defined value other than any valid BS identifier.

[Add the followings after line 45 page 50]

If Handover mode is set to “10” in MOB-BSHO-REQ message, MSS shall perform a handover to the Target BS in recommended BS list after sending MOB-HO-IND message. Handover mode “10” means that the MSS may not use the safety channel in its serving cell and therefore it shall change its connection to the Target BS. The Serving BS informs the Target BS of the Serving BS’s safety channel information over the backbone. The Target BS shall allocate the channel to the MSS performing Safety channel handover to itself.

[Change the table 92h in page 25]

Table 92h – MOB-BSHO-REQ Message Format

| Syntax | Size | Notes |
| --- | --- | --- |
| MOB-BSHO-REQ_Message_Format(){ |  |  |
| Management Message Type = 52 | 8bits |  |
| Handover mode | 2bits | 00: Network Assisted HO is not supported 01: Network Assisted HO is supported 10: Safety Channel Handover 11: reserved |
| For(j=0;j<N_Recommended;j++){ |  | N_Recommended can be derived from the known length of the message |
| Neighbor BS-ID | 48bits |  |
| Service level prediction | 8bits |  |
| } |  |  |
| Reserved | 6bits | Reserved; shall be set to zero |
| HMAC Tuple | 21bytes | See 11.4.11 |
| } |  |  |

[Change the following in line 25-31 page 26]

Handover mode

This flag indicates that the Serving BS supports the Network Assisted HO features. This flag also indicates the Serving BS forces the MSS to handover to the Target BS in N_Recommended list. The following applies:

00 = Network Assisted HO is not supported

01 = Network Assisted HO is supported

10 = Safety Channel Handover

11 = reserved

[Insert the following after the end of section 11.8.3 in page 108]

11.8.4 SafetyCH_Threshold

This field indicates the predefined CINR threshold to report MSS’s scanning result for safety channel handover operation.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 1 | 1 | SafetyCH_Threshold | SBC-RSP (see 6.3.2.3.24) |

[Insert the following messages after the end of section D.2.13 in page 125]

D. 2.14 SafetyCH-Info message

This message is sent from the Serving BS to the neighbor BS to request the neighbor BS’s safety channel zone information. This message is also sent from the neighbor BS to the Serving BS to inform the neighbor’s safety channel information in case where the neighbor BS received SafetyCH-Info message with Info-request set to 0. The message contains the following information:

| Field | Size | Notes |
| --- | --- | --- |
| SafetyCH-Info_Message_Format(){ |  |  |
| Global Header | 152bits |  |
| Info-request | 1bit | 0: Request safety channel information 1: Inform safety channel information |
| TLV_Safety_channel_info | Variable | Safety channel information for case where info-request value is set to 1. |
| Security field | TBD | A means to authenticate this message |
| CRC field | 32bits | IEEE CRC-32 |
| } |  |  |

| TLV_Safety_channel_info(){ |  |  |
| --- | --- | --- |
| OFMDA symbol offset | 8bit |  |
| Subchannel offset | 7bit |  |
| No. OFDMA symbols | 7bit |  |
| No. subchannels | 7bit |  |
| } |  |  |

D. 2.15 SafetyCH-Alloc-Info message

This message is sent from the Serving BS to the neighbor BS which provided its safety channel information to inform whether the safety channel is successfully allocated to the MSS. If the safety channel allocation fails, the Serving BS informs the neighbor BS of the MSS’s handover. The message contains the following information:

| Field | Size | Notes |
| --- | --- | --- |
| SafetyCH-Alloc-Info_Message_Format(){ |  |  |
| Global Header | 152bits |  |
| Alloc flag | 1bit | Indicate whether the BS allocates the safety channel which provided from other BS. 0: Allocation fail and Safety Channel Handover for the MSS will happen. 1: Allocation success |
| MSS unique identifier | 48bits | 48bit unique identifier used by MSS. This field informs the ID of MSS to perform Safety Channel Handover and is activated for case where Alloc flag is set to 0 |
| TLV_Safety_channel_info | Variable | This field informs the BS’s safety channel information and is activated for case where Alloc flag is set to 0. (the same format with TLV_Safety_channel_info in SafetyCH-Info message) |
| Security field | TBD | A means to authenticate this message |
| CRC field | 32bits | IEEE CRC-32 |
| } |  |  |