| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Minimization of IP Connection Re-establishment Procedure |  |
| Date Submitted | 2004-06-17 |  |
| Source(s) | Sungjin Lee, Jungje Son Changhoi Koo Samsung Electronics Samsung Electronic, Suwon P.O.Box 105, 416, Maetan-3dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, Korea 442-742 | Voice: +82 31 279 5248 Fax: +82 31 279 5130 steve.lee@samsung.com jungje.son@samsung.com chkoo@samsung.com |
| Re: | This is a response to a Call for Comments in IEEE802.16e Handover Adhoc |  |
| Abstract | In this contribution, the MOB_NBR-ADV message modification is proposed |  |
| Purpose | Adoption as part of Handover Adhoc recommendation to IEEE802.16e |  |
| Notice | This document has been prepared to assist IEEE 802.16. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy and Procedures | The contributor is familiar with the IEEE 802.16 Patent Policy and Procedures <http://ieee802.org/16/ipr/patents/policy.html>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard." Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication. Please notify the Chair <mailto:chair@wirelessman.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.16 Working Group. The Chair will disclose this notification via the IEEE 802.16 web site <http://ieee802.org/16/ipr/patents/notices>. |  |

Minimization of IP Connection Re-establishment Procedure

Sungjin Lee, Jungje Son, Changhoi Koo

SAMSUNG Electronics

# Introduction

In IEEE 802.16e system, a MSS may be re-allocated IP address by DHCP, DHCPv6 or Mobile IP after it completes network re-entry with target BS. However it may not be needed to re-allocate IP address whenever it changed BS because the IP address could be reusable as far as the MSS moving around within a same IP subnet. Therefore we need to provide the information indicating whether a target BS reside in same IP subnet with serving BS so that the BS can decide whether it trigger IP re-establishment process or reuse the old IP address skipping IP re-establishment process. We also should consider the idle mode case. Even if a MSS is deregistered and transfer the mode to idle state, the MSS still move around between BSs, so that the idle mode MSS also need to be informed whether the BS has same Paging zone ID or not.

There have been a number of proposals in 802.16e HO ad-hoc to provide IP subnet information and the information could be provided before handoff through MOB_NBR-ADV and after handoff with RNG-REQ/RSP in contributions. However, we also need to understand that information of BS as one of the capability of that BS. If a MSS or BS decides the target BS to handoff to it finally should decide only one target BS to handoff based on the information, service level prediction and CINR and etc., collected from BS or scanning procedure. In this case, we also need to consider the equality of IP subnet ID and Paging zone when a MSS/BS make a decision of target BS because the MSS can skip the IP re-establishment or paging zone update if the MSS/BS choose a target BS located in same IP subnet or same Paging zone.

In this contribution we propose a modified MOB_NBR-ADV message with flags to indicate IP subnet and Paging zone equality of neighbor BSs.

# Overview of Proposed Solution

[TBD]

# Proposed Text Changes

3.1 HO Message Modifications

[Modify the Neighbor Advertisement (MOB-NBR-ADV) message, to indicate whether neighbor BSs are located in same IP subnet and same Paging zone]

3.2 Text change

[TBD]