| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Modification for Draft 802.16m Evaluation Methodology Document: Email Traffic Model |  |
| Date Submitted | 2007-9-18 |  |
| Source(s) | Yih-Guang Jan, Yang-Han Lee Department of Electrical Engineering, Tamkang University, Taiwan Chih-Wei Su Institute for Information Industry, Taiwan Bong Ho Kim, Posdata Jeongho Park, Samsung Hua Xu, Motorola | yihjan@yahoo.com cwsu@iii.org.tw bhkim@posdata-usa.com jeongho.jh.park@samsung.com Hua.Xu@motorola.com |
| Re: | IEEE 802.16m-07/080r3, “Call for Comments on Draft 802.16m Evaluation Methodology Document |  |
| Abstract | This contribution proposes a new section, “Email Traffic Model” for the Draft 802.16m Evaluation Methodology Document |  |
| Purpose | Add the propose email traffic model text and table in the EVM document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text and Table for Draft 802.16m Evaluation Methodology:

Email Traffic Model

Yih-Guang Jan, Yang-Han Lee

Department of Electrical Engineering,

Tamkang University, Taiwan

Chih-Wei Su

Institute for Information Industry, Taiwan

Bong Ho Kim, Posdata
Jeongho Park, Samsung

Hua Xu, Motorola

# I. Introduction

Email is one of the major applications that plays an important role and occupies a high percentage of traffic in the Internet traffics. Email application is considered in the UMTS Forum 3G traffic model and ITU R M.2072.  It, however, is not considered in the traffic model section in the IEEE 802.16m Evaluation Methodology document.

# II. Text Proposal

----------------------------------------------Start of the text-----------------------------------------------------------------

[Add the following references after the line#8 of the page#15 in C802.16m-07/080r3]

- UMTS Forum, 3G Offered Traffic Report, 30 June 2003

- ITU R M.2072, World mobile telecommunication market forecast

- Bong Ho Kim, and Y Hur, “Application Traffic Model for WiMAX Simulation,” POSDATA, Ltd, April 2007

- Laura A. Dabbish, Robert E. Kraut, Susan Fussell and Sara Kiesler, “Understanding Email Use: Predicting Action on a Message,” Proceedings of the ACM Conference on Human Factors in Computing Systems (CHI’05), NY: ACM Press, pp.691-700.Ffff

- V. Bolotin, Y. Levy, and D. Liu,” Characterizing Data Connection and Messages by Mixtures of Distributions on Logarithmic Scale, ITC 99, Edinburgh

- G.. Brasche, B. Walke, “ Concepts Services, and Protocols of the New GSM Phase 2+ General Packet Radio Service, IEEE Communications Magazine, August 1997

- M. S. Borella,” Source Models of Network Game Traffic”, Computer Communications, 23 (4), pp. 403-410

---------------------------------------------------------End of the text---------------------------------------------------------

.

----------------------------------------------------------Start of the text--------------------------------------------------------

[add text and Table:  after line#11 of the page#114 in C802.16m-07/080r3]

# 10.7 Email Traffic Model

Email is one of the major applications that plays an important role and occupies a high percentage of traffic in the Internet traffics, and email application is considered in the UMTS Forum 3G traffic model and ITU R M.2072 [1][2].

Interactions between email servers and clients are governed by email protocols.  The three most common email protocols are POP, IMAP and MAPI.  Most email software operates under one of these (and many products support more than one).  The Post Office Protocol (currently in version 3, hence POP3) allows email client software to retrieve email from a remote server.  The Internet Message Access Protocol (now in version 4 or IMAP4) allows a local email client to access email messages that reside on a remote server.  The Messaging Application Programming Interface (MAPI) is a proprietary email protocol of Microsoft that can be used by Outlook to communicate with Microsoft Exchange Server.  It provides somewhat similar but more functionality than an IMAP protocol.

The email traffic model in this section considers both POP3 and MAPI since these protocols generate different traffic patterns. To model POP3, FTP model can be used, and an email transaction with MAPI protocol can be modeled with multiple MAPI segment transactions in series. Each MAPI fragment is transmitted using TCP protocol and segmented into smaller segments again based on the TCP configuration. The maximum MAPI fragment size of 16896 bytes has been found so far, and this information is indicated in the first packet of a MAPI fragment. Outlook finishes all the TCP ACK packet transmission for the current MAPI segment and the Exchange server waits for the MAPI fragment completion indication packet before sending the next one. The last packet in the MAPI fragment sets the “PUSH” bit in the TCP packet to transmit all of the packets in the TCP buffer to the application layer at the receiver side [3].

The Email traffic can be characterized by ON/OFF states. During the ON-state an email could be transmitted or received, and during the OFF-state a client is writing or reading an email. Figure 1 depicts a simplified email traffic pattern.

Figure 1 Email Traffic model

The parameters for the email traffic model are summarized in Table 1 [3][4][5][6][7].

Table 1 Email Traffic Model

| Parameter | Distribution | Parameters | PDF |
| --- | --- | --- | --- |
| E-Mail Protocol | N/A | POP3, MAPI | N/A |
| E-Mail Average Header Size (Bytes) | Deterministic | 1 K | N/A |
| Number of email receive | Lognormal | Mean μ= 30 Std σ = 17 | fx=12πσexp−(lnx−μ)22σ2x≥0 |
| Number of email send | Lognormal | Mean μ= 14 Std σ = 12 | fx=12πσexp−(lnx−μ)22σ2x≥0 |
| Email reading time (sec) | Pareto | α=1.1,,k=2,m=65, mean = 60, maximum = 63 | fx=αkαxα+1,k≤x≤mfx=kmα,x=m |
| Email writing time (sec) | Pareto | α=1.1,,k=2,m=125, mean = 120, maximum = 123 | fx=αkαxα+1,k≤x≤mfx=kmα,x=m |
| Size of email receive/send without attachment (Kbytes) | Cauchy | median μ=22.7 Kbytes, 90%-tile = 80Kbytes | fx=Aπ((x−μ)2+1), A is selected to satisfy 90%-tile value |
| Size of email receive/send with attachment (Kbytes) | Cauchy | median μ=227 Kbytes , 90%-tile = 800 Kbytes | fx=Aπ((x−μ)2+1), A is selected to satisfy 90%-tile value |
| Ratio of email with attachment | Deterministic | Without attachment: 80% With attachment: 20% | N/A |

-------------------------------------------------------End of the text--------------------------------------------------------

----------------------------------------------------------Start of the text--------------------------------------------------------

[Modify the Table 39 on page#116 in C802.16m-07/080r3]

- section number change from “10.7 Traffic Mixes” to “10.8 Traffic Mixes.”

- Add one row between "VT only" and "Traffic Mix", and put "Email only" in the left most field.

- Add one column between "VT" and "Full Buffer,” and put “Email” in the top most field.

- Put 15% in the field crossing the "Traffic Mix" row and "Email" column.

- Change the value for the HTTP, NRTV, Gaming fields in the "Traffic Mix" to 15%.

-------------------------------------------------------End of the text--------------------------------------------------------.