| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Modification for Draft 802.16m Evaluation Methodology Document: 4. Link-to-System Mapping |  |
| Date Submitted | [The date this revision of the document is uploaded, in the format 2007-06-19. Tell the truth. Automatic numbering is prohibited.] |  |
| Source(s) | Hyunkyu Yu Jeongho Park xxx Samsung Electronics | hk.yu@samsung.com, jeongho.jh.park@samsung.com, |
| Re: | IEEE 802.16m-07/023– Call for Comments on Draft 802.16m Evaluation Methodology Document |  |
| Abstract | This contribution proposes to add the subsection to the Section ‘4.5. Per-tone SINR Computation’ for supporting simulation assumptions in the Section 2.2. |  |
| Purpose | For discussion and approval by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Modification on Section 4 Link-to-System Mapping

Hyunkyu Yu, Jeongho Park

Samsung Electronics

# I. Introduction

This contribution proposes text to Section 4 in the evaluation methodology document IEEE C802.16m-07/080r2.

In the Section 2.2 simulation assumptions, MIMO 2x2 (Matrix A & Matrix B) is specified as one of the baseline system assumption for the multi-antenna transmission format in downlink. The Section 4, however, only suggests the per-tone SINR description about Matrix B (MIMO multiplexing with the MMSE receiver) and does not include the description about Matrix A.

Therefore, we are proposing to add the subsection about the per-tone post processing SINR for STBC (Matrix A) in front of the Subsection 4.5.1.

# II. Proposed Text for Section on Per-tone SINR Computation

-------------------------------------------------------Start of the text------------------------------------------------------------

[Add the following text after the line#21 of the page62 in C802.16m-07/080r2]

4.5.1. Per-tone Post Processing SINR for MIMO STBC

In order to obtain the per tone post processing SINR for the MIMO STBC (matrix A), we consider a 2 transmit and  receive antennas system. The transmission scheme for Interferers is also assumed the STBC. The received signal at the n-th subcarrier in the 1st and the 2nd STBC symbol interval are expressed as

(x)

where

is the received antenna index,

is the received signal in the i-th STBC symbol interval for the target user, ,

is the transmitted symbol in the i-th STBC symbol interval, ,

is the channel gain between the t-th transmit and the r-th receive antenna, ,

is the receiver thermal noise in the i-th STBC symbol interval, .

The 1st and the 2nd STBC symbols are obtained through the following processes as

(y)

After decoding process of STBC, the post-processing SINR of the desired user for the n-th subcarrier is given as

.  (z)

Note that post-processing SINR for the 1st STBC symbol is the same as that for the 2nd STBC symbol and the equation above is expressed based on the 1st STBC symbol.

-------------------------------------------------------End of the text------------------------------------------------------------