| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MIMO Channel Models for 802.16m in Indoor Hotspot Scenario |  |
| Date Submitted | 2007-07-09 |  |
| Source(s) | Jianhua Zhang, Ding Xu, Lei Guan Beijing University of Posts & Telecommunications. Postal Add.: P.O. Box 92#, BUPT, Bei Jing 100876, China. Guanyi Liu, Zhigang Yan China Mobile No. 53A, Xi Bian Men Nei Avenue, Xuanwu District, Beijing, 100053, China | Voice: 86-010-62282107 E-mail: jhzhang@bupt.edu.cn xuading@gmail.com bupt.gluser@gmail.com Liuguangyi@chinamobile.com yanzhigang@chinamobile.com |
| Re: | Call for Comments on Draft 802.16m Evaluation Methodology Document Deadline: 9 July 2007 AOE” 2007-06-19, IEEE 802.16m-07/023r1 |  |
| Abstract | This document presents the MIMO channel model for 802.16m in indoor hotspot scenario. |  |
| Purpose | To be included entirely or partially in the evaluation documents (Draft IEEE 802.16m Evaluation Methodology Document, IEEE C802.16m-07/080r2) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# MIMO Channel Models for 802.16m in Indoor Hotspot Scenario

Jianhua Zhang*, Ding Xu*, Lei Guan*, Guanyi Liu+, Zhigang Yan+

*Beijing University of Posts & Telecommunications

+China Mobile

# Introduction

For the design and testing of wireless communication systems, channel model is an absolute necessity. IEEE 802.16m is aiming to meet the requirements of IMT-Advanced, so its channel model is more likely to involve wide bandwidth, high data rate, multiple antennas and some other aspects.

The test environments are considered as a basic factor in channel modeling, because different test environments have different channel parameters. For the simulation and evaluation of IEEE 802.16m systems, several test environments have been defined, such as urban macro-cell, suburban macro-cell, urban micro-cell, indoor small office and outdoor to indoor environments [1].

However, another important scenario has not been defined and modeled until now. As a mobile cellular system grows uneven traffic load, it will develop into “hotspot’’ cells, that is, the cells with the traffic load substantially higher than the designed load. The hotspot test environment concentrates on the propagation conditions in the urban cities with densely ranked tall buildings and the wider pedestrian streets, a lot of users, etc. In IMT-Advanced system, hotspot will meet the very higher traffic demands and will be an important part in evaluation.

In this contribution, channel models for indoor hotspot test environment are proposed, referring to the geometry based stochastic models and clustered delay line (CDL) models that defined in [2]. Details about measurement environment and channel models can be found in [3].

# Channel modeling approach for evaluations of proposed 802.16m air interface technologies

The proposed geometry-based stochastic model can be also called double directional channel model [2]. It does not explicitly specify the locations of the scatters, but rather the directions of the rays, like the well-known SCM model [4]. Modeling principles and parameters are determined stochastically, based on statistical characteristics obtained from channel measurements. Geometry-based modeling of the radio channel enables separation of propagation parameters and antenna geometries.

The concept of CDL models is a spatial extension of tapped delay line (TDL) models [2]. TDL models contain usually power, delay and Doppler spectrum information for the taps. CDL models define power, delay and angular information. Doppler is not explicitly defined, because it is determined by power and angular information combined with array configurations.

# Scenario

# Indoor Hotspot

The indoor hotspot test scenario concentrates on the propagation conditions in a hotspot in the urban with the very higher traffic, like the conference hall, shopping mall and teaching building. The indoor hotspot scenario is also different from the indoor office scenario due to the construction structure. Scenario A2 represents a typical shopping building, where the area per floor is about 5400 m2, number of floors is 8 and wider hall dimensions are different. The layout of the scenario is shown in Figure 1.

Figure 1

Layout of the indoor hotspot scenario

# Path Loss

Table 4-1 Summary table of the extended path-loss models

| Scenario |  | path loss [dB] | shadow fading standard dev. (dB) | applicability range and antenna height default values |
| --- | --- | --- | --- | --- |
| Indoor Hotspot | LOS | 11.8log10(d[m])+49.3+ 20log10(fc[GHz]/5.0) |  = 1.5 | 20 m < d <60 m hBS = hMS =1-2.5 m |
|  | NLOS | 43.3log10(d[m])+25.5+ 20log10(fc[GHz]/5.0) |  = 1.1 | 20 m < d <80 m hBS =hMS =1-2.5 m |

# Channel Model Parameters

Channel model parameters are given in the following sub-sections. The parameters are extracted from extensive radio channel measurements conducted in China-863 project and from existing literature [2], [4].

# Geometry Based Stochastic Models

The complete parameterization for the geometry-based stochastic models is given in Table 5-1.

Table 5-1 Channel model parameters

| Scenarios |  |  | Indoor Hotspot |  |
| --- | --- | --- | --- | --- |
|  |  |  | LOS | NLOS |
| Delay spread DS log10([s]) |  |  | -7.71 | -7.41 |
|  |  |  | 0.18 | 0.14 |
| AoD spread ASD log10([]) |  |  | 1.60 | 1.63 |
|  |  |  | 0.18 | 0.25 |
| AoA spread ASA log10([]) |  |  | 1.62 | 1.77 |
|  |  |  | 0.22 | 0.16 |
| Shadow fading SF [dB] |  |  | 1.5 | 1.1 |
| Cross-Correlations | ASD vs DS |  | 0.6 | 0.4 |
|  | ASA vs DS |  | 0.8 | 0.3 |
|  | ASA vs SF |  | -0.5 | -0.4 |
|  | ASD vs SF |  | -0.4 | -0.1 |
|  | DS vs SF |  | -0.8 | -0.5 |
|  | ASD vs ASA |  | 0.4 | -0.1 |
| Delay distribution |  |  | Exp | Exp |
| Delay scaling parameter r |  |  | 3.6 | 3 |
| XPRV [dB] |  |  | -0.17 | 9.32 |
|  |  |  | 0.97 | 3.73 |
| XPRH [dB] |  |  |  |  |
|  |  |  |  |  |
| AoD and AoA distribution |  |  | Laplacian |  |
| Number of clusters |  |  | 15 | 19 |
| Number of rays per cluster |  |  | 20 | 20 |
| Cluster ASD |  |  | 5 | 5 |
| Cluster ASA |  |  | 8 | 11 |
| Per cluster shadowing std  [dB] |  |  |  |  |
| K-factor [dB] |  |  | 15.3- 0.25d |  |
| Correlation distance [m] |  | DS | 8 | 5 |
|  |  | ASD | 7 | 3 |
|  |  | ASA | 5 | 3 |
|  |  | SF | 10 | 6 |

- Table 5-2 Expectation (median) output values for large scale parameters.

| Scenario |  | DS (ns) | AS at BS (º) | AS at MS (º) |
| --- | --- | --- | --- | --- |
| Indoor hotspot | LOS | 21 | 40 | 42 |
|  | NLOS | 34 | 43 | 59 |

# Reduced Variability Models

In the CDL model each cluster is composed of 20 rays with fixed offset angles and identical power. In the case of cluster where a ray of dominant power exists, the cluster has 20+1 rays. This dominant ray has a zero angle offset. The departure and arrival rays are coupled randomly. The CDL tables of interest are given in Table 5-3 and Table 5-4, where the cluster power and the power of each ray are tabulated. The CDL models offer well-defined radio channels with fixed parameters to obtain comparable simulation results with relatively simple channel models for calibration purpose.

The CDL parameters of LOS and NLOS condition are given below. In the LOS model Ricean K factor are 15.3 dB and 10.4 dB, respectively for the first and second clusters.

- Table 5-3 LOS Clustered delay line model, indoor hotspot.

| Cluster # | Delay [ns] | Power [dB] | AoD [º] | AoA [º] | Ray power [dB] |  | Cluster ASD = 5º | Cluster ASA = 8º |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 0 | -0.1* | -28.4** |  |  |
| 2 | 5 | -3.4 | 64 | -73 | -3.7* | -27.1** |  |  |
| 3 | 10 | -9.2 | 115 | 80 | -22.2 |  |  |  |
| 4 | 20 | -18.9 | 7 | 13 | -31.9 |  |  |  |
| 5 | 30 | -17.1 | 11 | 16 | -30.1 |  |  |  |
| 6 | 40 | -16.3 | -7 | -34 | -29.3 |  |  |  |
| 7 | 50 | -13.7 | -60 | -12 | -26.7 |  |  |  |
| 8 | 60 | -16.3 | -43 | -17 | -29.3 |  |  |  |
| 9 | 70 | -16.8 | 11 | -59 | -29.8 |  |  |  |
| 10 | 80 | -17.9 | 8 | -78 | -30.9 |  |  |  |
| 11 | 90 | -15.9 | 14 | -65 | -28.9 |  |  |  |
| 12 | 100 | -17.4 | -1 | -56 | -30.4 |  |  |  |
| 13 | 110 | -25.8 | -11 | -57 | -38.8 |  |  |  |
| 14 | 120 | -31.0 | -129 | -22 | -44.0 |  |  |  |
| 15 | 130 | -33.4 | -123 | -12 | -46.4 |  |  |  |

*  Power of dominant ray,

** Power of each other ray

- Table 5-4 NLOS Clustered delay line model, indoor hotspot.

| Cluster # | Delay [ns] | Power [dB] | AoD [º] | AoA [º] | Ray power [dB] | Cluster ASD = 5º | Cluster ASA = 11º |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -6.9 | 2 | 2 | -19.9 |  |  |
| 2 | 5 | 0 | -2 | 9 | -13.0 |  |  |
| 3 | 10 | -0.7 | -7 | 14 | -13.7 |  |  |
| 4 | 15 | -1.0 | 87 | -111 | -14.0 |  |  |
| 5 | 20 | -1.4 | -88 | 126 | -14.4 |  |  |
| 6 | 25 | -3.8 | -15 | -18 | -16.8 |  |  |
| 7 | 30 | -2.6 | 0 | -3 | -15.6 |  |  |
| 8 | 35 | -0.2 | -26 | -3 | -13.2 |  |  |
| 9 | 45 | -3.6 | -29 | 14 | -16.6 |  |  |
| 10 | 55 | -5.7 | 1 | 44 | -18.7 |  |  |
| 11 | 65 | -11.6 | 4 | 13 | -24.6 |  |  |
| 12 | 75 | -8.9 | -5 | 65 | -21.9 |  |  |
| 13 | 95 | -7.3 | -11 | 46 | -20.3 |  |  |
| 14 | 115 | -11.2 | -4 | 35 | -24.2 |  |  |
| 15 | 135 | -13.5 | -3 | 48 | -26.5 |  |  |
| 16 | 155 | -13.4 | -7 | 41 | -26.4 |  |  |
| 17 | 175 | -12.2 | 8 | 7 | -25.2 |  |  |
| 18 | 195 | -14.7 | 4 | 69 | -27.7 |  |  |
| 19 | 215 | -15.8 | -11 | 133 | -28.8 |  |  |

# Proposal

It is proposed that IEEE 802.16 task group m discusses this contribution and that Sections 3, 4 and 5 are included – entirely or partially – in the family of 802.16m channel models as appropriate.

# References

- IEEE C802.16m-07/080r2 , “Draft IEEE 802.16m Evaluation Methodology Document”.

- P. Kyösti (Ed.), “IST-WINNER II D1.1.1, WINNER II interim channel models”, V1.2, Feb. 2007. (https://www.istwinner.org/deliverables.html).

- PROPOSED new test environments and channel models of PRELIMINARY DRAFT NEW REPORT M.[IMT.EVAL],Document 8F/1252-E, Question ITU-R 229/8, 22nd Meeting of Working Party 8F Kyoto, 23-31 May 2007.

- 3GPP TR 25.996, “3rd Generation Partnership Project; technical specification group radio access networks; spatial channel model for MIMO simulations (release 6)”, V6.1.0.