| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | First Person Shooter Gaming Traffic Model for 802.16m Evaluation Methodology Document |  |
| Date Submitted | 2007-07-09 |  |
| Source(s) | Hua Xu, Pranav Joshi, Yi-Chiun Chen, Xiao Xu Motorola, Inc. 1501 West Shure Drive, Arlington Heights, IL 60004, USA Jeongeun Julie Lee, Belal Hamzeh Intel Corp. 2111 NE 25th Ave, Hillsboro, Oregon 97124, USA Bong Ho Kim Posdata Co, Ltd. 2901 Tasman Dr. Suite 219 Santa Clara, CA 95054, USA Ronny (Yong-Ho) Kim, Kiseon Ryu LG Electronics Inc. LG R&D Complex, 533 Hogye-1dong, Dongan-gu, Anyang, 431-749, Korea | E-mail: hua.xu@motorola.com E-mail: jeong.julie.lee@intel.com E-mail: bhkim@posdata-usa.com E-mail: ronnykim@lge.com |
| Re: | Response to a call for comments and contributions on draft 802.16m Evaluation Methodology Document |  |
| Abstract | In this document, the first person shooter gaming traffic model is proposed. |  |
| Purpose | Propose to modify the gaming traffic model in the IEEE 802.16m draft Evaluation Methodology Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Gaming Traffic Model

Hua Xu1, Pranav Joshi1, Yi-Chiun Chen1, Xiao Xu1, Jeongeun Julie Lee2, Belal Hamzeh2

Bong Ho Kim3, Ronny (Yong-Ho) Kim4, Kiseon Ryu4

1Motorola, Inc.   hua.xu@motorola.com

2Intel Corp.   jeong.julie.lee@intel.com

3Posdata Co, Ltd.   bhkim@posdata-usa.com

4LG Electronics Inc.   ronnykim@lge.com

The following slides provide the technical detail in supporting the changes indicated below.

[Change 10.6 as indicated.]

# 10.6. Gaming Traffic Model

Gaming is a rapidly growing application embedded into communication devices, and thus wireless gaming needs to be considered.  Games in different genre, such as First Person Shooter (FPS), Role Play Game (RPG), etc., show dramatic different traffic behaviors.  FPS model is recommended to represent the gaming traffic model in this document because it posts additional requirements to the system performance, such as real time delay with irregular traffic arrivals.

First Person Shooter (FPS) is a genre of video games.  It is a good representation of the modern Massively Multiplayer Online (MMO) game.  Due to the nature of the FPS game, it has stringent network delay requirement.  For the FPS game, if the client to server to client round trip delay (i.e., ping time, or end to end delay) is below 150ms, the delay is considered excellent.  When the delay is between 150ms to 200ms, the delay is noticeable especially to the experienced player.  It is considered good or playable.  When ping time is beyond 200ms, the delay becomes intolerable.

This end to end delay budget can be break down into internet delay, server processing delay, cellular network delay, air interface delay, and client processing delay, etc.  Let the IP packet delay be the time that the IP packet entering the MAC SDU buffer to the time that the IP packet is received by the client HARQ and reassembled into IP packet.  The IP packet delay is typically budgeted as 50ms to meet the 200ms end to end delay.   A gamer is considered in outage if 10% of its packet delay is either lost or delayed beyond the budget, i.e., 50ms.

The FPS traffic can be modeled by the Largest Extreme Value distribution. The starting time of a network gaming mobile is uniformly distributed between 0 and 40 ms to simulate the random timing relationship between client traffic packet arrival and reverse link frame boundary.  The parameters of initial packet arrival time, the packet inter arrival time, and the packet sizes are illustrated in Table 28.

For both the downlink and uplink, a packet is dropped if any part of the packet (including HARQ operation) has not started within 160msec of the time the packet entered the transmitting station’s buffer. Packet delay of a dropped or erased packet is counted as 180 ms. Currently understanding is that 50 ms lag is considered excellent quality while 100 ms lag is considered good quality. Ping times above 150 ms are often reported to be intolerable but many players claim to have no problmes with ping times around 200ms [55]. A user is in outage in wireless gaming if the per user average packet delay is greater than 60msec, where average delay includes the delay of packets delivered and the delay of packets dropped or erased between MS and BS. Some study shows that many players who experience high ping times of over 225 ms do not quit and continue to play. The impact of packet loss is rarely discussed as it is experienced as lag as well. However, a high ping time without packet loss is preferable to a small ping time with packet loss of around 10%. As per general system outage requirements, no more than [3]% of users can be in outage. Table 28 provides the traffic model parameters for ’Counter Strike’ internet gaming.

# [Change Table 28 on page 98 line 10 as indicated.]

| Component | Distribution |  | Parameters |  | PDF |
| --- | --- | --- | --- | --- | --- |
|  | DL | UL | DL | UL |  |
| Initial packet arrival | Uniform | Uniform | a=0, b=40 ms | a=0, b=40 ms | f(x)=1b−aa≤x≤b |
| Packet arrival time | Extreme | Deterministic Extreme | a=55 50 ms, b=6 4.5 ms | a = 40 ms b = 6 ms | f(x)=1be−x−abe−e−x−ab,b>0 [] Y∈U(0,1) |
| Packet size | Extreme | Extreme | a=120 330 bytes, b=36 82 bytes | a=80 45 bytes, b = 5.7 bytes | f(x)=1be−x−abe−e−x−ab,b>0 [X=a−bln−lnY+2,] [] Y∈U(0,1) |

Table 28.  Counter Strike FPS Gaming Traffic Model