| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Transmit power definition for evaluation methodology |  |
| Date Submitted | 2007-07-13 |  |
| Source(s) | Yuval Lomnitz, Intel Corporation Tom Harel, Intel Corporation Yang-seok Choi, Intel Corporation | yuval.lomnitz@intel.com tom.harel@intel.com yang-seok.choi@intel.com |
| Re: | IEEE 802.16m-07/023– Call for Comments on Draft 802.16m Evaluation Methodology Document |  |
| Abstract | Proposal to modify the definition of transmit power to allow consideration of PAPR effects (reduction/increase) |  |
| Purpose | For discussion and approval by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Transmit power definition for evaluation methodology

Yuval Lomnitz, Tom Harel, Yang-seok Choi

Intel Corporation

# Problem description

Currently the evaluation methodology document [2] defines DL/UL transmit power rather than 1dB compression point. Since different transmit methods may have different compression characteristics (backoffs), by defining transmit power we disregard the benefits of PAPR reduction methods.

# Discussion

In order to motivate modulation methods which are more power efficient, we propose that the transmit rms power is determined according to backoff. This ensures that modulations with lower PAPR which require smaller backoff will show an advantage in power-limited situations (and vice versa).

We suggest that the backoff be determined by condensing the signal through a PA model and measuring the spectral masks and EVM (rather than relying on CM/PAPR which are less accurate). We propose that the parameters associated with the PA model, spectral mask and EVM be defined in the evaluation methodology. There are many choices for these parameters but for evaluation purposes a single set is sufficient.

In order to refrain from complicating the simulations and requiring additional work, we propose that the transmit power numbers should be specified in the evaluation methodology document (rather than the 1dB compression point directly); however if proposals include modulation methods that change the backoff, then the document will specify how the transmit power is adapted. So as long as no change is made from the baseline OFDMA system there is no need to go through the procedure described.

The support required in system level simulation is only in configuring the maximum average transmit power and the EVM parameters per MCS/mode. These parameters will be computed off-line using the procedure described.

The proposed methodology applies for both downlink and uplink. Each link can be considered separately for evaluation purposes. In the uplink the power efficiency is critical and we expect possible improvement from PAPR reduction methods, therefore a methodology for considering this improvement is important. In the downlink we expect possible PAPR increase due to DL power control, beamforming/precoding, and the power efficiency may be less critical. This methodology allows a fair comparison between methods.

# Proposed text changes

2.3. Deployment Scenarios

[Make the following additions in table 3]

| BS Tx Power *see note 1 | 46 dBm | 46 dBm | 46 dBm | [TBD] | [46 dBm] |  |
| --- | --- | --- | --- | --- | --- | --- |
| MS Tx Power * see note 1 | 23 dBm | 23dBm | 23 dBm | [23 dBm] | [23 dBm] |  |
| EVM * see note 1 | 30dB | 30dB | 30dB | 30dB | 30dB |  |

Note 1:

Transmit power and EVM numbers for BS and MS reflect baseline values for OFDM transmission. In case other modulation methods are considered these numbers will be updated based on the procedure described in section 4.9. These numbers reflect maximum power (without power control).

[Add a new subclause]

4.9 Transmit power and EVM

Different modulation methods may have different PAPR and spectral characteristics, affecting the maximum output power. Table 3 specifies baseline output power and EVM values for the BS and MS, which are applicable for OFDM transmission. Methods that reduce or increase PAPR( beamforming) may be evaluated.

In the case that a proposed modulation method yields different PAPR and/or different spectral characteristics which affect the maximum output power, these numbers shall be calibrated accordingly.

Reference parameters required for calibration are defined below:

| Parameter | Value | Notes |
| --- | --- | --- |
| PA model | RAPP-2 (s=2) | AM/AM compression model. See below. |
| Spectral masks | FCC | See appendix XX |
| EVM (error vector magnitude) | To be specified in the proposal | May be chosen to optimize performance per MCS |
| Over sampling | 4 |  |
| RBW (resolution bandwidth) | 1% of signal bandwidth (100Khz for 10Mhz BW) | For emission measurement. See appendix XX |
| Reference OFDM transmission | Full bandwidth UL/DL PUSC |  |

Table XXX: reference parameters for TX power calibration

Equation (ZZ) defines the RAPP model. x(t),y(t) are the complex baseband representations of the PA input and PA output respectively, and the parameter s controls the smoothness of the curve. A value of s=2 will be used by default. It is also recommended to supply results for s=30 to represent a linearized PA. C is the saturation amplitude of the PA.

(ZZ)		y(t)=x(t)1+x(t)C2s12s

y(t)=x(t)1+x(t)C2s12s

The proponents should provide simulation results where the modulated signal is passed through a PA compression model and the spectral masks and EVM are computed. The maximum transmit power is the maximum power which meets the spectral masks and the required EVM. The maximum transmit power of flat, full bandwidth modulated OFDM reference signal shall be compared with the maximum effective transmit power of the proposed modulation (with the same PA and mask parameters), and the difference (power gain or loss) will be added to the BS/MS transmit power as defined in Table 3. . The saturation power C shall be set so that the maximum power that the reference OFDM system can transmit is according to the power defined in Table 3.The EVM may be chosen per MCS/mode and results in potentially different maximum transmit power per MCS. The EVM required for the reference OFDM system is as defined in Table 3. Effective transmit power and EVM are defined below.

EVM is defined as the ratio between the effective transmit power and the power of the error vector, both described below. Error vector power is measured over all subchannels, including unmodulated subchannels. Subcarriers which do not carry information for any user (guard, DC subcarriers,  reserved subcarriers for PAPR reduction) are not included (neither for the error calculation nor for the power calculation). The error signal may be computed using pilot based equalization (as described in [67] 802.16e-2005, subclause 8.4.12.3), or by comparing the transmitted signal with an undistorted (but possibly filtered) signal. In the second case since the distortion error is correlated with the signal, a suitable gain should be applied to undistorted signal such that the error signal becomes uncorrelated with the undistorted signal (and the error vector could be abstracted as additive uncorrelated noise).

The effective transmit power is defined as the power of the distorted signal which is correlative to the ideal signal (so that the power does not include neither the error vector nor any extra energy for PAPR reduction).

The error vector power and effective transmit power are accumulated in linear domain and their ratio is converted to dB. The EVM is accumulated over a single transmission (TTI). In case the EVM varies between different cases in the same transmission mode (e.g. between different subchannels), the 10% percentile shall be used.

4.5. Per-tone SINR Computation

[Change the text after eq(15) as follows (changes marked in blue)]

X (j) is the transmitted symbols by the j-th user/sector. X(j) is composed of desired signal plus transmit error vector with power lower by EVM2=10-EVM/10 than the signal effective power.

[Change equation 16 as follows]

SINR0n=PTX0Ploss0H0n2σ2+j=1NiPTXjPlossjGjn2+σEVM2⋅PTX0Ploss0H0n2

SINR0n=PTX0Ploss0H0n2σ2+j=1NiPTXjPlossjGjn2+σEVM2⋅PTX0Ploss0H0n2

[add new appendix]

Appendix XX (informative) - FCC spectral mask

The following table specifies FCC spectral mask regulations for mobile stations taken from FCC 27.53:

| Frequency band | Maximum signal power | RBW |
| --- | --- | --- |
| First 1 MHz from channel edge | -13dBm/RBW, | 1% of signal BW, for example 100KHz for 10 MHz signal |
| 1MHz to 5.5 MHz from channel edge | -13dBm/RBW | 1MHz |
| 5.5 MHz or more from edge | -25dBm/RBW | 1MHz |

Table YY: FCC spectral mask

References

[add the following reference to the existing references in IEEE C80216m-07_080r2]

[67]	IEEE 802.16e-2005 - IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems. Amendment 2: Physical and Medium Access Control Layers for Combined Fixed and Mobile Operation in Licensed Bands and Corrigendum 1)

# References for this contribution

  - IEEE 802.16e-2005 - IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems. Amendment 2: Physical and Medium Access Control Layers for Combined Fixed and Mobile Operation in Licensed Bands and Corrigendum 1)

  - IEEE C80216m-07-080r2 - Draft IEEE 802.16m Evaluation Methodology Document, 2007-06-18

  - FCC regulations: http://www.hallikainen.com/FccRules/2007/27/53/
http://www.access.gpo.gov/nara/cfr/waisidx_06/47cfr27_06.html (see 27.53 emission limits)