| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modifications to Appendix I: Overhead Calculations |  |
| Date Submitted | 2007-07-09 |  |
| Source(s) | Hujun Yin, Intel Corporation Roshni Srinivasan, Intel Corporation | hujun.yin@intel.com roshni.m.srinivasan@intel.com |
| Re: | IEEE 802.16m-07/023– Call for Comments on Draft 802.16m Evaluation Methodology Document |  |
| Abstract | Proposal to modify Appendix I: Overhead Calculations |  |
| Purpose | For discussion and approval by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modifications to Appendix I: Overhead Calculations

Hujun Yin, Roshni Srinivasan

Intel Corporation

# Introduction

Appendix I currently contains bracketed text. This contribution modifications (indicated by strikethrough and red color) text from section I-1 for inclusion in this Appendix proposes and deletion of bracketed text in section I-2.

It is also proposed that the title of the Appendix be changed to “Modeling of Overhead Channels and Signaling Errors”

# Proposed Text for Appendix I

[

# Overhead Channels

# Dynamical Simulation of the Downlink Overhead Channels

Dynamically simulating the overhead channels is essential to capture the dynamic nature of these channels. The simulations should be done as follows:

The performance of the overhead channels should be included in the system level simulation results unless the overhead channel is taken into account as part of fixed overhead e.g., if an overhead channel is time division multiplexed, and takes all the bandwidth, the percentage of time used translates into the same percentage decrease of throughput.

There are two possible types of overhead channels depending on the proposal:  static and dynamic. A static overhead channel requires fixed base station power and bandwidth. A dynamic overhead channel requires dynamic base station power and (or) bandwidth.

The demodulation performance (i.e., frame error rate) of the downlink control channel could be assessed using the link abstraction method used to model traffic channels, with proper modifications, if necessary, to reflect any difference in the transmission or coding format of the control channel.

[The link level performance should be evaluated off-line by using separate link-level simulations.  The performance is characterized by curves of detection, miss, false alarm, and error probability (as appropriate) versus Eb/No (or some similar metric depending on the interface between the link and system simulations).

The system level simulations need not directly include the coding and decoding of overhead channels.  The link level performance should be evaluated off-line by using separate link-level simulations.  The link level performance is characterized by curves of detection, miss, false alarm, and error probability (as appropriate). There are two aspects that are important for the system level simulation:  the required Ec/Ior (or some similar metric depending on the interface between the link and system simulations) during the simulation interval, and demodulation performance (detection, miss, and error probability — whatever is appropriate).]

For static overhead channels, the system simulation should compute the received [Eb/No (or similar metric)] [SINR] and predict the demodulation performance.

For dynamic overhead channels with open-loop control (if used), the simulations should take into account the estimate of the required downlink power or bandwidth that may be needed to be transmitted to the mobile station for transmission of the overhead channels.  During the reception of overhead information, the system simulation should compute the received [SINR] [Eb/No (or similar metric)].

Once the received [SINR] [Eb/No (or similar metric)] is obtained and the frame error rate is predicted, then the various miss error events should be determined.  The impact of these events should then be modeled.   The false alarm events are evaluated in link-level simulation, and the simulation results shall be included in the evaluation report. The impact of false alarm, such as delay increases and throughput reductions for both the downlink and uplink, shall  the impact of the detection, miss, false alarm, error probability should be appropriately taken into account in system-level simulation.

All overhead channels should be modeled or accounted for.

If a proposal adds messages to an existing channel (for example sending control on a data channel), the proponent should justify that this can be done without creating undue loading on this channel. The system level and link level simulation required for this modified overhead channel as a result of the new messages should be performed according to 3) and 4), respectively.

# Uplink Modeling in Downlink System Simulation

The proponents should model feedback errors (e.g. power control, acknowledgements, rate indication, etc.) and measurements (e.g. C/I measurement). In addition to supplying the feedback error rate average and distribution, the measurement error model and selected parameters, the estimated power level required for the physical reverse link channels should be supplied.

# Signalling Errors

Signaling errors should be modeled and specified as in the following table.

| Signaling Channel | Errors | Impact |
| --- | --- | --- |
| ACK/NACK channel (if proposed) | Misinterpretation, missed detection, or false detection of the ACK/NACK message | Transmission (frame or encoder packet) error or duplicate transmission |
| Explicit Rate Indication (if proposed)/ mode selection | Misinterpretation of rate / mode selection | One or more Transmission errors due to decoding at a different rate (modulation and coding scheme) or selection of a different mode |
| User identification channel (if proposed) | A user tries to decode a transmission destined for another user; a user misses transmission destined to it. | One or more Transmission errors due to HARQ/IR combining of wrong transmissions |
| Rate or C/I feedback channel (if proposed) | Misinterpretation of rate or C/I | Potential transmission errors |
| Transmit sector indication, transfer of H-ARQ states etc. (if proposed) | Misinterpretation of selected sector; misinterpretation of frames to be retransmitted. | Transmission errors |

Table 1: Signaling Errors

Proponents should quantify and justify the signaling errors and their impacts in the evaluation report.

]

[

# Calculation of Overhead

# Calculation of L1 overhead

The L1 overhead shall be accounted for the purpose of calculation of system performance metrics such as spectral efficiency, user throughput, etc. The following are the list of L1 overhead that shall be accounted for in the overhead calculation:

- Number of sub-carriers that carry preamble (Npreamble)

- Number of sub-carriers that are used as the guard carriers and DC sub-carrier (Nguard).

- Number of sub-carriers that are assigned to the TX/RX switching points (Ngap) (for TDD duplex scheme only).

- Number of sub-carriers that are used to carry pilots (Npilot) in downlink or uplink sub-frame.

- The cyclic prefix (CP) or the guard interval (GI) is accounted for through the actual number of OFDM symbols within a frame.

Given transmission bandwidth (BW) and sub-carrier spacing (1/T when there is no cyclic prefix), the ideal number of sub-carriers per OFDM symbol becomesBW⋅T. The number of OFDM symbols over frame duration (Tf) excluding L1 overhead can be expressed asTfT. Hence, the total number of available sub-carriers over BW and Tf becomesM=BW⋅Tf.

However, due to L1 overhead, the actual number of available sub-carriers (N) over a frame and the transmission bandwidth will be smaller than M.

L1overhead=M−NM×100 in %

As an example in IEEE 802.16e reference system, M=10  MHz * 5 ms = 50000. Using DL PUSC, 47 OFDM symbols are available (including the preamble and cyclic prefix overhead). The number of usable sub-carriers for both data and pilot is 840*47 including the effect of guard sub-carriers. In PUSC, the number of pilots is 840*47/56*8=5640. Thus, N is equal to 840*47*48/56=33840. Hence, the L1 overhead becomes 32.3%.

# Calculation of L1+L2 overhead

The L2 overhead includes the following:

- The number of OFDM symbols (or sub-carriers) that are used for MAP

- The average number of ODFM symbols (or sub-carriers) that are used for system configuration information (FCH, DCD/UCD).

- The number of OFDM symbols (or sub-carriers) that are used for ACK/NACK,

- The number of OFDM symbols (or sub-carriers) that are used for CQICH,

- The number of OFDM symbols (or sub-carriers) that are used for Ranging

Let L denote the number of sub-carriers over a frame and the bandwidth excluding L2 overhead such as MAP, control channel, etc. Then, the L1+L2 overhead is defined

L1+L2overhead=M−LM×100 in %.

The L2 overhead can be written as

L2overhead=N−LN×100 in %.

Note thatL1+L2overhead≠L1overhead+L2overhead.

]