| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Baseline Simulation Test Scenario for Calibration of System Performance Metrics |  |
| Date Submitted | 2007-07-18 |  |
| Source(s) | Roshni Srinivasan, Sassan Ahmadi Intel Corporation Ken Stewart, Mark Cudak, Jeff Zhuang Motorola Adam Pollard Vodafone John Humbert Sprint | roshni.m.srinivasan@intel.com sassan.ahmadi@intel.com ken.stewart@motorola.com mark.cudak@motorola.com jeff.zhuang@motorola.com adam.pollard@vodafone.com John.J.Humbert@sprint.com |
| Re: | IEEE 802.16m-07/023– Call for Comments on Draft 802.16m Evaluation Methodology Document |  |
| Abstract | Definition of a baseline test scenario in draft 802.16m Evaluation Methodology Document for evaluation of system performance metrics in 802.16m SRD |  |
| Purpose | For discussion and approval by TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Baseline Simulation Test Scenario for Calibration of System Performance Metrics

Roshni Srinivasan, Sassan Ahmadi, Intel Corporation
Ken Stewart, Jeff Zhuang, Mark Cudak,  Motorola
Adam Pollard, Vodafone
John Humbert, Sprint

# Introduction

This contribution defines the necessary assumptions and configuration of a baseline simulation scenario that should be used calibrating system performance metrics in the 802.16m SRD [1]. The baseline simulation scenario should be included the Draft 802.16m Evaluation Methodology Document [2]

The configuration parameters are largely aligned with NGMN evaluation criteria [3] and address the pedestrian/vehicular test environment defined in IMT-EVAL.

# Proposed Text

It is proposed that Table 3 in Section 2.3 of the Draft 802.16m Evaluation Methodology Document [2] be replaced by the following table:

| Scenario/ Parameters | Baseline Configuration | Urban Macrocell | Suburban Macrocell | Urban Microcell | [Indoor Small Office] | [Outdoor to Indoor] |
| --- | --- | --- | --- | --- | --- | --- |
| Mandatory/Optional | Mandatory | Mandatory | Mandatory | Mandatory | [TBD] | [TBD] |
| Site-to-Site distance | 0.5 km 1.732 km | 1.5 m | 1.5 km | 0.5 km | [50 m] | [TBD] |
| BS Height | 32 m | 32 m | 32 m | 12.5 m | [1-2.5m] | [TBD] |
| BS Tx Power | 46 dBm | 46 dBm | 46 dBm | 46 dBm | [TBD] | [46 dBm] |
| MS Tx Power | 24 dBm | 23 dBm | 23 dBm | 23 dBm | [23 dBm] | [23 dBm] |
| MS Height | 1.5 m | 1.5 m | 1.5 m | 1.5 m | [1.5m] | [1.5m] |
| Penetration Loss | 20.0 (includes MS antenna gain and BS cable loss) | 0 dB | 0 dB | 0 dB | [0 dB] | [0 dB] |
| Path Loss Model | Loss (dB) = 128.1+37.6log10(R) (R in km) | Refer to Section 3.2.3.1 | Refer to Section 3.2.3.2 | Refer to Section 3.2.3.3 | [Refer to Section 3.2.3.4] | [Refer to Section 3.2.3.5] |
| Lognormal Shadowing Standard Deviation | 8 dB | 8 dB | NLOS 8 dB LOS 4 dB | NLOS 4 dB LOS 3 dB | [NLOS (Room to corridor) 8 dB NLOS (Through light wall) 6 dB NLOS (Through heavy wall) 8 dB] | [7 dB] |
| Mobility | Ped / Veh | 0-350 kmph | 0-350 kmph | 0-120 kmph | [TBD] | [TBD] |
| Channel Mix | ITU Ped B 3 km/hr – 60% ITU Veh A 30 km/hr – 30% ITU Veh A 120 km/hr – 10% | TBD | TBD | TBD | [TBD] | [TBD] |
| Spatial Channel Model | ITU with spatial correlation a) Uncorrelated b) Correlated (Corr. Coeff. = 0.8) | TBD | TBD | TBD | [TBD] | [TBD] |

It must be noted that the specified baseline configuration results in four cases for the defined site-to-site distances and spatial correlation factors.

All other simulation assumptions for the baseline test scenario are defined in section 2.2 of the Draft 802.16m Evaluation Methodology Document [2]. Additionally, section 2.4 of [2] defines all parameters required for calibration with the 802.16e reference system in evaluating relative performance metrics.

# References

- Draft IEEE 802.16m Requirements (80216m-07_002r2.pdf)

- Draft IEEE 802.16m Evaluation Methodlology Document (C80216m-07_080r2.pdf)

- Next Generation Mobile Networks Radio Access Performance Evaluation Methodology, NGMN White Paper, June 2007 (L80216-07_042)