| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Evaluation Methodology Ad-Hoc Group Report |  |
| Date Submitted | 2007-07-19 |  |
| Source(s) | Roshni Srinivasan, Intel Corporation Editor, 802.16m Draft Evaluation Methodology Document TGm Evaluation Methodology Ad-Hoc Group | roshni.m.srinivasan@intel.com |
| Re: | Updates to Draft 802.16m Evaluation Methodology Document |  |
| Abstract | Definition of a baseline configuration in draft 802.16m Evaluation Methodology Document for evaluation of system performance metrics in 802.16m SRD |  |
| Purpose | For discussion and approval by TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Evaluation Methodology Ad-Hoc Group Report

Roshni Srinivasan, Intel Corporation and

TGm Evaluation Methodology Ad-Hoc Group

This contribution summarizes the discussion and outputs of the Evaluation Methodology Ad-Hoc Group constituted in TGm on 07-18-07 to define the baseline configuration for calibrating system performance metrics in the 802.16m SRD [3].

Updates to the table listing test scenarios for simulation provided in [1] and changes have been tracked for reference. This table includes harmonized test scenarios from [2] and an additional baseline configuration which specifies that parameters and values should be used for calibrating system performance metrics in the 802.16m SRD [3]. The configuration parameter values are largely aligned with the NGMN evaluation criteria [4].

It is proposed that Table 3 in Section 2.3 of the Draft 802.16m Evaluation Methodology Document [2] be replaced by Table 1 in this report. Test scenarios that were identified as optional scenarios are listed in Table 2 which may be included in an annex. Parameters for the harmonized test scenarios in Table 1 need to be defined Ooptional scenarios in Table 2 are bracketed and need to be defined.

In order to limit the number of baseline configurations that will need to be simulated and calibrated to provide input to the 802.16m SRD, the following discussion items require further resolution by the working group:

- PUSC or AMC for evaluation of reference system performance. A straw poll in the ad-hoc group indicated strong support for PUSC only for the baseline configuration.

- EESM or MI for link-to system mapping (related comments in Evaluation Methodology not yet resolved) for the baseline configuration.

- The choice of frequency reuse of 1 or 3 for the baseline configuration.

- The use of the word ‘baseline’ should be clarified in of the Draft 802.16m Evaluation Methodology Document. It is currently defined as follows in section 2: “Baseline assumptions are specified for comparison with the reference system as defined by the 802.16m requirements.” Currently these assumptions clearly specify configuration parameters related to allowed options in the reference system as defined in the SRD.

- Harmonized test scenarios in Table 1 in this report need to be classified as optional or mandatory for 802.16m performance evaluation.

# Proposed Text

It is proposed that Table 3 in Section 2.3 of the Draft 802.16m Evaluation Methodology Document [2] be replaced by the following table:

| Scenario/ Parameters | Baseline Configuration | Urban Macrocell | Suburban Macrocell | Urban Microcell |
| --- | --- | --- | --- | --- |
| Requirement |  |  |  |  |
| Site-to-Site distance | 0.5 km | 1.5 m | 1.5 km | 0.5 km |
| Carrier Frequency | 2.5 GHz | 2.5 GHz | 2.5 GHz | 2.5 GHz |
| Operating Bandwidth | 10 MHz for TDD | 10 MHz for TDD / 5 MHz per uplink and downlink for FDD | 10 MHz for TDD / 5 MHz per uplink and downlink for FDD | 10 MHz for TDD / 5 MHz per uplink and downlink for FDD |
| BS Height | 32 m | 32 m | 32 m | 12.5 m |
| BS Tx Power per sector | 46 dBm | 46 dBm TDD 43 dBm FDD | 46 dBm TDD 43 dBm FDD | 46 dBm TDD 43 dBm FDD |
| MS Tx Power | 23 dBm | 23 dBm | 23 dBm | 23 dBm |
| MS Height | 1.5 m | 1.5 m | 1.5 m | 1.5 m |
| Penetration Loss | 20.0 | TBD | TBD | TBD |
| Path Loss Model | Loss (dB) = 128.1+37.6log10(R) + 2.52 (R in km) | Refer to Section 3.2.3.1 | Refer to Section 3.2.3.2 | Refer to Section 3.2.3.3 |
| Lognormal Shadowing Standard Deviation | 8 dB | 8 dB | NLOS 8 dB | NLOS 4 dB LOS 3 dB |
| Correlation distance for shadowing | 50m | TBD | TBD | TBD |
| Mobility | 3 km/hr | 0-120 kmph | 0-350 kmph | 0-10 kmph |
| Channel Mix | ITU Ped B * 3 km/hr | TBD | TBD | TBD |
| Spatial Channel Model | ITU with spatial correlation a) Uncorrelated b) Correlated (BS Correlation Coefficient = 0.5) | TBD | TBD | TBD |

Table 1: Test Scenarios

* Proposal to extend ITU profiles provided in C80216m-07_157r1

It must be noted that the specified baseline configuration results in two cases for the defined spatial correlation factors.

All other simulation assumptions for the baseline test scenario are defined in section 2.2 of the Draft 802.16m Evaluation Methodology Document from [2]. Additionally, section 2.4 of from [2] defines all parameters required for calibration with the 802.16e reference system in evaluating relative performance metrics.

It is proposed that the following table be included as an annex in [2].

| Scenario/ Parameters | Indoor Small Office | Outdoor to Indoor | Indoor Hotspot | Rural |
| --- | --- | --- | --- | --- |
| Requirement | Optional | Optional | Optional | Optional |
| Site-to-Site distance | [50 m] | [TBD] | [80m] | [TBD] |
| Carrier Frequency | 2.5 GHz | 2.5 GHz | 2.5 GHz | 2.5 GHz |
| Operating Bandwidth | 10 MHz for TDD / 5 MHz per uplink and downlink for FDD | 10 MHz for TDD / 5 MHz per uplink and downlink for FDD | 10 MHz for TDD / 5 MHz per uplink and downlink for FDD | 10 MHz for TDD / 5 MHz per uplink and downlink for FDD |
| BS Height | [1-2.5m] | [TBD] | [1~2.5m] | [TBD] |
| BS Tx Power | [TBD] | [46 dBm] | [TBD] | [TBD] |
| MS Tx Power | [23 dBm] | [23 dBm] | [23 dBm] | [TBD] |
| MS Height | [1.5m] | [1.5m] | [1.5m] | [TBD] |
| Penetration Loss | [0 dB] | [0 dB] | [TBD] | [TBD] |
| Path Loss Model | [Refer to Section 3.2.3.4] | [Refer to Section 3.2.3.5] | [Refer to Section 3.2.3.6 to be created] | [TBD] |
| Lognormal Shadowing Standard Deviation | [NLOS (Room to corridor) 8 dB NLOS (Through light wall) 6 dB NLOS (Through heavy wall) 8 dB] | [7 dB] | [LOS] [TBD] [NLOS] [TBD] | [TBD] |
| Mobility | [TBD] | [TBD] | [0~3Kmph] | [TBD] |
| Channel Mix | [TBD] | [TBD] | [TBD] | [TBD] |
| Spatial Channel Model | [TBD] | [TBD] | [TBD] | [TBD] |

Table 2: Optional Test Scenarios

The following reference should be added to the Draft 802.16m Evaluation Methodology Document from [2].

- Next Generation Mobile Networks Radio Access Performance Evaluation Methodology, NGMN White Paper, June 2007 (L80216-07_042)

# References for this contribution

- R. Srinivasan et al., Baseline Simulation Test Scenario for Calibration of System Performance Metrics, July 2007 ( C80216m-07_156.doc)

- Draft IEEE 802.16m Evaluation Methodology Document (C80216m-07_080r2.pdf)

- Draft IEEE 802.16m Requirements (80216m-07_002r2.pdf)

- Next Generation Mobile Networks Radio Access Performance Evaluation Methodology, NGMN White Paper, June 2007 (L80216-07_042)