| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed SRD section 6.1 modifications |  |
| Date Submitted | 19-July-2007 |  |
| Source(s) | John Humbert Sprint | Voice: (816) 210-9611 E-mail: John.J.Humbert@sprint.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | 802.16m-07/002r2 – TGMREQ |  |
| Abstract | Harmonized Peak Data Rate Requirements |  |
| Purpose | Adopt contribution |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Replace section 6.1 with the following text:

6.1 Peak data rate

This section defines the peak data rate achievable between a BS and a MS under ideal conditions.

IEEE 802.16m shall support the peak rate, expressed as a normalized peak rate (i.e. absolute maximum supported data rate divided by the occupied channel bandwidth), as specified in Table 1.

Table 1–Normalized peak data rate

| Requirement Type | Link Direction | MIMO Configuration | Normalized Peak Data Rate (bps/Hz) |
| --- | --- | --- | --- |
| Baseline | Downlink | 2x2 | 8.0 |
|  | Uplink | 1x2 | 2.8 |
| Target | Downlink | 4x4 | 15.0 |
|  | Uplink | 2x4 | 9.3 7.8 |
|  | Uplink | 1X2 | 4.7 3.9 |

Notes applicable to Table 1:

- The baseline  requirement denotes the minimum peak data rate achievable between a BS and an MS equipped with the minimum supported antenna configuration defined in Section 5.7.

- The target requirement denotes the minimum peak data rate achievable between a BS and a MS equipped with a higher order antenna configuration, as specified in Table 1, that exceeds the minimum MS antenna configuration.

- Other MIMO antenna configurations beyond those of Table 1 can be used. For example, the extended uplink requirement for the 2x4 configuration would apply to an uplink 2x2 configuration etc.

- The specified requirements of normalized peak rates are not distinguished by duplex mode. Rather, 100% of available radio resources are assumed – for the purposes of computing Table 1– allocable to downlink and uplink respectively regardless of duplexing mode. For example, for TDD, when assessing downlink performance, all available radio resources are assigned for downlink transmission.

- Table 1 accounts for layer 1 overhead due to provisioning of radio resources for essential functions such as pilots, cyclic-prefix, guard bands and guard intervals.

- The specified minimum supported normalized peak rates are applicable to all bandwidths in Section 5.