| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Collaborative MIMO Based on Multiple Base Station Coordination |  |
| Date Submitted | 2007-08-29 |  |
| Source(s) | Yang Song, Liyu Cai, Keying Wu, Hongwei Yang Alcatel-Lucent | Voice: + 1 973 428 7734 E-mail: dgal@alcatel-lucent.com |
| Re: | IEEE 802.16m Requirements - Elaboration of MIMO Techniques |  |
| Abstract | This contribution is to introduce a collaborative MIMO (Co-MIMO) technique, which realizes a joint MIMO transmission from multiple BSs towards one or multiple MSs through BS coordination to reduce the ICI and improve the system coverage and spectral efficiency. The conceptual architecture and working principle of Co-MIMO are also described. As the precoding operation in Co-MIMO is performed individually within each BS, there is no need for excessive CSI exchange among coordinating BSs, which leads to very limited information transfer over the backbone as well as low computational complexity. |  |
| Purpose | For 802.16m discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Collaborative MIMO Based on Multiple Base Station Coordination

Yang Song, Liyu Cai, Keying Wu, Hongwei Yang

Alcatel-Lucent Research and Innovation

# Introduction

The capacity of a cellular wireless network is often limited by inter-cell interference (ICI), especially with low frequency reuse factor. If a mobile station (MS) is located at the cell boundary, interference from neighboring cells is a crucial factor leading to severe performance degradation in terms of throughput and coverage. To combat the ICI, some techniques such as frequency reuse and power control have been raised. However, they are radio resource consuming resulting in low spectral efficiency.

In this proposal, we present a collaborative MIMO (Co-MIMO) technique, which realizes a joint MIMO transmission from multiple BSs towards one or multiple MSs through BS coordination to reduce the ICI and improve the system coverage and spectral efficiency. This technique has the advantages of low backbone requirement and low computational complexity.

# Concept of Collaborative MIMO

Co-MIMO is an extension of conventional single-BS based MIMO techniques. It allows multiple BSs to serve one or multiple MSs simultaneously over the same radio resource through coordination among BSs. Co-MIMO is characterized by the following three features:

1) In Co-MIMO, each MS is jointly served by multiple BSs, which is different from conventional MIMO techniques where each MS is only served by a single BS. Through BS coordination, the ICI among these coordinating BSs can be reduced greatly especially in the case of low frequency reuse.

2) In Co-MIMO, each BS can also serve several MSs simultaneously using the same radio resource in order to further increase the system throughput. The co-channel interference (CCI) among these MSs can be minimized by spatial division multiple access (SDMA) such as beamforming or multi-user MIMO (MU-MIMO) techniques.

3) In Co-MIMO, SDMA processing is implemented independently within individual coordinating BS based on the channel state information (CSI) between this BS and its served MSs. Therefore, Co-MIMO limits the amount of information exchange between BSs, which is different from other BS coordination approaches requiring massive information exchange.

In summary, Co-MIMO has the advantages of ICI mitigation and spectral efficiency improvement. Moreover, it provides an acceptable tradeoff between the performance gain and the stringent requirement on the backbone traffic and computational complexity.

The area which is covered jointly by multiple coordinating BSs through Co-MIMO transmission is defined as “collaborative region” in this proposal. Figure 1 gives an illustration of Co-MIMO where three neighboring BSs jointly serve three MSs in the collaborative region denoted by the hexagonal shadowed area. The scheduler, a logical functional module in the network, is responsible for collecting the information from coordinating BSs and determining the serving relationship between BSs and MSs. Figure 1 shows an example of the serving relationship: MS1 is served by BS1 and BS3, MS2 by BS1 and BS2, and MS3 by BS2 and BS3. Furthermore, more BSs can be involved into the coordination for Co-MIMO transmission depending on the acceptable complexity and the backbone capability. In the wireless network realized by distributed antenna system, Co-MIMO can also be realized among the distributed antenna arrays or remote radio heads connected with a central processing unit.

Figure 1  An example of Collaborative MIMO

A conceptual Co-MIMO architecture is depicted in Figure 2. In this figure, M coordinating BSs are connected with a scheduler via backbone. BSi (i =1,…, M) contains a precoding matrix generator which calculates its own precoding matrix Wi to implement SDMA. At the BS side, Co-MIMO may work in the following steps:

1) Every BS reports to the scheduler certain channel information, such as receive signal strength indicators (RSSIs) or channel quality indicators (CQIs), between MSs and itself through backbone.

2) The scheduler determines the serving relationship between BSs and MSs, and transfers the scheduling decision to each coordinating BS. Based upon the scheduling decision, user data streams shall be delivered by the user data distributor to the corresponding BSs. In Figure 2, di denotes the user data streams that shall be transmitted by BSi.

3) Based on the scheduling decision, BSi estimates the CSI, Hi, between the served MSs and itself. Then the precoding matrix generator in BSi independently derives the precoding matrix Wi to implement e.g. beamforming or MU-MIMO techniques for BSi itself.

As the precoding operation in Co-MIMO is performed individually within each BS, there is no need for excessive CSI exchange among coordinating BSs, which leads to very limited information transfer over the backbone as well as low computational complexity. Although it will result in a little amount of residual ICI within the collaborative region, significant capacity improvement can still be achieved from the system perspective.

Figure 2  Conceptual architecture of Collaborative MIMO

Any single-user MIMO (SU-MIMO) modes (spatial multiplexing, space-time coding, etc.) can be applied to one MS over the equivalent MIMO channel generated by Co-MIMO. It follows that one or multiple data streams can be transmitted from multiple coordinating BSs in Co-MIMO. The coordinating BSs may perform link adaptation, including the desired SU-MIMO mode and modulation and coding, that the MS recommended based on the equivalent MIMO channel.

# Simulation Assumptions and Results

In this section, we use numerical results to demonstrate the advantage of the proposed Co-MIMO technique. In our system level simulation, the collaborative region is set to be three neighboring sectors belonging to different BSs as shown in Figure 3. As the ICI is most severe in the cell edge, we focus on the evaluation of Co-MIMO performance in the cell-edge area. The cell-edge area is determined by the cell-edge length in our simulation and is marked green in Figure 3.

We assume a 19-collaborative-region configuration in the simulation as depicted in Figure 4. The users are randomly dropped with a uniform distribution in the cell-edge area, one user per sector. The system performances with different sizes of cell-edge area are evaluated in the system by setting the cell-edge length to be 150m, 100m, and 50m. In each collaborative region, three coordinating BSs communicate with three MSs located in its cell-edge area using the same time and frequency resource. Through scheduling, every MS in the cell-edge area will be served by two coordinating BSs and every BS will serve two MSs simultaneously. Precoding matrix for the two served MSs is calculated independently within each single sector of a coordinating BS. Every coordinating BS transmits with full power and equal power allocation to the two MSs. A single narrowband subcarrier is assumed in the system without loss of generality.

Traditional SU-MIMO is used as the benchmark for comparison where three MSs in the cell-edge area were served independently by different BSs without any coordination in frequency reuse 1 deployment.

Figure 3  Cell-edge area in a collaborative region          Figure 4  Cell configuration

The parameters used in the system-level simulation are listed in Table 1.

Table 1 Parameters in system-level simulation

| Center frequency | 2.5 GHz | Cell radius | 500 m |
| --- | --- | --- | --- |
| Frequency reuse factor | 1 | Number of drops | 1000 |
| Thermal noise spectral density | -174.3 dBm/Hz | Noise figure | 5 dB |
| Pathloss model | COST-231 | Fast fading model | SCM: Urban micro |
| Wall penetration loss | 10 dB | MS speed | 30 km/h |
| Shadowing correlation | 0.5 (inter site) 1.0 (intra site) | Lognormal shadowing | 8 dB |
| BS Tx power per Tx antenna per subcarrier | 5.3 dBm | MS implantation loss | 5 dB |
| BS Tx antenna | 4 Tx antennas per sector per BS with 4 wavelength spacing | MS Rx antenna | 2 Rx antennas per MS with 1/2 wavelength spacing |
| BS antenna gain | 17 dBi | MS antenna gain | 4.1 dBi |
| BS height | 30 m | MS height | 1.5 m |
| BS antenna pattern | Aθ=−min12θθ3dB2,Am = 70 degree, Am = 20 dB | RF band filter loss | 0.5 dB |

It can be seen from Figure 5, even though there remains certain amount of residual ICI, the interference power from neighboring cells within the collaborative region is considerably reduced. Taken into account the interference from cells outside the collaborative region, the reduction of overall interference power is less significant. This can be further solved by extending the collaborative region to involve more coordinating BSs. The cumulative distribution function (CDF) of cell-edge capacity per sector of traditional SU-MIMO and Co-MIMO are plotted in Figure 6. As shown in Figure 7, the average capacity gain of Co-MIMO over SU-MIMO are 31.7%, 50.2% and 71.9% with respect to cell-edge length of 150m, 100m and 50m. The capacity gain of Co-MIMO over SU-MIMO increase with the shrinking of the cell-edge area. This implies that Co-MIMO is more applicable to cell-edge users. So Co-MIMO is a complement to the traditional SU-MIMO and MU-MIMO in the future cellular system.

Overall interference

Interference from cells within collaborative region

Figure 5  CDF of interference power of the cell-edge users

Figure 6  CDF of cell-edge capacity			Figure 7  Average capacity gain

# Summary

Co-MIMO is a promising MIMO technique that can be applied in the next generation wireless networks to combat ICI. It is effective for the cell-edge users and has the advantage of low backbone requirement and low computational complexity. Hence, Co-MIMO, a compromise between complexity and performance gain, is feasible to evolve from current cellular networks. Based on these considerations, we recommend applying Co-MIMO into IEEE 802.16m systems.