| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Dynamic Interference Coordination |  |
| Date Submitted | 2007-08-29 |  |
| Source(s) | Christian Gerlach Alcatel-Lucent R&I Liu Yang Alcatel Shanghai Bell | Voice: E-mail: christian.gerlach@alcatel-lucent.de liu.yang@alcatel-sbell.com.cn |
| Re: | IEEE 802.16m Requirements - Elaboration of Dynamic Interference Coordination Techniques |  |
| Abstract | The contribution is on Dynamic Interference Coordination in a cellular mobile radio system using orthogonal waveforms (e.g., OFDM) with a frequency reuse of 1. In such a system the performance at the cell edge is limited by inter-cell interference. The goal is to break this bottleneck by distributing restrictions on the transmit power on certain frequencies or frequency blocks (physical resource blocks) in order to reduce the interference on these frequencies in the adjacent cells. Interference Coordination may be based on static frequency patterns. The focus here is to extend these static schemes in order to be dynamically adaptive to changing user traffic situations. |  |
| Purpose | For 802.16m discussion and eventual adoption for standardization of means and methods to support dynamic interference coordination in multi-cell networks |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Dynamic Interference Coordination

Christian Gerlach

Alcatel-Lucent Research and Innovation

Liu Yang

Alcatel Shanghai Bell

# Introduction

In the following of the paper frequency blocks are called “physical resource blocks”.

Interference Coordination works then with power restrictions on frequency blocks or physical resource blocks in downlink or with power preferences on physical resource blocks in uplink in order to shape the power (interference) spectrum in downlink or the interference spectrum in uplink.

The power restriction of a physical resource block in DL leads to an improved SIR as seen in a neighbor cell which leads to an improved cell edge throughput. As can be defined static interference coordination (or re-use schemes) work with static restrictions or preference distribution while dynamic schemes allow dynamic changing restrictions or preferences based on cell loads or geometrically changing user distribution.

The proposal is then on dynamic Interference Coordination to lead to more throughput and improved QoS.

The restrictions and preferences are exploited by the base station schedulers. In the following, first the meaning of resource settings is discussed. Secondly the criterions or framework for the process of inter-cell coordination is discussed. Finally conclusions are drawn after that.

# Inter-cell resource distribution

Resources have to be defined as frequency patterns as seen appropriate for the downlink and uplink directions (e.g. diverse patterns to allow also diverse allocations for VoIP). In case of dynamic restriction or preference setting the number of the frequency pattern (frequency subset number) has to be signalled to a neighbour base station and one scalar power restriction or boost value.

This signalling has to be done in the order of seconds.

The actual inter-cell resource coordination shall take place in the given multi-node architecture.

We propose that a static Interference Coordination setting can define the baseline restriction (or preference) distribution based on a cell planning in the given topology. The dynamic Interference Coordination would then come on top of this predefined restrictions. This way basestation to basestation communication is reduced to the amount necessary for cell load and mobile station concentration balancing.

  - Description of resource setting

The definition of resource setting shall be considered now. In DL the frequencies preferred by a mobile station (MS) are defined by the neighbor cells restricted physical resource blocks. So if there shall be more throughput for certain cell edge MSs, more physical resource blocks of the MS´s neighbor cell are requested to be restricted.

A corresponding dual formulation can be found for the UL.

In UL the preferred frequencies are also defined by the MS´s location. The neighbor cell can accept an interference burden on a frequency subset. If there shall be more throughput in uplink for the MS T1 the neighbor cell of T1 is asked to accept an interference burden on additional PRBs (i.e. a frequency subset). If this request is accepted, the effect as in downlink is that the throughput for T1 can be immediately raised. Further no other cells have to be asked (e.g. to reduce their uplink MS interference on some frequencies) in order to achieve the effect. Additionally if a cell B accepts this interference burden on Fi all other neighbours of cell B can also benefit and load this frequency subset Fi to raise their UL throughput.

  - Decision over resource setting

The task of the dynamic Interference Coordination is to allow cells that are only lightly loaded to give support to highly loaded neighbor cells or neighbor cells with local high concentration of MSs at the cell border.

For example there is a mixture of real-time or guaranteed bit-rate (GBR) traffic and best effort traffic in the cells. So the idea could e.g. be that the best effort traffic can be reduced in the cells neighboring a highly loaded cell in order to help supporting the large guaranteed bit-rate traffic in that cell.

Consequently the strategy of reducing best effort traffic in a cell on its own and to do a muting of frequency subsets does not make sense, since this cell has no idea of its neighbor cells traffic load.

Because the above evaluation must be traffic load based, and MS location based, the coordination must start with a request from the loaded cells.

Thus we have the following mechanism:

Overloaded cells shall send mute requests or requests to accept interference burdens to their neighbor cells.

Obviously overloaded cells need to send only to those neighbor cells to where a high number of served MSs are adjacent to.

The radio resource management is then distributed: The following picture sketches the sending of such requests and the responding with grants:

Figure 1: Transmission of mute requests between neighbour base stations

Regarding DL mute requests to a cell B or UL requests to accept interference we can say that only the cell B scheduler can decide whether to accept these requests or not. This decision will be based on its own traffic load and the usefulness of its own frequencies for his own traffic. So in the requests for frequencies, priorities shall be included. This is also needed since the scheduler or a connected entity has to decide on multiple requests from different cells. Finally after a decision has been made it will be communicated.

The requested base stations should afterwards give grants and inform the requesting base station.

This way the decision process can be carried through.

# System Simulation Results

- Description of Setting

Using the principle described above with local base station-base station request and grant exchange, first system simulations have been carried out. As first shot, simulations have been considered to examine simple negotiation algorithms.

The usual system simulation assumptions have been applied in a multi-cell system simulation using the usual grid of (21) sites. In all normal loaded sectors about 10 MSs are distributed in average. In one overloaded sector n3 a number of 30 MSs are distributed across the border area to one neighbor cell as shown in .

Figure 2: Multi-cell area with overloaded sector and user distribution in black border area

A scheduler has been used that tries to achieve a minimum terminal bitrate for all terminals before distributing the bandwidth more opportunistic. This minimum terminal bitrate value is the same in all cells. This bitrate value has been stepped through to get values of 5%ile and sector throughput.

- Results and Interpretation

Figure 3 shows the result of 5%ile and sector throughput for the overloaded cell.

Figure 3: 5%ile user throughput over sector throughput for overloaded cell n3

The green curve shows the NoIFC simulation, that is without Interference Coordination at all. The red curve shows the static IFC results and the blue curve shows the results of dynamic Interference Coordination.

The operation works as follows. As long as in the overloaded sector the minimum terminal bitrate of 50 or 100 kbit/s can be achieved the sector is not asking for help to neighbor sectors and red and blue curves are identical. When increasing the desired bitrate further without outside help, more than 130 kbit/s is not possible in the static case and the red curve saturates.

In case of dynamic coordination the overloaded sector n3 asks for help from the neighbor cell (where the terminals are adjacent to). It gets granted resources which means that this strongest interfering neighbor puts power restrictions on additional resources which strongly improves the SIR for the MSs in this border region. The result is, as can be seen from the blue curve, that the sector throughput increases by approximately 40% and the 5%ile increases by approximately 23%!

In the next step in the dynamic case the 5%il can be increased even further to 180 kbit/s while the sector throughput decreases again somewhat.

Thus as result it can be seen that the help from neighbor sectors can provide a lot improvement. The graph at the moment is a little rough. The algorithm should be refined to get a somewhat smother curve. It should be implemented that the help should be requested somewhat earlier.

This help from the outside is a measure to increase the fairness. It should be emphasized that the total throughput over the cells is not the focus here. This is shown in Figure 4 where the statistics are done over the other lightly loaded (including helping) cells. Here it is shown that the sector throughput measured over these sectors decrease a little (seen over all cells the change is not right visible).

Figure 4: 5%ile user throughput over sector throughput for all other lightly loaded cells

Most important instead is the achieved increase in 5%ile cell edge bit rate in Figure 3. Translating this into a real-time service one can calculate the number of terminals that can still be served with a guaranteed bit rate (GBR) service of 180 kbit/s in the cells.

As seen from Figure 3 by dynamic Interference Coordination it is now possible to serve in this overloaded cell 30 terminals with 180 kbit/s without a call drop, while without dynamic Interference Coordination approximately only 22 terminals can be served at this bit rate.

This shows that the dynamic Interference Coordination can effectively absorb peaks in cell load and user distribution. It consequently contributes to higher cell edge rates and Quality of Service.

# Conclusion

In this document the principles to be followed in dynamic Interference Coordination have been investigated. Subject of the coordination could be the individual frequency blocks or physical resource blocks which involves much signalling effort. To reduce the effort, frequency subsets are build from physical resource blocks. These are used for the coordination process.

Further it was proposed to build dynamic Interference Coordination on top of static Interference Coordination to limit inter base station communication needs.

The definition of the resource setting and associated usage has been clarified which gives a dual formulation for the meaning of preferences in downlink and in uplink.

Finally the methodology for the decision process was presented to consist of mute requests (or burden requests) and grants in response. It was clarified that only schedulers who know about their (own) cell traffic load are able to decide about requests.

This way a distributed decision process can be implemented. It is proposed that the above framework is considered in the standardization process for dynamic Interference Coordination.

Further system simulations have been carried out for a certain scenario in a first shot with low inter-cell communication. It exemplifies the big potential of dynamic Interference Coordination.