| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | PHY Abstraction for MIMO H-ARQ |  |
| Date Submitted | 2007-09-07 |  |
| Source(s) | Jungwon Lee, Hui-Ling Lou, Adina Matache Marvell Semiconductor, Inc. 5488 Marvell Ln Santa Clara, CA 95054 Dimitris Toumpakaris University of Patras, Rio, Greece 265 00 | E-mail: junglee@marvell.com, hlou@marvell.com, amatache@marvell.com, dtouba@upatras.gr *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m-07/031 “Call for contributions for Draft 802.16m Evaluation Methodology” |  |
| Abstract | This contribution proposes PHY abstraction methodology for MIMO receivers with HARQ. |  |
| Purpose | For discussion and approval of the proposed text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# PHY Abstraction for MIMO H-ARQ

Jungwon Lee, Hui-Ling Lou, Adina Matache, and Dimitris Toumpakaris

Marvell Semiconductor, University of Patras

# Introduction

For MIMO systems with H-ARQ, there are many different methods of combining the received signals. One method is doing bit-level combining, which is a straightforward extension of the SISO H-ARQ case. This bit-level combining is applicable both for Chase Combining (CC) H-ARQ and Incremental Redundancy (IR) H-ARQ. Another method of combining is doing symbol-level combining, which does combining before MIMO equalization or MIMO ML decoding. This symbol-level combining method is known to perform better than the bit-level combining in case of CC H-ARQ.

The current draft of the evaluation methodology document [1] contains the PHY abstraction methodology for H-ARQ, which is applicable to bit-level combining. However, it does not deal with the case of symbol-level combining for MIMO CC H-ARQ. This contribution describes how the PHY abstraction can be done for symbol-level combining for MIMO CC H-ARQ.

# Combining for CC H-ARQ with MIMO Equalizers

For H-ARQ with MIMO equalization, there are three different methods to combine the information from each transmission depending on the location of combining in the receiver. The first method is bit-level combining. The combining for bit-level combining happens at the output of the bit-metric calculator. The second method is symbol-level combining after MIMO equalization. This combining happens at the output of the MIMO equalizer. Finally, the third method is symbol-level combining before MIMO equalization. This symbol-level combining happens before the MIMO equalizer.

In the following section, we describe the symbol-level combining before MIMO equalization.

# Symbol-Level Combining before MIMO Equalization

The received signal model at the i-th H-ARQ transmission for MIMO is

Yi=HiXi+Zi,

where

Yi is the received signal for the i-th transmission

Hi is the MIMO channel for the i-th transmission,

Xi is the transmit signal for the i-th transmission,

Zi is the noise plus interference vector for the i-th transmission.

In case of Chase-Combining, the same signal is repeatedly transmitted, i.e.,

Xi=X.

In this case, the symbol-level combining before MIMO equalization can be done by concatenation approach. The concatenated received signal model after the m-th H-ARQ transmission is

Yc,m=Hc.mX+Zc,m,

where

Yc,m=Y1Y2⋮Ym is the concatenated received signal after the m-th transmission,

Hc,m=H1H2⋮Hm is the concatenated channel matrix after the m-th transmission,
Zc,m=Z1Z2⋮Zm is the noise vector after the m-th transmission.

The equalization can be performed based on this concatenated received signal model. This is what is called as symbol-level combining before equalization. The MIMO equalization is followed by the bit-metric or log-likelihood ratio (LLR) calculation, which is used as an input to a decoder. Thus, the post-processing SINR can be calculated after doing symbol-level combining followed by MIMO equalization.

# Comparison of Various Combining Methods

As was stated earlier, there are three different combining methods for MIMO CC H-ARQ depending on at which stage in the receiver the combining is done: bit-level combining, symbol-level combining after equalization, and symbol-level combining before equalization.

As the combining happens at the later stage, there is greater amount of information loss in general. The information loss at the stage of the bit-metric calculator may be minimal, and there is little performance difference between the bit-level combining and the symbol-level combining after equalization. So, both the bit-level combining and the symbol-level combining performance can be predicted easily by calculating the SINR at the output of MIMO equalizer for each transmission and summing the SINRs over all the past and current transmissions.

However, the information loss at the stage of the MIMO equalizer can be quite significant, and the performance of the symbol-level combining before equalization can be much better than the performance of the symbol-level combining after equalization. For example, consider a MIMO system with two transmit antennas, two receive antennas, and two spatial streams. For the first transmission, let’s assume that the channel matrix happens to be 1111. Because the rank of the channel matrix is only one, two streams are not likely to be passed reliably. So, the receiver asks for retransmission. And for the second transmission, let’s assume that the channel matrix happens to be 1−11−1. Again the rank of the channel matrix is only one, and the MIMO equalizer will not handle this second transmission well for the case of symbol-level combining after equalization. However, for the case of symbol-level combining before equalization, the concatenated channel matrix has a rank of two; two column vectors of the concatenated channel matrix are orthogonal. Thus, the symbol-level combining before equalization can handle the second transmission well.

The above example illustrates the importance of the symbol-level combining before equalization in case of MIMO systems. Roughly speaking, for MIMO systems, decoding error happens for the three reasons:

- Deep fading in channel gain

- Large noise

- Self-interference among multiple streams

The symbol-level combining before equalization handles the self-interference among multiple streams better than the symbol-level combining after equalization.

As the performance of the symbol-level combining after equalization differs from that of the symbol-level combining before equalization, the post-processing SINR for symbol-level combining after equalization is also different from that the post-processing SINR for symbol-level combining before equalization. The post-processing SINR for symbol-level combining before equalization is the post-processing SINR calculated with the concatenated received signal model. Proposed Text Section shows how the post-processing SINR can be computed.

# Conclusion

The symbol-level combining before equalization is one of the promising combining methods for MIMO CC H-ARQ. The PHY abstraction methodology in the current draft of the evaluation methodology document is applicable to the bit-level combining and the symbol-level combining after equalization. However, the PHY abstraction methodology for the symbol-level combining before equalization is not described in the current draft of the evaluation methodology document. So, we propose to include the PHY abstraction methodology for the symbol-level combining before equalization.

# Proposed Text

4.7.2.1 Symbol-Level Combining before MIMO Equalization with H-ARQ

For symbol-level combining before MIMO equalization with H-ARQ, the post-processing SINR can be obtained by computing the post-processing SINR of the concatenated receive signal model.

As in Section 4.5.4, for the i-th transmission, the received signal at the n-th subcarrier is

Yi(0)(n)=Ptx(0)Ploss(0)Hi(0)(n)Xi(0)(n)+j=1NIPtx(0)Ploss(0)Hi(j)(n)Xi(j)(n)+Ui(0)(n),

where the subscript i represent the transmission index. For every transmission, the same signal is transmitted for the intended user, i.e., Xi(0)(n)=X(0)(n) for every i.

The concatenated received signal model after the m-th transmission is as follows:

Yc,m(0)(n)=Ptx(0)Ploss(0)Hc,m(0)(n)X(0)(n)+j=1NIPtx(0)Ploss(0)Hc,m(j)(n)Xc,m(j)(n)+Uc,m(0)(n),

where

Yc,m(0)(n)=Yc,1(0)(n)Yc,2(0)(n)⋮Yc,m(0)(n) is a concatenated received signal vector,

Hc,m(0)(n)=Hc,1(0)(n)Hc,2(0)(n)⋮Hc,m(0)(n) is a concatenated direct channel matrix,

Hc,m(j)(n)=Hc,1(j)(n)0⋯00Hc,2(j)(n)⋯0⋮⋮⋱⋮00⋯Hc,m(j)(n) is a block diagonal interfering channel matrix for the j-th interfering signal,

Xc,m(j)(n)=Xc,1(j)(n)Xc,2(j)(n)⋮Xc,m(j)(n) is a concatenated transmit signal vector of the j-th interfering signal,

Uc,m(0)(n)=Uc,1(0)(n)Uc,2(0)(n)⋮Uc,m(0)(n) is a concatenated noise vector,

The post-processing SINR can be calculated as in Section 4.5.4 with the concatenated receive signal model.

# References

[1] IEEE C802.16m-07/080r3, “Draft IEEE 802.16m Evaluation Methodology Document”, Aug. 29, 2007.