| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Modification for Draft 802.16m Evaluation Methodology Document: 10.1 Web Browsing (HTTP) Traffic Model |  |
| Date Submitted | 2007-9-13 |  |
| Source(s) | Jeongeun Julie Lee, Belal Ham, Intel Bong Ho Kim, Posdata John Harris, Hua Xu, Motorola Jeongho Park, Samsung | jeong.julie.lee@intel.com bhkim@posdata-usa.com hua.xu@motorola.com jeongho.jh.park@samsung.com |
| Re: | IEEE 802.16m-07/080r3, “Call for Comments on Draft 802.16m Evaluation Methodology Document” |  |
| Abstract | This contribution proposes a text modification on the “Web Browsing (HTTP) Traffic Model” section (section 10.1) of the Draft 802.16m Evaluation Methodology Document |  |
| Purpose | Propose the text and table changes to provide a recent web browsing traffic model. |  |
| Notice | This document has been prepared to assist IEEE 802.16. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy and Procedures | The contributor is familiar with the IEEE 802.16 Patent Policy and Procedures <http://ieee802.org/16/ipr/patents/policy.html>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard." Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication. Please notify the Chair <mailto:chair@wirelessman.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.16 Working Group. The Chair will disclose this notification via the IEEE 802.16 web site <http://ieee802.org/16/ipr/patents/notices>. |  |

Text Modification for Draft 802.16m Evaluation Methodology:

10.1 Web Browsing (HTTP) Traffic Model

Julie Lee, Belal Hamzeh

Intel

Bong Ho Kim

Posdata

John Harris, Hua Xu

Motorola

Jeongho Park

Samsung

# I. Introduction

1. Even if the model in the table 27 is captured recently with tremendous number of measurements, it is still limited to the 50 selected web sites. The model introduced in this contribution is based on a new HTTP model that represents the traffic actually seen by a user platform and parameter values were determined with three days of the recent actual user traces collected at 10 world wide proxy servers. Thus, it should be more realistic than the current artificial measurement since the model has been derived from a live web traffic trace.

2. The mixture of different version of HTTP such as HTTP 1.0 and HTTP 1.1 can be properly modeled with additional information such as the number of persistent connections and the number of pipelining. This information, however, is not presented in IEEE 802.16m-07/080r3.

# II. Text Proposal

-------------------------------------------------------Start of the text------------------------------------------------------------

[Add the following references after the line#8 of the page#15 in C802.16m-07/080r3]

- RFC 2616, Hypertext Transfer Protocol -- HTTP/1.1

- J. J. Lee, M. G “A New Traffic Model for Current User Web Browsing Behavior”, http://blogs.intel.com/research/2007/09/a_new_traffic_model_for_curren.html, 2007

-------------------------------------------------------End of the text------------------------------------------------------------

-------------------------------------------------------Start of the text------------------------------------------------------------

[change text, Figure17 and Table 27:  line#11 of the page#93 in C802.16m-07/080r3]

HTTP traffic characteristics are governed by the structure of the web pages on the World Wide Web (WWW), and the nature of human interaction. The nature of human interaction with the WWW causes the HTTP traffic to have a bursty profile, where the HTTP traffic is characterized by the ON/OFF periods as shown in Figure 17.

First packet of session

Reading Time

Reading Time

Last packet of session

Web

Request

HTML Object

Embedded Object 1

Embedded Object n

Time

Parsing time

Reading Time

HTML Inter-Arrival-Time

…

Figure 17 1: HTTP Traffic Model and Traffic Pattern

The ON periods represent the sequence of packets in which the web page Web Request is being transferred from source to destination; while the OFF periods represent the time the user spends reading the webpage before transitioning to another page a new Web Request starts. This time is also called as Reading Time. [43][44].

The amount of information passed from the source to destination during the ON period is governed by the web page structure. In this model, a web page may consist of multiple HTML objects and other embedded objects. A webpage Web Request is usually defined to be composed of a main an HTML object and several embedded objects as shown in Figure 1. The size of the main HTML object, in addition to the number and size of the embedded objects define the amount of traffic passed from source to destination.

In summary, the HTTP traffic model is defined by the following parameters:

SM:  Size of main HTML object in page

Nd:  Number of embedded objects in a page between two consecutive HTML objects

SE:  Size of an embedded object in page

Dpc: Reading time, time between the last embedded object end time and the next HTML object start time

Tp:  Parsing time for the HTML object, time between the HTML object start time and the first embedded object start time

In addition to the model parameters, HTTP traffic behavior is also dependent on the HTTP version used. Currently HTTP 1.0 and HTTP 1.1 are widely used by servers and browsers [45]-[48]. In HTTP 1.0, also known as burst mode transfer, a distinct TCP connection is used for each object in the page, thereby facilitating simultaneous transfer of objects. The maximum number of simultaneous TCP connections is configurable, with most browsers using a maximum of  2 to 6 simultaneous TCP connections, andRFC2616 specifies that only a maximum of 2 persistent connections should be established between the browser and server. In HTTP/1.1, also known as persistent mode transfer, all objects are transferred serially over a single persistent TCP connection. This often reduces the page loading times and especially reduces the numbers of packets needed for a page (re)load. In many cases, pipelining is OFF by default.

Table 27 01.1-1 provides the model parameters for HTTP traffic for downlink and uplink connections based on the model and measurements in [2]. Error: Reference source not found and the model in Error: Reference source not found.

|  |  | σ=1.37,μ=8.35 σ=0.8,μ=10.55 | σ=1.37,μ=8.35 | fx=12πσxexp−lnx−μ22σ2,x≥0 |
| --- | --- | --- | --- | --- |
|  |  | σ=2.36,μ=6.17 σ=1.97,μ=7.1 | σ=1.69,μ=7.53 | fx=12πσxexp−lnx−μ22σ2,x≥0 |
|  |  |  | α=1.1,k=2,m=55 | fx=αkαxα+1,k≤x<mfx=kmα,x=m |
|  |  |  | λ=0.033a=0 b=10 | DL: fx=λe−λx,x≥0 UL:fx=1b−a,a≤x≤b |
|  |  |  | λ=7.69 | fx=λe−λx,x≥0 |

| Component | Distribution | Parameters |  | PDF |
| --- | --- | --- | --- | --- |
|  |  | Downlink | Uplink |  |
| Main HTML object size (SM) | Truncated Lognormal | Mean = 11872 bytes SD= 38306bytes Min = 196bytes Max = 2Mbytes σ=1.76,μ=7.9 | Mean = 9055 bytes SD = 13265 bytes Min = 100 bytes Max = 100 Kbytes | if x>max or x<min, discard and generate a new value for x |
| Embedded object size (SE) | Truncated Lognormal | Mean = 12460bytes SD = 116050bytes Min = 196bytes Max = 6Mbytes σ=2.17,μ=7.51 | Mean = 5958 bytes SD = 11376 bytes Min = 50 bytes Max = 100 Kbytes | if x>max or x<min, discard and generate a new value for x |
| Number of embedded objects (Nd) | Gamma | Mean = 5.07 Max. = 300 a=0.11,b=33.16 | Mean = 4.229 Max. = 53 a=0.11,b=33.16 | fx=1baΓ(a)xa−1expxb,x>0 if x>max, discard and regenerate a new value for x |
| Reading time (Dpc) | Truncated Lognormal | Mean = 39.7 sec Max. = 10000 sec σ=2.77,μ=−0.5 | Mean = 30 sec σ=2.77,μ=−0.5 | if x>max, discard and regenerate a new value for x |
| Parsing time (Tp) | Truncated Lognormal | Mean = 3.12 sec Media = 0.30 sec Max. = 300 sec σ=2.08,μ=−1.25 | Mean = 0.13 sec σ=2.08,μ=−1.25 | if x>max, discard and regenerate a new value for x |
| Number of Persistent Connection | 2 |  |  |  |
| Pipelining | Off |  |  |  |

Table 01.1-1: HTTP Traffic Model

-------------------------------------------------------End of the text------------------------------------------------------------