| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Location Estimation and Tracking System for Mobile Devices in Project 802.16m SDD |  |
| Date Submitted | 2007-11-07 |  |
| Source(s) | Pei-Kai Liao, I-Kang Fu, Paul Cheng MediaTek Inc. No.1, Dusing Road 1, Science-Based Industrial Park, Hsinchu, Taiwan 300, R.O.C. Chung-Hsien Hsu, Yuan Pin Lin, Chun Fang Lee, Fan-Shuo Tseng, Chieh Yuan Ho, Kai-Ten Feng, Ta-Sung Lee, Yu-Ted Su, Wen-Rong Wu, Ching Yao Huang, NCTU/MediaTek Inc. 1001 Ta Hsueh Road, Hsinchu, Taiwan 300, R.O.C. | pk.liao@mediatek.com paul.cheng@mediatek.com stanley.hsu@mediatek.com |
| Re: | IEEE 802.16m-07/040, “Call for Contributions on Project 802.16m System Description Document (SDD)” |  |
| Abstract | This contribution proposes a section/sub-section to describe hybrid location estimation and tracking system in the Table of Contents (ToC) of IEEE 802.16m SDD to provide better accuracy of location estimation for mobile devices than current IEEE 802.16 system. |  |
| Purpose | Propose to have a section/subsection “Mobile Station Location Estimation and Tracking” in TGm SDD ToC. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Location Estimation and Tracking System for Mobile Devices in

IEEE 802.16m

Pei-Kai Liao, I-Kang Fu, Paul Cheng

MediaTek Inc.

Chung-Hsien Hsu, Yuan Pin Lin, Chun Fang Lee, Fan-Shuo Tseng, Chieh Yuan Ho, Kai-Ten Feng, Ta-Sung Lee, Yu-Ted Su, Wen-Rong Wu, Ching Yao Huang

NCTU/MediaTek Inc.

I. INTROUCTION

Wireless location technologies have drawn a significant amount of attention over the past few decades. Different types of location-based services (LBSs) have been proposed and studied, including the emergency 911 (E-911) subscriber safety services [1], the location-based billing, the navigation system, and applications for the intelligent transportation system. Due to the emergent interests in LBSs, it is required to provide enhanced precision in the location estimation of mobile device under different environments. A variety of wireless location techniques have been studied and investigated [2] [3] [4]. These techniques can be classified into the network-based and the satellite-based location schemes. The representative algorithms for the network-based location techniques are the time difference-of-arrival (TDOA) and the angel-of-arrival (AOA). The TDOA scheme measures the time difference of signals coming from different wireless base stations; while the AOA technique observes the arriving angle of signal from the mobile device. The well-adapted technology for the satellite-based location method is the global positioning system (GPS). It measures the time-of-arrival (TOA) of the signals coming from different satellites. It has been studied in several researches [5] [6] that the performance of the location techniques listed above varies under different environments. Due to weak incoming signals or shortage of signal sources (e.g. at rural area), the network-based (i.e. TDOA, AOA) methods result in degraded performance for the location determination of mobile devices [7] [8]. On the other hand, the major problem for the satellite-based (i.e. GPS) method is that the performance considerably degrades while the satellite signals are severely blocked (e.g. at urban valley area).

In order to achieve better accuracy for location estimation, a hybrid approach should be considered to satisfy the requirements under different environments (i.e. urban, suburban, and rural areas). In this contribution, a hybrid location estimation and tracking system is proposed for the mobile stations (MSs). The proposed location scheme determines the MS’s location by combining the outcomes from both the network-based and the satellite-based techniques. In addition to the longitude (x) and latitude (y) of the MS can be estimated, the altitude (z) information of the MS is also obtainable from the proposed system. The performance of the proposed hybrid location scheme is evaluated via simulations under different environments.

The rest of this contribution is organized as follows. The ensuing section describes the system architecture. In Section III, we provide some numerical simulation results for comparison. Then, some concluding remarks are given in Section IV. Finally, proposed sections/subsections in the table of content (ToC) for IEEE 802.16m SDD are described in the last section.

II. System Architecture

Fig. 1 shows the schematic diagram of the system architecture for the proposed hybrid location estimation. The hybrid system combines the signals coming from both the satellites and the cellular networks. In order to obtain the TOA measurements from the satellites, it is assumed that each MS is equipped with a GPS receiver. As shown in Fig. 1, the TDOA measurements are conducted at the MS by obtaining the signals via the downlink from the BSs. The AOA signals are transmitted from the MS to the BSs using the uplink. The AOA measurements are performed at the BS using its antenna arrays for two-dimensional adaptive beam steering. In order to avoid signal degradation due to the near-far effect, it is assumed in the proposed system that only the home BS provides the capability of the AOA measurements.

Figure 1. The schematic diagram of the hybrid mobile location scheme

The hybrid location estimation scheme proposed in this contribution can be applied to either MS-based or MS-assisted system. The choice between these two types of systems depends on the requirement of the communication bandwidth and the computation power of the MS.

III. Simulation Results and Comparison

Simulations are performed to show the effectiveness of the proposed hybrid location scheme. The MS is assumed to travel at a constant speeds of (3, 4, 0) m/s along the x and y directions, i.e. (x, y, z) = (27 + 3t, 36 + 4t, 300) in meters. Fig. 2 shows the performance of the proposed hybrid location system (measured in RMS error) comparing with the GPS and the cellular system under urban, suburban, and rural environments. It can be seen that the GPS system has the worst performance comparing with the other two systems in the urban area; while the cellular system causes degraded results in the rural area. The hybrid system is capable of adjust itself to accommodate different situations, which provides consistent performance comparing with the GPS and the cellular systems.

Figure 2. Performance comparisons between the GPS, the cellular, and the proposed hybrid system in urban (top), suburban (middle), and rural (bottom) areas

IV. Conclusion

A hybrid location estimation and tracking system is introduced in this contribution. The system is capable of estimating the three dimensional position and velocity of the mobile devices. It is shown in the simulation results that the proposed scheme provides consistent location estimation accuracy under different environments. However, the consistent location estimation accuracy will only be obtained in the condition that the both systems, the satellite-based systems and network-based systems, have sufficient signal sources simultaneously.

V. Proposed Sections/Subsections in the Table of Content (ToC)

This contribution is to present a hybrid method of location estimation and tracking for mobile devices. According to IEEE 802.16 standard documents, current system does not support the function of location estimation and tracking for mobile devices. Therefore, it is suggested to include this functionality in IEEE 802.16m system to enhance the accuracy of location estimation and tracking for mobile devices. Required modifications to current system and proposed sections/subsections in ToC are shown as follows.

Proposed sections/subsections:

---------------------------------------------------------Start of the Text---------------------------------------------------------

[Adopt the following text in the ToC of P802.16m System Description Document (SDD)]

x.y Mobile Station Location Estimation and Tracking

[Although the GPS technology is widely used in position, not all of the MSs will be equipped with it. In order to support this functionality, therefore, three location estimation and tracking schemes are proposed to provide location-based service.]

x.y.1 GPS based MS Location Estimation and Tracking

x.y.2 Network-assisted MS Location Estimation and Tracking

x.y.3 Hybrid MS Location Estimation and Tracking

[It has been known that the performance of the GPS based and network-assisted MS location systems vary under different environments. As a result, a hybrid location scheme, which combining the outcomes from both the network-based and the satellite-based techniques, is proposed to adaptation to various scenarios for location estimation. It adopts the existing infrastructure and can provide consistent location estimation accuracy under different environments.]

-----------------------------------------------------------End of the Text--------------------------------------------------------

REFERENCEs

- Federal Communications Commission, “Revision of the Commissions Rules to Insure Compatibility with Enhanced 911 Emergency Calling Systems,” 1996.

- H. Koshima, and J. Hoshen, “Personal Locator Services Emerge,” IEEE Spectrum, vol. 37, pp. 41-48, Feb. 2000.

- J. H. Reed, K. J. Krizman, B. D. Woerner, and T. S. Rappaport, “An Overview of the Challenges and Progress in Meeting the E-911 Requirement for Location Service,” IEEE Commun. Mag., vol. 36, pp. 30-37, Apr. 1998.

- Y. Zhao, “Standardization of Mobile Phone Positioning for 3G Systems,” IEEE Communications Magazine, vol. 40, pp. 108-116, July 2002.

- Y. T. Chan, C. H. Yau, and P. C. Yau, “Linear and Approximate Maximum Likelihood Localization from TOA Measurements,” IEEE Signal Processing and Its Applications, vol. 2, pp. 295-298, July 2003.

- X. Wang, Z. Wang, and B. O’Dea, “A TOA-Based Location Algorithm Reducing the Errors due to Non-Line-of-Sight (NLOS) Propagation,” IEEE Trans, vol. 52, Jan. 2003.

- D. Porcino, “Performance of a OTDOA-IPDL Positioning Receiver for 3GPP-FDD Mode,” IEE 3G Mobile Communication Technologies, pp. 221-225. Mar. 2001.

- SnapTrack, “Location Technologies for GSM, GPRS, and UMTS Network,” a QUALCOMM Company white paper, Jan. 2003.