| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | A New Approach for Inter-cell Interference Randomization based on Cell/Sector-specific Interleaver Pattern |  |
| Date Submitted | 2008-01-17 |  |
| Source(s) | Yunzhou Li, Tsinghua Univ. Shidong Zhou, Tsinghua Univ. Xibin Xu, Tsinghua Univ. Xin Su, Tsinghua Univ. Xiaofeng Zhong, Tsinghua Univ. Jing Wang, Tsinghua Univ. Xiaolu Dong, CATR Ying Du, CATR Shanpeng Xiao, CMCC Wenqi Liao, CMCC Lian Yang, Huawei Technologies Shiqiang Suo, Datang Mobile Zhongkai Wang, Tianjin Hi-Tech Industrial Park, Tianjin, China Weijun Xu, Tianjin Municipal Government, Tianjin, China Thomas Li, Legend Silicon Corp., Fremont, CA, USA | liyunzhou@tsinghua.edu.cn, zhousd@tsinghua.edu.cn xuxb@tsinghua.edu.cn suxin@tsinghua.edu.cn zhongxf@tsinghua.edu.cn wangj@tsinghua.edu.cn dongxiaolu@mail.ritt.com.cn duying@mail.ritt.com.cn xiaoshanpeng@chinamobile.com liaowenqi@chinamobile.com yang.lian@huawei.com suoshiqiang@datangmobile.cn wangzk@thip.gov.cn xwj@tipp.gov.cn thomasli@legendsilicon.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Response to IEEE 802.16m-07/040 “Call for Contributions on Project 802.16m System Description Document (SDD)” |  |
| Abstract | This document proposes a new approach to randomize inter-cell interference based on cell/sector-specific interleaver pattern. |  |
| Purpose | For discussion and approval by TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

A New Approach for Inter-cell Interference Randomization based on Cell/Sector-specific Interleaver Pattern

Yunzhou Li, Shidong Zhou, Xibin, Xu, Xin Su, Xiaofeng Zhong, Jing Wang

Tsinghua Univeristy

Dong Xiaolu, Du Ying

CATR

Xiao Shanpeng, Liao Wenqi

China Mobile

Lian Yang

Huawei Technologies

Shiqiang Suo

Datang Mobile

Zhongkai Wang

Tianjin Hi-Tech Industrial Park, Tianjin, China

Weijun Xu,

Tianjin Municipal Government, Tianjin, China

Thomas Li,

Legend Silicon Corp., Fremont, CA, USA

# 1.0 Purpose

This contribution proposes a new approach to randomize inter-cell interference based on cell/sector-specific interleaver pattern. Meanwhile, some simulation results to verify the advantages of the new approach are presented in this contribution, too.

# 2.0 Introduction

Inter-cell-interference randomization is a necessary component to make sure the cellular system with high efficiency of frequency reuse work smoothly. In the traditional system, inter-cell-interference randomization can be done by scrambling, applying (pseudo) random scrambling after channel coding/interleaving or by frequency

hopping. Sometimes a spreading is also included. The cell-specific randomization in general help the subscribe stations to correctly detect and receive the information from the desired cell.

However, in this contribution we recommend a new randomization approach based cell-specific interleaving, also known as Interleaved Division Multiple Access (IDMA), to be used as inter-cell multiple access and instead of cell-specific scrambling.

# 3.0 Why and how to use cell-specific interleaving

## 3.1 IDMA

Interleave-Division Multiple-Access (IDMA) is a recently proposed multi-access scheme, in which users are distinguished by different interleaving patterns. When IDMA is used as inter-cell multiple access, it can play a role just as CDMA in whitening CCI. Moreover, if the subscriber station has the capability of iterative multi-user detection, IDMA can bring 1~2 times capacity gain over CDMA due to its high coding and diversity gain.

As showed in Fig. 1, the kernel feature of IDMA system is that users employ user-specific interleaver patterns, which would be used to extract multiple users’ signals from received signal. All redundancy in whole IDMA system can be utilized in encoder, so the more coding gain can be achieved.

Figure 1. Illustration of transmitter of IDMA-OFDM system

The two types of IDMA-OFDM receivers are showed in Fig. 2. Single user detection has a low complexity with the performance similar to a CDMA system with single-user match filter. Multiuser detection (MUD) would achieve considerable performance gain at the cost of complexity. In general, the performance of IDMA with MUD substantially outperforms CDMA.

Figure 2. Illustration of two kind of detection mode in IDMA-OFDM receiver.

## 3.2 IDMA vs. DS-CDMA

The advantages of IDMA over DS-CDMA at least focus on two aspects:

Firstly, DS-CDMA system can be regarded as a system with a channel coding concatenated with a repeated coding (repeated coding does not have any coding gain). Both the FEC and repeated coding will lead to spectrum spreading. While for an IDMA system, all spread spectrum can be used to FEC coding for achieving larger coding gain.

On the other hand, IDMA system can achieve single-user performance with iterative multi-user decoding. Take a two-user system for instance. After deinterleaving, the interference to N adjacent symbols of the desired data sequence come from N random-distributed symbols of the interfering sequence. Therefore the decodings of the two users are fully uncorrelated, thus the iterative reception can achieve a significant gain. However, in a system using scrambling codes to distinguish users, a desired data segments (consists of N adjacent symbols) coincides with the interfering segment. Thus a severe positive feedback will take place in the iterative decoding. This effect can be easily illustrated with generalized Tanner graphs analysis [2].

Unlike that in uplink, the most severe CCI in downlink appears at the cell edge, where an SS may receive strong interference from 1 or 2 adjacent base stations (SIR up to 0dB). In this case, we should evaluate the performance of multiple access with a small number of users, but with much higher coding rate (compared with uplink).

## 3.3 How to use IDMA in IMT-A

As discussed above, cell-specific interleaving brings more robust performance than cell-specific scrambling. The advantage of interleaving over scrambling seems very important for cell-edge subscriber stations to receive broadcast services, such as common signaling broadcasting, because some advanced transmitting techniques (ex. Beamforming) for unicasting can’t be used for broadcasting.

Figure 3. An example of scenario using IDMA in downlink.

# 4.0 Generation and allocation of cell-specific interleaver patterns

Usually, an interleaver is used as a component of a channel encoder to enhance the coding gain, or as a channel interleaver to combat the time/frequency coherent fading by scrambling burst errors into random errors. However, cell-specific interleaving can also be used to randomize the inter-cell interference. Simple random interleaving is suitable for the cell-specific interleaving, which can provide sufficient number of interleaver patterns with satisfactory anti-fading performance and correlation property. A method to generate randomized interleaver patterns is introduced below:

Considering an interleaver with length L, and m is the minimum k which satisfies 2k>L, we will introduce an positive integer parameter , which is less than 2m. The input sequence is {xi}, each element can be a coded bit.

Let Ij denote the original index of the input sequence for the jth output element, i.e., the output sequence {yj} is y0y1y2⋯yL−1=xI0xI1xI2⋯xIL−1.

Let the m sequence generator with order m be gm(x), and can be written in binary form gm, e.g., g3(x)=x3+x2+1 can be written as gm=1101(B).

The Ij, j=0~L-1 can be generated by the following steps, the operator  means bit-by-bit module 2 sum. a<<b means the left shifting a by b bits. A>>b means the right shifting a by b bits.

s=;

j=0;

for k=0: 2m-1

t=s-1;

if t<L

Ij=t;

j=j+1;

end if;

if s>>(m-1)==1

s=(s<<1)gm;

else

s=s<<1;

end if;

end;

Different generating seeds  will lead to different interleaving patterns for different cells. The number of the different interleaver patterns generated with the above method is up to the length of interleaver.

Regarding the allocation of cell-specific interleavers, different interleaver patterns can be allocated to different cells, by generating the interleavers with the corresponding seeds. A base station or subscriber station can identify the interleaver pattern (seed) of each cell by checking its interleaver pattern ID.

In principle, hundreds of distinct seeds can be provided. However, using a distinct pattern for each cell in the system is unnecessary. Hence, it is recommend to reuse a relatively small number of interleaver patterns (seeds) generation seed. The reuse of the seeds can be realized in a manner similar to that of frequency reuse in a cellular system.

# 5.0 Simulation results

Here we present some simulation results to illustrate the performance of multi-user IDMA-OFDM system. In the following, we first give a performance comparison between IDMA-OFDM and OFDM with scrambling under AWGN channel, and then present the performance of IDMA-OFDMA system under multi-path channel.

As described above, whitening techniques could be employed in downlink for co-channel reuse between adjacent cells, including IDMA and random scrambling (DS-CDMA). Actually, it is easy to find that the two approaches provide the same performance with single-user detection/decoding. However, their performances with iterate multiuser decoding are substantially different, as shown in the following simulation.

In our simulation, the following parameters are assumed:

Modulation: QPSK

FEC: rate 1/2 or 1/4 convolutional code

Channel: AWGN

User number: 2 or 3

Iteration times: 3

Definition of SNR: received signal power of one user to the noise power

MA methods comparison: IDMA, CDMA without repeating (Random scrambling)

Fig. 4 suggests that, if using scrambling for co-channel reuse between two cells, the coding rate should be reduce to 1/4 in order to achieve reasonable BER performance. The total spectral efficiency is: 2cells*(2*1/4)bit/s/Hz/cell=1bit/s/Hz. On the contrary, if IDMA is used in the same case, rate 1/2 coding can be used for both the cells. The total spectral efficiency is 2cells*(2*1/2)bit/s/H/cell=2bit/s/Hz, which is twice of that of DS-CDMA. HH

Fig. 5 shows the co-channel reuse capability of a three-cell system. DS-CDMA system with rate 1/4 code cannot even provide satisfactory performance whereas IDMA can still work with rate 1/2 code, resulting a total spectral efficiency of 3bit/s/Hz.

Fig. 6 demonstrates the performance of multiuser system based on IDMA will be converge to that of single user in high SNR region, which implies and encourages us to enhance the throughput of users around by using IDMA at the cost of affordable complexity.

Fig. 4 BER performance Comparison between Scrambling and Interleaving, under the conditions of AWGN channel, Two Users, QPSK, convolutional code with constraint length 5

Fig. 5 BER performance Comparison between Scrambling and Interleaving, under the conditions of AWGN channel, Three Users, QPSK, convolutional code with constraint length 5

Fig. 6 BER performance Comparison between single user and multi-user (K=3), under the conditions of Rayleigh channel, Three Users, QPSK, convolutional code with constraint length 5

# 6.0 Conclusion

Fundamentally, inter-cell-interference randomization aims at randomizing the interfering signal(s) and thus to allow for interference suppression at the receiver in line with the processing gain.

Methods considered for inter-cell-interference randomization includes:

- Cell-specific scrambling, applying (pseudo) random scrambling after channel coding/interleaving

- Cell-specific interleaving, also known as Interleaved Division Multiple Access (IDMA)

A third means for randomization is to apply different kinds of frequency hopping.

With regards to inter-cell-interference randomization, cell-specific scrambling and cell-specific interleaving (IDMA) basically have the same performance (regarding IDMA for inter-cell interference cancellation, see below).

A pseudo-random method can be used to generate the cell-specific interleaver patterns for IDMA. The number of the available patterns (seeds) is determined by the length of interleaver. A SS can identify the interleaver pattern of the cell by checking its interleaver pattern ID. The seeds can be reused between “far-spaced” cells in a manner similar to that of frequency reuse in a cellular system.

# 7.0 Reference

[]