| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Feedback-aided MU-MIMO Scheduling Technique |  |
| Date Submitted | 2007-11-07 |  |
| Source(s) | Keying Wu, Lei Wang, Hongwei Yang, Liyu Cai Alcatel-Lucent | {Keying.Wu, Lei.p.Wang, Hongwei.Yang, Liyu.Cai}@alcatel-sbell.com.cn *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | [Cite the specific document number of the appropriate Call for Contributions, the ballot number, etc. Contributions that are not responsive to this section of the template, may be refused or assigned a low priority for review.] |  |
| Abstract | The contribution presents a user feedback-aided scheduling technique to improve the performance of MU-MIMO in the case of imperfect channel state information at the transmitter (CSIT). The rational comes from the observation that the inter-user co-channel interference (CCI) due to imperfect CSIT increases with the number of users simultaneously supported in MU-MIMO transmission, so that decreasing the user number can reduce the CCI level and sometimes even increase the overall throughput. The optimal user number depends on the level of CSIT error. Motivated by this observation, we design a feedback-aided scheduling technique, which adjusts the number of simultaneous users based on the user feedback of CSIT error covariance or CCI measurement. Considerable performance improvement has been shown for this technique using numerical results. |  |
| Purpose | To standardize means and methods to support user feedback of CSIT error information or CCI measurement to aid the successful application of MU-MIMO. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Feedback-aided MU-MIMO Scheduling Technique

Keying Wu, Lei Wang, Hongwei Yang, Liyu Cai

Alcatel-Lucent

# Introduction

The downlink multiuser multiple-input-multiple-output (MU-MIMO) technology aims at increasing the average system throughput by supporting multiple users over the same time-frequency resource and using multiuser precoding to minimize the inter-user co-channel interference (CCI). It has the potential to significantly improve the system throughput, and has been defined as a MIMO option in the System Requirement Document of 802.16m. MU-MIMO requires channel state information (CSI) of all users at the transmitter. The accuracy of CSI at the transmitter (CSIT) has a great impact on the performance of MU-MIMO. Imperfect CSIT will increase CCI among users and deteriorate the capacity of MU-MIMO seriously, especially at high signal-to-noise-ratios (SNRs). This problem has become one of the major challenges that MU-MIMO has to face in real systems.

In this proposal, we present an advanced MU-MIMO scheduling technique to improve the capacity of downlink MU-MIMO with imperfect CSIT. The rational behind the proposed scheme is the observation that the level of CCI among users increases with the number of users simultaneously supported in the MU-MIMO transmission. Thus, decreasing the number of simultaneous users can reduce the CCI and sometimes lead to better sum capacity. The optimal user number depends on the level of the CSIT error. Motivated by this observation, we design a feedback-aided scheduling technique, which adjusts the number of simultaneous users based on the CSIT error covariance estimation or CCI measurement at the user side. When users are capable of estimating the covariance of the CSIT error, they can feedback the covariance so that the transmitter can adjust the scheduling results based on such information. When users have difficulty obtaining the knowledge of CSIT error, they can estimate the CCI power and check if it is too high. They then feed back their decisions to the transmitter, which collects feedbacks from all users and adjusts the user number accordingly. Simulation results have shown that considerable capacity improvement can be achieved by the proposed scheduling technique. And such performance gain is obtained at the cost of only one or a few bits of additional feedback per user and marginal complexity increase at both the transmitter and user side.

This technique is suggested as an enhanced MU-MIMO scheduling solution for IEEE 802.16m systems.

# Feedback-aided MU-MIMO Scheduling

Consider the downlink of a MU-MIMO system with NT transmit antennas at the base station (BS) and K users receiving services from the same BS over the same time-frequency resource, each equipped with NR receive antennas. Denote the channel matrix for user-k byHk=[hk,n(m)], wherehk,n(m)is the fading coefficient between the nth transmit antenna and the mth receive antenna of user-k. Assume that the BS only has imperfect CSIT Hk. The CSIT errorHk, whereHk=Hk−Hk, can be caused by various reasons such as channel estimation (CE) error, feedback delay, and feedback quantization, etc. Denote by sk the number of data streams intended to user-k. It is always assumed that skNR andsk≤NT. At the BS, a length-sk data vector xk for user-k is first transformed to a length-NT symbol vector by multiplying a NTsk precoding matrix Tk. In this proposal, we will always assume that {Tk} are generated using the zero-forcing (ZF) type of multiuser precoding algorithms, such as channel inverse (CI) and block diagonalization (BD). The symbol vectors of K users are then linearly superimposed and lunched into the channel from the antenna array simultaneously. We always assume that the elements of xk are independent and identically distributed (i.i.d.) with zero mean and unit variance. The signal vector received at user-k is

yk=(Hk+Hk)k'=1KTk'xk'+nk=(a)(Hk+Hk)Tkxk+Hkk'≠kTk'xk'+nk			(1)

where nk is a vector of samples of an additive white Gaussian noise (AWGN) process with zero mean and variance 2. In (1), equation (a) follows from the fact that HkTk'=0 for k’k when the ZF type of precoding algorithm is used.

The second term in the last formula of (1) denotes the residual inter-user CCI due to CSIT error, whose average power, denoted by PI, can be calculated as

PI=tracek'≠kHkTk'Tk'HHkH							(2)

As we can see from (2), the average power of the residual inter-user CCI increases with the CSIT error, the total transmission powerPT≡trace(TkHTk), and the interfering user number. This means that decreasing the simultaneous user number can lead to reduced CCI, and sometimes even increased sum capacity, with imperfect CSIT.

Fig. 1 shows the relationship between the sum capacity and the number of simultaneous users with both perfect and imperfect CSIT. For imperfect CSIT, we consider the case of slight CSIT error and severe CSIT error. Here we employ NT=4 and NR=2. The SNR is set to 20dB in this simulation. We fix the simultaneous user number K at 1, 2, 3 and 4, with one data stream assigned to each user. From Fig. 1, we can see that with different accuracy of CSIT, the optimal sum capacity is achieved with different simultaneous user numbers: The optimal simultaneous user number is 3 with perfect CSIT, and drops to 2 with severe CSIT error. With slight CSIT error, the user numbers of 2 and 3 can achieve very similar performance.

Figure 1. The relationship between sum capacity and the number of simultaneous users with perfect and imperfect CSIT.

According to the above discussion, the optimal user number depends on the level of CSIT error, so the accuracy of CSIT should be taken into account in the MU-MIMO scheduler for user selection. However, it is not always possible for the BS to acquire the knowledge of CSIT error. Sometimes, partial information of the CSIT error, such as mean or covariance, is available at the BS through feedback, and sometimes there may be no knowledge at all for the CSIT error at the BS. In the following, we will consider separately the two cases that the BS has partial and no knowledge of the CSIT error.

CSIT error covariance feedback

In some cases, statistical properties of the CSIT error, such as mean and covariance, can be obtained at the user side. For example, if the CSIT error comes from the quantization of channel matrixes or the average of CSI feedback over a frequency band, the user can estimate the statistical properties of such error. In this circumstance, we propose a feedback-aided scheduling technique, which requires the selected users to predict the covariance of their CSIT errors and feed them back. The BS then utilizes such information to improve the scheduling result. Assuming that the capacity-maximization criterion is used for MU-MIMO scheduling, the scheduler selects a subset of users with the maximum sum capacity. From (1) we can see that the residual CCI power has an impact on the sum capacity. In the proposed technique, the BS can generates a coarse estimate of the average power of residual CCI using (2) for each user subset, based on the covariance of CSIT errors and the precoding matrixes of this subset, and take its effect into account when evaluating the sum capacity of each subset.

CCI measure feedback

In some cases, for example, when uplink sounding is used for the BS to acquire CSIT, both the users and the BS have difficulty to obtain the knowledge of CSIT error. For such circumstances, we propose a feedback-aided scheduling technique, which requires users to measure the CCI level from other users, check if it is too high, and then feed back its decision to the BS. The BS then collects feedbacks from all users and adjusts its scheduling results accordingly. The proposed technique works as follows.

Step 1: 	The BS performs conventional scheduling based on Hk assuming no CSIT error.

Step 2: 	The BS generates precoding matrixes for selected users based on Hk according to the scheduling results.

Step 3:	The BS sends precoded data sequences together with pilots to selected users.

Step 4:	Each selected user estimates the average CCI power using pilot signals, and decides whether to maintain or reduce the current user number according to a certain criterion.

Step 5:	Each selected user feeds back its decision.

Step 6:	The BS collects the feedbacks from all selected users. If a large enough part of selected users indicates cutting down the user number, it reduces the user number by 1 by dropping the user with the smallest SINR. Otherwise, it remains the original scheduling result.

Step 7:	Steps 2~6 are repeated using the updated scheduling result.

# Simulation Assumptions and Results

In this section, we use numerical results to demonstrate the advantage of the proposed feedback-aided scheduling technique. The elements in channel matrixes {Hk} are modeled as i.i.d. complex white Gaussian random variables with zero mean and unit variance. The elements in CSIT error matrixes Hk are modeled as i.i.d. complex white Gaussian noises with zero mean and varianceσe2. Here we employ NT = 4 at the base station, NR = 1 or 2 per user, a total number of 4 users in a cell, andσe2= 0.1 or 0.5. With two receive antennas per user, both MMSE and non-MMSE receivers are considered. With the MMSE receiver, the user needs to estimate the equivalent fading coefficients of all interfering users, and then use the MMSE technique to suppress inter-user interference. With the non-MMSE receiver, the inter-user interference is regarded in the same way as the additive Gaussian noise without any interference suppression operation.

Figs. 2 and 3 compare the performance of MU-MIMO systems with and without the proposed feedback-aided scheduling technique. In Fig. 2, the CSIT error covariance feedback is used and in Fig. 3, the CCI measure feedback is used. As we can see, the proposed scheduling technique can lead to considerable capacity improvement in all circumstances, especially with high CSIT error and at high SNRs. Also note that such advantage is obtained at the cost of only one or a few bits of additional feedback for each selected user and marginal complexity increase at both the transmitter and user side.

Figure 2. Performance comparison between MU-MIMO systems with and without the proposed scheduling technique based on the CSIT error covariance feedback.

Figure 3. Performance comparison between MU-MIMO systems with and without the proposed scheduling technique based on the CCI measure feedback.

# Conclusions

In this proposal, we have shown that when imperfect channel information is used at the BS, the performance of MU-MIMO can be significantly improved if users can feed back some information about the statistical property of CSIT error or CCI level. We also presented a feedback-aided scheduling technique to utilize such information at the BS to reduce the detrimental effect of CSIT error on the MU-MIMO performance. Simulation results have shown that considerable performance gain can be achieved, especially with high CSIT error or at high SNRs, at the cost of one or a few bits of additional feedback for each selected user and marginal complexity increase at both the transmitter and user side. Therefore, we recommend that user feedbacks of CSIT error information or CCI level should be supported in IEEE 802.16m systems in the MU-MIMO mode to aid MU-MIMO scheduling.

# Text Proposal to SDD

------------------------------------Start text proposal------------------------------------

Multiuser MIMO Transmission

In the MU-MIMO mode, UE feedback of CSIT error information or inter-user CCI level information should be supported to aid the MU-MIMO scheduling.

------------------------------------End text proposal------------------------------------