| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Video Streaming Traffic Model for 802.16m Evaluation Methodology Document |  |
| Date Submitted | 2007-11-07 |  |
| Source(s) | Hua Xu Motorola, Inc. 1501 West Shure Drive, Arlington Heights, IL 60004, USA | E-mail: hua.xu@motorola.com |
| Re: | Proposed modification to IEEE 802.16m-037r1 |  |
| Abstract | This document proposes changes to the video streaming modeling in the 802.16m EVM. |  |
| Purpose | Adapt this contribution to correct the video streaming model in 802.16m EVM. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Video Streaming Traffic Model

Hua Xu

Motorola, Inc.   hua.xu@motorola.com

Purpose:

The video streaming traffic model specified in the “802.16m Evaluation Methodology Document” has multiple mistakes.  The proposed corrections are described below.

Proposed Changes:

# [Replace Table 40 on page 108 line 17 with the following table and paragraph.]

| Component | Distribution | Parameters | PDF |
| --- | --- | --- | --- |
| Inter-arrival time between the beginning of each frame | Deterministic | 100 ms (Based on 10 frames per second) |  |
| Number of packets (slices) in a frame | Deterministic | 8 packets per frame |  |
| Packet (slice) size | Truncated Pareto | Mean =100 bytes, Max = 250 bytes (before truncation) | fx=αkαxα+1,k≤x<m fx=kmα,x=m α=1.2,k=40bytes,m=250bytes if x>max, discard and regenerate a new value for x |
| Inter-arrival time between packets (slices) in a frame | Truncated Pareto | Mean=6 ms, Max=12.5 ms (before truncation) | fx=αkαxα+1,k≤x<m fx=kmα,x=m α=1.2,k=2.5ms,m=12.5ms if x>max, discard and regenerate a new value for x |

Table 1: Near Real Time Video Streaming Traffic Model Parameters

The other network protocol overhead, such as IP, TCP/UDP header should be added on each packet (slice) generated by the video streaming model described above.