| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Reference Model and Protocol Architecture in SDD |  |
| Date Submitted | 2008-1-18 |  |
| Source(s) | Haihong Zheng, Shashikant Maheshwari, Yousuf Saifullah, Adrian Boariu Nokia Siemens Networks 6000 Connection Drive, TX, 75039, USA | Voice: [+1 972-894-5000] E-mail: [Haihong.Zheng@nsn.com, shashikant.maheshwari@nsn.com, Yousuf.Saifullah@nsn.com] *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m-07/047 – Call for Contribution on IEEE 802.16m System Description Document |  |
| Abstract | The contribution proposes reference model and protocol architecture in SDD document. |  |
| Purpose | Accept the proposal for the baseline SDD document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Reference Model and Protocol Architecture for SDD

Haihong Zheng, Shashikant Maheshwari, Yousuf Saifullah and Adrian Boariu

NSN

# Introduction

This contribution proposes the network reference model and protocol architecture for the 802.16 system. The     text to be inserted into SDD document is defined in the following section.

# Specific Text Change

[Insert following text into subclause 1.4 and 1.5]

1.4 Network Reference Model

This section describes the reference model for 802.16m network and WiMAX radio access network.

1.4.1 Radio Access Network Reference Model

Figure 1 shows a reference model for the radio access network for 802.16m system. It consists of Mobile Stations (MS), Relay Stations (RS), Base Stations (BS), Access Service Network Gateways (ASN GW) and Connectivity Service Network (CSN). The reference model supports both legacy 802.16e and regular 802.16m stations. 802.16m SS/RS/BS refers to 802.16m stations that comply with 802.16m specification. 802.16e SS/BS refers to stations that comply with 802.16e specification.

Figure 1: Radio Access Network Reference Model

Interface Ri (i=1..8) is defined in [1]. The 802.16m MS communicates with the 802.16m BS via the R1+ interface, and the legacy 802.16e MS communicates with the 802.16m or 802.16e BS via R1 interface, which is a subset of interface R1+. The 802.16m RS may communicate with the 802.16m BS directly via R1x+ interface, which is based on R1x interface with certain enhancement, or via one or more 802.16m RSs using R1x+’ interface. 802.16m RSs The 802.16j RS communicates with the 802.16m BS via R1x interface which is to be defined. The interfaces between the network nodes (i.e., R6+, R5+, R4+, R3+) are based on R6, R5, R4 and R3 interfaces with certain enhancement. If no enhancement to R3 and R5 interfaces is required, they remain the same as in the legacy system.

1.4.2 802.16m Network Reference Model

A simplified reference model for 802.16 network is shown in Figure 2. An 802.16m entity is the logical entity in a SS, RS or BS that comprises the PHY and MAC layers of the data/control plane. The Network Control and Management functions provide various control and management functions (including AAA, security, mobility, radio resource management, service flow management, paging and idle mode, MBS, location based services, MIH function, SS management, network management, regulatory services, multi-RAT interworking, etc.) and resides in both MS and BS. A set of SAPs provide interfaces between them and 802.16m entities.

Figure 2: 802.16m Network Reference Model

1.5 Protocol Architecture model

Protocol architecture of 802.16 networks is described in this section. Figure 3  and Figure 4 shows the user plane protocol stack on the SS and BS with and without Relay station in between. Figure 5shows the control plane protocol stack on the SS and BS. MAC layer is comprised of MAC-CS (convergence sub layer) and MAC-CPS (Common Part sub layer).

Figure 3: Data Plane Protocol Stack

Figure 4: Data Plane protocol Stack (with Relay Station)

Figure 5: Control Plane Protocol Stack

MAC – CS only exists in the user plane. It implements the functions like Header compression/Header suppression and mapping of upper layer e.g. IP packets to MAC SDU and deliver MAC SDUs to MAC CPS through MAC SAP.

MAC-CPS implements functions like ARQ, HARQ, concatenation, packing, fragmentation, scheduling and security. It also provides the functions such as system access, bandwidth allocation, radio resource management, idle mode management, sleep mode management, connection establishment, and connection maintenance, etc.

Functional decomposition of MAC protocol layer (MAC-CS and MAC-CPS) is described in Figure 6.

Figure 6: Functional decomposition of MAC Layer Protocol

Reference

[1] WiMAX End-to-End Network Systems Architecture (Stage 2: Architecture Tenets, Reference Model and Reference Points) Release 1.0.0, WiMAX Forum