| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | A New Stream Mapping Rule for Vertically Encoded STC System in IEEE 802.16m |  |
| Date Submitted | 2007-11-13 |  |
| Source(s) | Chung-Lien Ho, Ren-Jr Chen, Chang-Lan Tsai, Chang-Lung Hsiao, Chi-Fang (Richard) Li, Ting-Chen (Tom) Song, ITRI Wern-Ho Sheen, NCTU/ITRI | Voice: + 886 3 5914520 E-mail: clho@itri.org.tw richard929@itri.org.tw |
| Re: | IEEE 802.16m-07/040 - Responds to Call for Contributions on Project 802.16m System Description Document (SDD) |  |
| Abstract | A new stream mapping rule is proposed for the closed-loop vertically encoded STC framework adopted in IEEE 802.16 to enhance the link quality performance. The basic idea is to properly allocating the relatively more important systematic part of the coded bits to the better channel. Simulation results show that the new scheme can significantly outperform the mapping rule currently being used in IEEE 802.16 especially for the case of low coding rate. |  |
| Purpose | For 802.16m discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

A New Stream Mapping Rule for Vertically Encoded STC System in IEEE 802.16m

Ren-Jr Chen, Chung-Lien Ho, Chang-Lan Tsai, Chang-Lung Hsiao, Chi-Fang (Richard) Li, Ting-Chen (Tom) Song,

ITRI

Wern-Ho Sheen

NCTU/ITRI

# Summary

This contribution introduces a block-wise stream mapping to enhance the error rate performance for IEEE 802.16m. Simulation results show that the new scheme provides a significant gain over the demux-wise mapping currently used in IEEE 802. 16 system especially for a low coding rate. The method is simple and is easy to fit in the closed-loop vertically encoded STC framework adopted in IEEE 802.16.

Text Proposal

------------------------------------------------------ Start of Text Proposal ------------------------------------------------------

X.X X. STC Data Stream Mapping for IEEE 802.16m

<note: the block-wise stream mapping rule for closed-loop vertically encoded STC system should be treated as an alternative to the conventional stream mapping rule. Our study shows that the performance gain using a block-wise stream mapping rule, which is listed in Table 2, was observed to be better than conventional demux-wise stream mapping rule shown in Table 1 [1].>

------------------------------------------------------ End of Text Proposal -------------------------------------------------------

# Introduction

This contribution describes a block-wise stream mapping rule for the closed-loop vertically encoded STC system based on the channel condition to enhance the error rate performance. The new scheme provides an attractive gain over the demux-wise mapping method currently used in the IEEE 802.16 OFDMA system. The new method can co-exist with the closed-loop vertically encoded STC framework adopted in the IEEE 802.16 OFDMA system.

# Vertically Encoded STC System

Fig. 1 is the vertically encoded STC system with MIMO precoding adopted in the IEEE 802.16 system.  The convolutional turbo code (CTC) [1] using a double binary circular recursive systematic convolutional code, as is shown in Fig. 2. The input information bits (A, B) are first encoded via CTC as the systematic bits (A, B) and parity bits (Y1, Y2, W1, W2), each of which is then individually interleaved by the interleaver shown in Fig. 3. Next, the coded sequences (i.e., systematic part and parity part) are aggregated (A, B, Y1, Y2, W1, W2) and punctured to match the desired coding rate. After puncturing, all the coded bits are modulated as complex-valued symbols. The modulated symbols are then converted to multiple transmission streams via a stream mapper. Finally, these streams are precoded by a pre-designed matrix and transmitted from the multiple antennas.

Fig. 1: A block diagram for vertically encoded STC systems with MIMO precoding.

Fig. 2: CTC encoder [1].

Fig. 3: Block diagram of the interleaving scheme [1].

# Demux-Wise Stream Mapping

According to the current IEEE 802.16 standard, all the modulated symbols are serial-to-parallel converted to multiple streams by a demux-wise mapping (see Fig. 324 of [1]). Fig. 4 illustrates a demux-wise stream mapping example with 1/2 coding rate over a 2  2 MIMO channel. It is noticed that in the demux-wise stream mapping, the modulated symbols are alternately mapped to the two streams as listed in Table 1 [1], each of which is passed through different channel. As shown in Fig. 4 and Table 1, the systematic part of the coded bits is distributed evenly over the two channels, where one channel could be much worse the other. If this is case, the performance will be dominated by the worse one, especially half of the relatively more important systematic part of coded bits is placed on it.

Fig. 4: A demux-wise stream mapping example for vertically encoded STC system in current IEEE 802.16 standard.

Table 1: Symbol allocation for vertically encoded STC system with two transmit antennas in current IEEE 802.16 standard [1]

# Block-Wise Mapping Rule

As mentioned previously, IEEE 802.16 CTC generates a block of coded sequence {A, B, Y1, Y2, W1, W2}, in which A and B are the systematic part and Y1, Y2, W1 and W2 are the parity part. This sequence imposes an important structure: the systematic part always occupies the leading part of the sequence followed by the parity part. Moreover, after the sub-block interleaving and puncturing, the coded sequence still keeps the same structure.

The basic idea of the proposed block-wise mapping rule is that to allocate as many as possible the relatively more important systematic part of coded bits to the well-conditioned channel to against the destructive fading and hence enhance the error correction capability. Fortunately, the distinctive structure of the coded bits described above facilitates the development of the proposed mapping rule. With this distinctive structure, the reliable stream mapping can be easily done by directly equal-length block segmenting the punctured sequence into multiple blocks, each of which (in “block-wise”) is then individually passed through the different channel, according to the channel condition. The overall design flow for the coding rate Rc = 1/2 is shown in Fig. 5 and the corresponding symbol allocation is modified in Table 2. Compared with Table 1, the label “antenna” has been generalized to “stream” in Table 2. From this example, we can see that all the systematic part indeed can be transmitted through the better channel. However, with the coding rate Rc increased, the performance gain provided by the block-wise mapping mechanism diminishes. This is because that more systematic bits will be allocated into the worse channel due to equal-length block segmentation. It is evidenced by Fig. 5 that the proposed mapping rule is easily realized without significant change from the existing framework (Fig. 1); therefore, it is very suitable for use in the vertically encoded STC system (especially for MIMO precoding) adopted in the IEEE 802.16 system, if the channel state information (CSI) is available at the transmitter. Note that CSI is already available at the transmitter side for precoded system. For the non-precoded systems, only log2N bits are needed for CSI reporting, where N is the bit-stream number. In addition, the demux-wise stream mapping will be used if no CSI is available at the transmitter.

Fig. 5: A block-wise stream mapping example for vertically encoded STC system.

Table 2: A Modified symbol allocation for vertically encoded STC system with two streams if CSI is available at the transmitter (assume that transmission chain of stream 0 is more reliable).

# Simulation Results

In this section, the proposed stream mapping rule is evaluated via the computer simulations. We consider the vertically encoded STC system in Fig. 1. The simulation parameters are given in Table 3.

Table 3: Simulation parameters

| Parameters | Value |
| --- | --- |
| Channel model | Fixed Rayleigh flat-fading channel |
| Channel estimation | Perfect estimation at TX and RX |
| MIMO configuration | 4  2 |
| Number of streams | 2 |
| Precoding scheme | Eigen-based IEEE 16e codebook-based [1] |
| MIMO detector | LMMSE |
| MCS sets | QPSK-1/2, 2/3, 3/4, 5/6 16QAM-1/2, 2/3, 3/4, 5/6 |
| Channel coding scheme | Code rate 1/3 CTC [1] |
| Decoding scheme | Max-Log-MAP algorithm |
| Number of bits per coding block | 480 |

The first set of simulation examines the bit error rate (BER) performance of the two stream mapping schemes for eigen-based precoding (i.e., the precoding matrix is of the right singular matrix of the MIMO channel matrix) and the results are shown in Fig. 6 and Fig. 7 respectively for QPSK and 16QAM modulations with the coding rate being a control parameter. From Figs 6 and 7, we can see that the proposed mapping rule provides significant gain over the demux-based mappin for coding rate of 1/2. The improvement becomes smaller when coding rate becomes higher. Similar results are observed for the 16e vector codebook-based precoding scheme, as shown in Figs 8 and 9.

# References

- IEEE DRAFT P802.16Rev2/D1, “Part 16: Air interface for broadband wireless access systems,” October, 2007.

Fig. 6: Bit error rate performance of the two different stream mapping rules with the coding rate being a control parameter. Eigen-based precoding scheme and QPSK modulation are used.

Fig. 7: Bit error rate performance of the two different stream mapping rules with the coding rate being a control parameter. Eigen-based precoding scheme and 16QAM modulation are used.

Fig. 8: Bit error rate performance of the two different stream mapping rules with the coding rate being a control parameter. 16e vector codebook-based precoding scheme and QPSK modulation are used.

Fig. 9: Bit error rate performance of the two different stream mapping rules with the coding rate being a control parameter. 16e vector codebook-based precoding scheme and 16QAM modulation are used.