| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Enhanced HARQ technique using Constellation Rearrangement |  |
| Date Submitted | 2007-11-15 |  |
| Source(s) | Jer-Chung Lin, Ren-Jr Chen, Chung-Lien Ho, Chang-Lan Tsai, Chang-Lung Hsiao, Chi-Fang (Richard) Li,Ting-Chen (Tom) Song, Yu-Chuan Fang, ITRI Wern-Ho Sheen, NCTU/ITRI Chung-Hsuan Wang, NCTU | Voice: E-mail: whsheen@itri.org.tw richard929@itri.org.tw |
| Re: | IEEE 802.16m-07/040 - Call for Contributions on Project 802.16m System Description Document (SDD) |  |
| Abstract |  |  |
| Purpose | For 802.16m discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Enhanced HARQ technique using Constellation Rearrangement

Jer-Chung Lin, Ren-Jr Chen, Chung-Lien Ho, Chang-Lan Tsai, Chang-Lung Hsiao,

Chi-Fang (Richard) Li, Ting-Chen (Tom) Song, Yu-Chuan Fang, ITRI

Wern-Ho Sheen, NCTU/ITRI

Chung-Hsuan Wang, NCTU

# Summary

An enhanced HARQ method using signal constellation rearrangement is presented in this contribution, where different mappings are used for retransmission to improve error rate performance. The proposed method has shown significant gain over the HARQ without constellation rearrangement. This contribution would like to propose to adopt this technique to SDD HARQ section for further investigation.

# Proposed SDD Text

# -----------------------------Begin Proposed Text ----------------------------------------------------------------------

X. HARQ

A HARQ method with signal constellation rearrangement for 16-QAM and 64-QAM modulation shall be used.

---------------------------End of Text Proposal ---------------------------------------------------------------------

# Introduction

This contribution proposes a signal constellation rearrangement technique to enhance HARQ performance; Different mappings are used for retransmission and MRC (maximal ratio combining) are employed at the receiver to combine all the received symbols to improve performance . The simulation results show that significant gain is achieved over the traditional Chase Combing.

- Description of Proposed Method

The inherent different bit reliabilities for different bit positions in Gray labeling, bit reliabilities remain unchanged if identical mappings are used for retransmission in HARQ. To average out the un-even bit reliabilities, we swap the less reliable bits with more reliable bits for retransmission. The averaged bit reliability will enhance the HARQ performance over the Chase Combining.

## 2.1 Constellation rearrangement for 16 QAM

For Gray labeling in 16QAM as shown in Figure 2.1.1, bit  is more reliable than. If the same labeling is retransmitted for repeated bits, the reliability of bit  will be further higher than  . To resolve the un-even bit reliabilities, we swap bit  with bit  and obtain different mapping for retransmission. Similar argument applies to  and

| Transmission No. | Bit mapping order | Figure No. |
| --- | --- | --- |
| 1 |  | Figure 2.1.1 |
| 2 |  | Figure 2.1.2 |
| 3 |  | Figure 2.1.3 |
| 4 |  | Figure 2.1.4 |
| Further transmissions | Repeatedly using constellations from transmissions 1-4 | Figure 2.1.1~2.1.4 |

Figure 2.1.1~2.1.4 Constellation rearrangement for 16 QAM (bars indicating rows/columns for which the respective bit equals 1)

## 2.2 Constellation rearrangement for 64 QAM

For Gray labeling in 64QAM, there are there different reliabilities levels as shown in Figure 2.2.1. The reliability levels from high to low are,,and  respectively.

Similar to the approach for 16 QAM, we swap bits with different bit reliability levels to obtain better performance.

| Transmission No. | Bit mapping order | Figure No. |
| --- | --- | --- |
| 1 |  | Figure 2.2.1 |
| 2 |  | Figure 2.2.2 |
| 3 |  | Figure 2.2.3 |
| 4 |  | Figure 2.2.4 |
| 5 |  | Figure 2.2.5 |
| 6 |  | Figure 2.2.6 |
| Further transmissions | Repeatedly using constellations from transmissions 1-4 | Figure 2.2.1~2.2.6 |

Figure 2.2.1~2.2.6 Constellation rearrangement for 64 QAM (bars indicating rows/columns for which the respective bit equals 1)

# Simulation Results

This section evaluates the error rate performance for both retransmission methods—the proposed HARQ method with constellation rearrangement vs. the Chase Combining. The simulation environment is AWGN channel. Log-MAP algorithm with 8iterations is performed in our simulation. The following table shows the simulation parameters including packet size, modulation and code rate.

| Modulation | Information bits per packet | Turbo code rate | Number of transmission | Figure No. |
| --- | --- | --- | --- | --- |
| 16-QAM | 96 | 1/2 | 1~4 | Figure 3.1 |
| 64-QAM | 144 | 1/2 | 1~4 | Figure 3.2 |

Figure 3.1 shows that the proposed strategy has a gain over Chase Combining of about 1.3dB, 2dB and 2.6dB for the first, second and third retransmission, respectively. The 2nd retransmission of the proposed method performs significantly better than the 3rd retransmission utilizing Chase Combining. I.e. instead of 3 retransmissions using Chase Combining two retransmissions with the proposed method would be sufficient.

Figure 3.1 Performance comparison with Nep=96bits and 16QAM modulation.

Figure 3.2 shows that the proposed strategy has a gain over Chase Combining of about 2.2dB, 4dB and 5dB for the first, second and third retransmission, respectively. In this case, we only requires one retransmission with the proposed strategy to outperform Chase Combining method.

Figure 4 Performance comparison with Nep=144bits and 64QAM modulation.

# Conclusions

In this contribution a new HARQ method using signal constellation rearrangement is presented. When transmitting repeated bits is required, this new method uses different mappings for retransmission and performing MRC (maximal ratio combining) to combine all the received symbols. The proposed method has shown significant gain over the conventional schemes.

# References

[1]	 R1-01-0237, Panasonic, “enhanced HARQ method with signal constellation rearrangement” Las Vegas, USA February 27- March 2, 2001