| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | High Level Protocol architecture to support multicarrier system |  |
| Date Submitted | 2008-01-22 |  |
| Source(s) | Jaehee Cho, Yungsoo Kim, Jaeweon Cho, Mihyun Lee, Hokyu Choi, Samsung Electronics Kamran Etemad, Intel Naftali Chayat Alvarion Mo-Han Fong, Nortel | E-mail: jaehee1.cho@samsung.com Kamran.etemad@intel.com Naftali.chayat@alvarion.com mhfong@nortel.com |
| Re: | 802.16m Protocol Architecture |  |
| Abstract | The contribution proposes a generic protocol architecture to support multicarrier system to be included in the 802.16m System Description Document (SDD). |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m SDD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

High Level Protocol architecture to support multicarrier system

Jaehee Cho,Yungsoo Kim,Jaeweon Cho, Mihyun Lee, Hokyu Choi
Samsung Electronics

Kamran Etemad

Intel

Naftali Chayat

Alvarion

Mo-Han Fong

Nortel

# Proposed Text for SDD

Insert the following text into Air-Interface Protocol Structure subclause (i.e. Chapter 8 in [1]):

-------------------------------  Text Start  ---------------------------------------------------

8.x  Basic Protocol architecture for Multicarrier Support

Generic protocol architecture to support multicarrier system is illustrated in Figure X. A common MAC entity may control a PHY spanning over multiple frequency channels. Some MAC messages sent on one carrier may also apply to other carriers. The channels may be of different bandwidths (e.g. 5, 10 and 20 MHz), be non-contiguous or belong to different frequency bands. The channels may be of different duplexing modes, e.g. FDD, TDD, or a mix of bidirectional and broadcast only carriers.

The multicarrier functionality may similarly be applied to multiple groups of subchannels within the frequency channels.

The MAC entity may support simultaneous presence of MSs with different capabilities, such as operation over one channel at a time only or aggregation across channels, operation over contiguous or non-contiguous channels.

Figure X. Generic protocol architecture to support multicarrier system

-------------------------------  Text End  ---------------------------------------------------

# References

[1] IEEE C802.16m-07/320r1, “Draft Table of Content for the IEEE 802.16m System Description Document.”