| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Calculation of Interference between Co-existing Multi-radio Systems and Their Required Minimum Separation Distance |  |
| Date Submitted | 2008-09-16 |  |
| Source(s) | Yih-Guang Jan, Yang-Han Lee, Hsien-Wei Tseng, Ming-Hsueh Chuang, Sheng-Bo Huang, Chih-Hsiang Tseng Tamkang University (TKU) Shiann-Tsong Sheu National Central University (NCU) Whai-En Chen National Ilan University (NIU) | yihjan@yahoo.com stsheu@ce.ncu.edu.tw wechen@niu.edu.tw |
| Re: | PHY: Multi-Radio Coexistence; in response to the TGm Call for Contributions and Comments 802.16m-08/033 for Session 57 |  |
| Abstract | This contribution proposes for calculation of interference between co-existing multi-radio systems and their required minimum separation distance |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m SDD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Calculation of Interference between Co-existing Multi-radio Systems and Their Required Minimum Separation Distance

Yih-Guang Jan, Yang-Han Lee, Hsien-Wei Tseng, Ming-Hsueh Chuang, Sheng-Bo Huang, Chih-Hsiang Tseng

Tamkang University (TKU)

Shiann-Tsong Sheu

- National Central University (NCU)

- Whai-En Chen

- National Ilan University (NIU)

- 1. Introduction

In this contribution we study the interference between the IEEE 802.16m system and other system and determine their required minimum separation distance for allowed interference level. Due to the rapid development of communication technologies, communication systems to meet various requirements have been developed and produced. Each system has its own working frequency and bandwidth, various systems with close working frequencies may be unable to operate in normal fashion due to the mutual interference; therefore the study of interference effect has been continuously proposed and the determination of minimum separation distance between close communication systems is one of the important issues in the study.

- 2. Determination of Minimum Separation Distance

- 2.1 System Spectrum Distribution and Interference Evaluation

In the evaluation of frequency sharing, it is based on the permitted interference level at the antenna terminal of the interfered or victim system. In order to ensure satisfactory co-existence of the 802.16m system and other system, it is important to be able to predict the interference level between these two systems to reasonable accuracy. Various prediction processes and models have been discussed, proposed and accepted by international communication organizations; and can be utilized in the prediction of interference between systems. ITU-R Recommendation P.452-8 [2] that has been accepted and adopted by ITU and CEPT is an evaluation model to assess the interference on the radio systems at microwave frequencies.

The required minimum attenuation loss, in dB, of the interfering system on the victim system can be determined from the following equation [3]:

Lmin=Pt+Gt+Gr+Lr−Imax		         (1)

Pt:	the transmit power of the interfering system in the reference bandwidth, in dBW

Gt:	the transmitter antenna gain of the interfering system, in dBi

Gr:	the receiver antenna gain of the victim receiver, in dBi

Lr:	Power loss of the interfering signal

Imax:	Maximum permissible interference power

The interfering signal power loss Lr for the 802.16 m interfering system can be derived through a power spectral density of the 802.16 m system. Assume the 802.16m system has N sub-carriers with rectangular pulse; its power spectral density can be expressed as:

SS(f)=i=0N−1PsRssinc2(fRs−i)					   (2)

wherePs is the power of a single 802.16m sub-carrier andRsis the sub-carrier frequency spacing and sinc(x)=sin(πx)πx.

Assume the other system has a rectangular power spectral density as:

Sv(f)=PvWvrect(fWv)				    		(3)

where Pv and Wvare the transmitting power and the bandwidth of the other system.

In the following it shows the spectrum overlapping of the 802.16m and the other systems, where in the figure the bandwidth Wvis assumed to be smaller than the bandwidth of the 802.16m system.

Fig. 1 PSD of 802.16m interferer overlapping PSD of other system

The interfering signal power attenuation can be expressed by the spectrum overlapping ratio as:

Lr=10log10Ss(f)dfPt				(4)

2.2 The Prediction of Minimum Separation Distance

From the predicted interfering signal power attenuation loss as derived in the above, the antennas gains of the interfering and the victim systems it can determine the minimum separation distance required between systems to ensure their co-existence with proper system performance.

The free space path loss can be found from the following equation:

Lmin (path loss)=−10logλ2((4π)2d2)=−10log(cf)2((4π)2d2)		(5)

where c is the light velocity in meter, fis the system’s center frequency (f=fc) and d is the distance in meter.

By using the derived equations (1) – (5) we list in the following the simulation results of the minimum required separation distance between these two systems when the interfering signal power varies from 0 dBW to 17 dBW. In the simulation it assumes that it has cluster loss of 20 dB in the communication path and it has return loss of 70 dB or 50 dB on the transmitting and receiving antenna.

Fig. 2 Minimum Separation Distance (clutter loss=20dB)

The parameters used in the simulations are listed in the following table

| Spectrum parameters of 802.16m system |  |
| --- | --- |
| Pt | 0~17dBW |
| Ps | (Pt/1024) W |
| Rs | 10.24kHz |
| N | 1024 |
| Parameters for LrCalculation |  |
| fc | 2500 MHz |
| Wv | 40 MHz |
| Parameters for LminCalculation |  |
| Gt | 14.5dBi |
| Gr | 42.5dBi |
| I max | -140.5 dBW/40MHz |

- Text Proposal for the ‘Multi-Radio Coexistence’

===================Start of Proposed Text======================

- 17 Solutions for Co-deployment and Co-existence

- 17.x  Determination of Minimum Separation Distance between Two Systems

17.x.1  System Spectrum Distribution and Interference Evaluation

In the evaluation of frequency sharing, it is based on the permitted interference level at the antenna terminal of the interfered or victim system. In order to ensure satisfactory co-existence of the 802.16m system and other system, it is important to be able to predict the interference level between these two systems to reasonable accuracy. Various prediction processes and models have been discussed, proposed and accepted by international communication organizations; and can be utilized in the prediction of interference between systems.

17.x.2  The Prediction of Minimum Separation Distance between Two Systems

From the predicted interfering signal power attenuation loss as derived in the above, the antennas gains of the interfering and the victim systems it can determine the minimum separation distance required between systems to ensure their co-existence with proper system performance.

===================End of Proposed Text======================

- References

- CEPT ERC Report 101: ‘A comparison of the minimum coupling loss method, enhanced minimum coupling loss method, and the Monte-Carlo simulation’, May 1999

- ITU-R Recommendation P. 452–8: ‘Prediction procedure for the evaluation of microwave interference between stations on the surface of the earth at frequencies above about 0.7 GHz’

- W.G Chung, et al,” Advanced MCL method for Sharing Analysis of IMT-advanced System”, Electronic Letters, 12th October, 2006, Vol. 42, No.21.