| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Downlink Control Structure for Multicarrier operations |  |
| Date Submitted | 2008-09-05 |  |
| Source(s) | Chie Ming Chou, Richard Li, Frank Ren, Chun-Yen Wang, ITRI | Voice: E-mail: chieming@itri.org.tw richard929@itri.org.tw |
| Re: | Multi-carrier operation; in response to the TGm Call for Contributions and Comments 802.16m-08/033 for Session 57 |  |
| Abstract | This contribution proposes the designs of downlink control structure for supporting Multicarrier operations. |  |
| Purpose | For discussion and approval by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Downlink Control Structure for Multicarrier operations

Chie Ming Chou, Richard Li, Frank Ren, Chun-Yen Wang

ITRI

- Introduction

Supporting Multicarrier is one function of 802.16m. In SDD [1], downlink control for single carrier operations have been defined. In this contribution, we will extend the designs to Multicarrier. Network entry, carrier switching, and resources allocation for Multicarrier operations will be take into consideration and regarding proposal is suggested.

- Description of the proposal

Multicarrier will allow network to operate in continuous or discontinuous frequency with different bandwidth. A separate FFT and RF module is used at each band, and one MAC entity is required to support Multicarrier functionalities. For Multicarrier control structure, the carriers may be configured as fully configured carrier and partially configured carrier. The fully configured carrier includes all control channels as same as a normal single carrier. The partially configured carrier may include essential control channel configuration and support MS access according to the indications of fully configured carrier. Besides, there are two operations which have been already described in SDD: (1) carrier aggregation (2) carrier switching. To efficiently support these two operations, the downlink/uplink control structure regarding to Multicarrier need special design. Before introducing the proposal of Multicarrier DL control structure, the consensus about downlink structure of single carrier in current SDD is addressed as follows:

  - The DL control channels contain the synchronization control channel (SCH), broadcast control channel (BCH), and unicast service control channel (USCCH).

  - SCH provides a reference signal for time, frequency, frame synchronization and BS identification. The allocation of SCH is fixed in the first sub-frame within a superframe.

  - BCH provides essential system parameters and system configuration information. The BCH can be further divided into PBCH and SBCH. The PBCH carries deployment wide common information. The SBCH carries sector specific information. Both PBCH and SBCH are included in SFH. The BCH is TDM with the SCH and FDM with data within the same sub-frame.

  - USCCH consists of both user-specific control information and non-user-specific control information. The first 802.16m DL sub-frame of each frame contains user-specific control information.

For Multicarrier downlink control, there are some considerations:

    - Initial network entry

During initial network entry, a MS needs to achieve downlink and uplink synchronization and acquire required information. For MS’s perspective, a MS will be served by a primary carrier and this primary carrier will be a fully-configured carrier which includes all control information. The feasible initial network entry procedure is let MS perform network entry at a fully configured carrier and then decide to initiate other partially configured carriers’ support or not. As a result, a MS can be guaranteed to get complete system information to successfully finalize the network entry process.

    - Carrier switching

There are two cases of carrier switching:

      - Switch between a primary carrier and a secondary carrier: This switching can be periodic or event-triggered. When periodic switching is adopted, a pre-defined indication is required to give on primary carrier first to identify the life interval of each carrier. And then a MS can perform switching based on this scheduling (see Figure 1 as an example). When event-trigger switching is adopted, an indication can be given on each carrier and perform corresponding switching.

Figure 1; an example of periodic carrier switching

      - Switch between a serving primary carrier and another primary carrier: This switching can be categorized into one scenario of handover process. An optimization intra-BS inter-frequency HO procedure will be applied.

    - Resource allocations

In Multicarrier, a MS may be able to transmit data through primary carrier or secondary carrier. How to indicate the resource allocations with low overhead and simple operation is a key issue.

As considering these issues, to help MS accesses to a fully configured carrier without any confusion, we propose that SCH will only be allocated at fully configured carriers. When a MS tries to perform cell selection, only fully configured carriers will be the candidates. A MS will select one of fully configured carrier as its primary carrier and do the initial network entry. Because that there is no SCH in partially configured carrier, when a MS switches to the partially configured carrier, its primary carrier shall provide the frequency offset to adjust the DL synchronization. Pilot structure and periodic ranging process can facilitate the adjustment. The details are FFS.

For PBCH in Multicarrier, a MS can read the essential system parameters on its primary carrier before initiating the Multicarrier support. This information is all the same in a BS and place this channel in secondary carrier will be a redundancy. Therefore we propose that only fully configured carrier needs to allocate PBCH with carrying associated information. For SBCH, it needs to indicate the sector specific information and the content may be different in each carrier. To support periodic carrier switching as addressed above, the SBCH of fully configured carrier shall have the capability of indicating associated partially configured carriers’ sector information. Figure 2 is an example in which the SBCH of fully configured carrier needs to point out the location of partially configured carriers’ USCCHs. Under this design, a MS can read the SFH at fully configured carrier first and switch to partially configured carrier for data transmission without going back to fully configured carrier in this superframe. For a partially configured carrier, it can optionally contain its SBCH.

Figure 2; an example of BCH design in Multicarrier

- Proposed Text

---------------------------------------------Start of the text------------------------------------------------

19 Support for Multi-carrier

[Insert the following text in Section 19 of IEEE 802.16m-08/003r4]

19. X. DL control structure for Multi-carrier

In a fully configured carrier, it includes SCH, PBCH, SBCH, and USCCH. The location and transmission format of these channels are same as descriptions in section 11.7. The SBCH carriers sector specific information with respect to itself and associated partially configured carriers in multi-carrier operations.

In a partially configured carrier, it includes USCCH and the allocation of SBCH is optional.

19. XX. Carrier switching for Multi-carrier

Carrier switching between a primary carrier and a secondary carrier can be periodic or event-triggered. When periodic switching is adopted, a pre-defined indication is required to give on primary carrier first to identify the life interval of each carrier. And then a MS can perform switching based on this scheduling. To support periodic carrier switching, the SBCH of fully configured carrier shall have the capability of indicating associated partially configured carriers’ sector information. When event-trigger switching is adopted, an indication can be given on each carrier and perform corresponding switching.

When a MS switches to the partially configured carrier, its primary carrier shall provide the frequency offset to adjust the DL synchronization. Pilot structure and periodic ranging process can facilitate the adjustment. The details are FFS.

---------------------------------------------End of the text------------------------------------------------

Reference

[1]	802.16m-08/003r4 “The Draft IEEE 802.16m System Description Document”

[2]	C802.16m-08/237 “Comments and Proposed SDD Text for Multi-carrier Frame Structure”.