| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Multi-carrier Operations and Channels in IEEE 802.16m |  |
| Date Submitted | 2008-09-05 |  |
| Source(s) | Richard Li, et. al. ITRI | Voice: +886-3-5917596 E-mail: richard929@itri.org.tw *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Multi - Carrier Operation; in response to the TGm Call for Contributions and Comments 802.16m-08/033 for Session 57 |  |
| Abstract | This contribution introduces more system design considerations for multi-carrier operations in IEEE 802.16m. |  |
| Purpose | For discussion and approval by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Multi-carrier Operations and Channels in IEEE 802.16m

Richard Li, et. al.

ITRI

# Introduction

This contribution presents multi-carrier operations and its channels design concept for IEEE 802.16m systems. In an IEEE 802.16m system, a BS is allowed to have multiple carrier frequencies to provide services for MSs. It is important for both BSs and MSs to operate through multi-carrier channels in an efficient way. The following section, first of all, provides modification on the existing text in current 802.16m SDD for efficient multi-carrier operations. More design concept will be provided in the revised version.

# Proposed text in multi-carrier operations

---- Text Proposal ----------------------------------------------------

# 19. Support for Multi-carrier

## 19.1 Multi-carrier operation Principles

The following is common in all modes of multi-carrier operation:

- The system defines N standalone fully configured RF carriers as defined in section Error: Reference source not found, each fully configured with all synchronization, broadcast, multicast and unicast control signaling channels. Each MS in the cell is connected to and its state being controlled through only one of the fully configured carriers as its primary carriers.

- The system defines M (M >= 0) partially configured RF carriers as defined in section 11.6.4, 11.7.4 each configured with the essential control channel configuration to support traffic exchanges during multicarrier operation.

- In the multicarrier operation a common MAC can utilize radio resources in one or more of the secondary carriers, while maintaining full control of MS mobility, state and context through the primary carrier.

- Some information about the secondary carriers including their presence and location shall be made available to the user MS through the primary carriers. The primary carrier may also provide user MS the information about the configuration of the secondary carrier.

- The resource allocation to a MS can span across a primary and multiple secondary RF carriers. Link adaptation feedback mechanisms should incorporate measurements relevant to both primary and secondary carriers.

- A multi-carrier system may assign secondary carriers to an MS in the downlink and/or uplink asymmetrically based on system load (i.e., for static/dynamic load balancing), peak data rate, or QoS demand.

- In addition to its primary RF carrier data transfer between a BS and an MS may dynamically utilize resources across multiple secondary RF carriers. Multiple MS, each with a different primary RF carrier may also share the same secondary carrier.

- The multiple carriers may be in different parts of the same spectrum block or in non-contiguous spectrum blocks. The use of non-contiguous spectrum blocks may require additional control information on the secondary carriers.

- Each MS will consider only one fully configured RF carrier to be its primary carrier in a cell. A secondary carrier for an MS, if fully configured, may serve as primary carrier for other MS’s.

There are two scenarios to multicarrier deployment.

Scenario 1: All carriers in the system are fully configured to operate standalone and may support some users as their primary carrier and others as their secondary carrier.

Scenario 2: In addition to fully configured and standalone RF carriers the system also utilizes additional partially configured supplementary radio carriers optimized as data pipes for certain services or traffic types using limited control signaling capability. Such supplementary carriers may be used only in conjunction with a primary carrier and cannot operate standalone to offer IEEE 802.16m services for a MS.

In multi-carrier operation, an MS can access multiple carriers. The following multi-carrier operations are identified:

- Carrier aggregation

  - MS shall always maintain its physical layer connection and monitor the control information on the primary carrier. MS can, in addition, access on secondary channels for throughput improvement, etc.

- Carrier switching

  - MS can switch its physical layer connection from the primary to the secondary carrier per BS’ instruction. When the MS is connected to the secondary carrier, the MS doesn’t need to maintain its physical layer connection to the primary carrier.

  - This mode may be used for the cases of single radio MS.

---- End of text proposal ---------------------------------------------------