| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Harmonized SDD Text Proposal on Link Adaptation by Link Rapporteur Group |  |
| Date Submitted | 2008-09-12 |  |
| Source(s) | I-Kang Fu Ali T. Koc Baowei Ji Link Rapporteur Group Chairs | IK.Fu@mediatek.com ali.t.koc@intel.com baowei.ji@samsung.com |
| Re: | PHY: Link; in response to the TGm Call for Contributions and Comments 802.16m-08/033 for Session 57 |  |
| Abstract | The harmonized text proposal on Link Adaption based on the latest consensus among Link RG participants with majority support |  |
| Purpose | Recommend TGm to adopt the harmonized text proposal into P802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Harmonized SDD Text Proposal on Link Adaptation

by Link Rapporteur Group

I-Kang Fu, Ali T. Koc and Baowei Ji

Link Rapporteur Group Chairs

# Introduction

The following text proposal on Link Adaption is prepared by Link Rapporteur Group (RG) Chairs based on the latest harmonization progress among the Link RG participants. There are three kinds of the text in the following text proposal:

1) The text in black color. This is the text already adopted in Link RG intermediate draft documents with the majority support from Link RG participants.

2) The bracketed text in other colors (e.g. red, purple, green etc.). This is the text proposed by members which have not been harmonized among Link RG.

# Text Proposal

-----------------------------------------------------------Start of the Text---------------------------------------------------------

11. x Link Adaptation

This section introduces the Link Adaption schemes which will adaptively adjust radio link transmission formats in response to change of radio channel for both downlink and uplink. .

11. x.1 DL Link Adaptation

11. x.1.1 Adaptive modulation and channel coding scheme

IEEE 802.16m shall support the adaptive modulation and channel coding (AMC) scheme for DL transmission. The serving BS can adapt the modulation and coding scheme (MCS) level based on the DL channel quality indicator (CQI) reported from MS. The definition of CQI is FFS.

11. x.2 UL Link Adaptation

11. x.2.1 Adaptive modulation and channel coding scheme

IEEE 802.16m shall support the adaptive modulation and channel coding (AMC) scheme for UL transmission. The serving BS can adapt the modulation and coding scheme (MCS) level based on the UL channel quality estimation and the maximum transmission power by MS. The definition of UL channel indicator is FFS. Note that the UL AMC may be integrated with UL power control and interference mitigation schemes to further achieve higher spectral efficiency.

11. x.3 Transmission Format

[Note: The content of this section shall not contradict with the transmission format determined by HARQ RG and PHY text RG]

IEEE 802.16m should support the transmission format used in legacy system. IEEE 802.16m transmission format is FFS. Transmission formats with channel code rates equal to the mother rate of the encoder (i.e. 1/3) will be supported.

An extended MCS table supporting some legacy transmission formats is shown in Table1.

Table 1: Modulation and Coding Schemes for 802.16m

[ Unresolved Text#1

11.x.4 Network Coding

For bi-directional communication within a cell, base station encodes multiple incoming streams from multiple mobile stations in uplink, and multicasts the encoded information to those mobile stations in downlink. In order to adapt to different MSs’ packet sizes and link conditions, network coding can be applied after channel coding.

]

[ Unresolved Text#2

11. x.4 MIMO Mode Selection

MIMO Mode Selection shall be supported. The MIMO Mode shall be selected based on measurements made at the user terminal side. The base station shall be aware of the criteria used at the user terminal side.

]

------------------------------------------------------------End of the Text---------------------------------------------------------

# References

- IEEE C802.16m-08/003r4, “Project 802.16m System Description Document (SDD)”.