| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Enhanced MBS Operation with Multi-Carrier Support for IEEE 802.16m |  |
| Date Submitted | 2008-09-05 |  |
| Source(s) | Kelvin Chou Yih-Shen Chen I-Kang Fu Paul Cheng MediaTek Inc. | Kelvin.Chou@mediatek.com Yihshen.Chen@mediatek.com IK.Fu@mediatek.com Paul.Cheng@mediatek.com |
| Re: | MAC: Multi-Carrier Operation; in response to the TGm Call for Contributions and Comments 802.16m-08/033 for Session 57 |  |
| Abstract | This contribution presents the scenarios of enhanced MBS with multi-carrier support. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m SDD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Enhanced MBS Operation with Multi-Carrier Support for IEEE 802.16m

Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng

MediaTek Inc.

# Introduction

With the support of multiple RF carriers, the BS can provide MBS on dedicated RF carriers other than its unicast carriers. The advantage is to provide more stable bandwidth resources for MBS without interleaving with the normal unicast transmission. A BS serving multiple MBS zones can either multiplex connections of multiple MBS zones onto a single RF carrier, or dedicate one RF carrier for each MBS zone. The MS with multiple RF carriers may maintain its normal operations with the serving BS on the primary carrier while subscribing MBS via the secondary RF carrier, as illustrated n Figure 1.

Figure 1. Dedicated RF carriers for MBS

The subscription of MBS can be arranged on either the primary carrier or the secondary carrier, as illustrated in Figure 2.

Figure 2. Example: using secondary carrier for MBS service flows

The MBS service flows may be conveyed on a single carrier, or be split on multiple carriers. However, service flows of the same MBS zone should be transmitted on the same RF carrier, as illustrated in Figure 3. The arrangement of the MBS flows should be made according to the status of RF carriers.

Figure 3. An example of splitting MBS service flows on multiple carriers

Feedback channel for the MBS connections may be provided either on the MBS carrier or on the unicast carrier (for registered MSs), as illustrated in Figure 4.

Figure 4. EMBS connection feedback via unicast carrier

During inter-MBS zone transition, the MS with multiple RF carriers can keep its existing MBS flows uninterrupted and perform MBS zone switch operations, such as MCID/LCID update, via another RF carrier.

Figure 5. Seamless inter-MBS zone transition

For the broadcast/multicast of high-definition multimedia data which is coded in layers (e.g. base layer and enhanced layer), the MBS agent may choose to provide base-layer data via one MBS zone while the enhanced-layer via another MBS zone. The MBS zone for base-layer data may use a more robust MCS for guaranteed coverage and baseline QoS, while the MBS zone for enhanced-layer data may adopt higher modulation order and/or code rate for better spectrum efficiency, as illustrated in Figure 6.

Figure 6. Layer-coded multimedia streaming via MBS zones with different MCS

MBS zones of different MCS may be provided on different RF carriers. The MS with multi-carrier support may subscribe to one MBS zone of base-layer data streaming for a perceptual video quality, and, if available, subscribe to the MBS zone of enhanced-layer data for the premium video quality, as illustrated in Figure 7.

Figure 7. Seamless inter-MBS zone transition

# Proposed Text for SDD

[In 80216m-08_003r4, Section 15, add the following text]

-------------------------------  Text Start  ------------------------------------------------------------------------------------

# 15.x Enhanced MBS with Multi-Carrier Support

With the support of multiple RF carriers, the BS can provide MBS on dedicated RF carriers other than its unicast carriers. A BS serving multiple MBS zones can either multiplex connections of multiple MBS zones onto a single RF carrier, or dedicate one RF carrier for each MBS zone. The MS with multiple RF carriers may maintain its normal operations with the serving BS on the primary carrier while subscribing MBS via the secondary RF carrier.

During inter-MBS zone transition, the MS with multiple RF carriers can keep its existing MBS flows uninterrupted and perform MBS zone switch operations, such as MCID/LCID update, via another RF carrier.

For the broadcast/multicast of high-definition multimedia data which is coded in layers (e.g. base layer and enhanced layer), the MBS agent may choose to provide base-layer data via one MBS zone while the enhanced-layer via another MBS zone. The MBS zone for base-layer data may use a more robust MCS for guaranteed coverage and baseline QoS, while the MBS zone for enhanced-layer data may adopt higher modulation order and/or code rate for better spectrum efficiency. MBS zones of different MCS may be provided on different RF carriers. The MS with multi-carrier support may subscribe to one MBS zone of base-layer data streaming for a perceptual video quality, and, if available, subscribe to the MBS zone of enhanced-layer data for the premium video quality.

-------------------------------  Text End  ------------------------------------------------------------------------------------