| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MS Initialization, Network Entry and Ranging to Support Multi-carrier Operation in IEEE 802.16m |  |
| Date Submitted | 2008-09-05 |  |
| Source(s) | I-Kang Fu, Yih-Shen Chen, Kelvin Chou, Stanley Hsu, Dora Chen, Paul Cheng MediaTek Inc. No.1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan, R.O.C. | IK.Fu@mediatek.com |
| Re: | 802.16m-08/033, “Call for Contributions and Comments on Project 802.16m System Description Document (SDD),” - MAC: Multi-Carrier Operation |  |
| Abstract | This contribution proposes the multi-carrier MS shall perform network entry and Ranging over the primary carrier first, then it can activate the data transmission over the secondary carrier. |  |
| Purpose | To facilitate the harmonization on multi-carrier operation for TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MS Initialization, Network Entry and Ranging to Support Multi-carrier Operation in IEEE 802.16m

I-Kang Fu, Yih-Shen Chen, Kelvin Chou, Stanley Hsu, Dora Chen, Paul Cheng

MediaTek Inc.

- Introduction

In P802.16m System Description Document (SDD) [1], the current text has the following definition on primary carrier (line#6, page#64):

A Primary carrier is the carrier used by the BS and the MS to exchange traffic and full PHY/MAC control information defined in 16m specification. Further, the primary carrier is used for control functions for proper MS operation, such as network entry. Each MS shall have only one carrier it considers to be its primary carrier in a cell.

The text shows that the members already have consensus that the multi-carrier MS shall perform network entry with the network via primary carrier first. Along this line, it is very obvious that the MS shall not activate its secondary carrier before completing the network entry procedure in primary carrier.

In order to clarify the network entry procedure to support multi-carrier MS, this contribution proposes the following figure to help member understand the network entry procedure for multi-carrier MS. So that member can base on this general idea to prepare and develop the detail designs and text proposals for stage-3 document.

- Text Proposal

---------------------------------------------------------Start of the Text-----------------------------------------------------------

[Add the following text and figure after the line#36, page#80 in 802.16m-08/003r4]

19.1.1 Initialization Procedure for Multi-carrier MS

Figure x shows the general initialization procedure for the MS equipped with multiple RF carriers. In order to simplify the network entry procedure, the multi-carrier MS shall perform the initial network entry with the serving BS through its primary carrier first. Then it may activate other carriers based on the negotiation with the serving BS.

Figure x Basic network entry procedure for multi-carrier MS

----------------------------------------------------------End of the Text-----------------------------------------------------------

References

[1] S. Hamiti, “The Draft IEEE 802.16m System Description Document,” IEEE 802.16m-08/003r4, July 2008.