| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | UL Power Control Procedure for Multi-Carrier in IEEE 802.16m |  |
| Date Submitted | 2008-09-09 |  |
| Source(s) | Yih-Shen Chen, I-Kang Fu, Kelvin Chou, and Paul Cheng MediaTek Inc. | Yihshen.Chen@mediatek.com IK.Fu@mediatek.com Paul.Cheng@mediatek.com |
| Re: | MAC: Multi-Carrier Operation; in response to the TGm Call for Contributions and Comments 802.16m-08/033 for Session 57 |  |
| Abstract | This contribution discusses the UL power control procedure IEEE 802.16m with multi-carrier support |  |
| Purpose | To be discussed and adopted by TGm for the P802.16m SDD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

UL Power Control Procedure for Multi-Carrier in IEEE 802.16m

Yih-Shen Chen, I-Kang Fu, Kelvin Chou and Paul Cheng

MediaTek Inc.

- Introduction

UL power control mechanism is used to compensate channel variation, which consists of open-loop power control (OLPC) and closed-loop power control (CLPC). The OLPC deals with long-term fading and the CLPC deals with short-term fading. The long-term fading comes from path-loss and shadowing, which is frequency-irrelevant. The short-term fading comes from multi-path effect, which is frequency-selective. Therefore, in multi-carrier transmission, all the carriers encounter the same long-term fading, but different short-term fading. In the contribution, we propose an efficient multi-carrier power control procedure for IEEE 802.16m.

- Efficient multi-carrier power control procedure

The basic concept of multi-carrier power control is to perform long-term fading (PL) measurement onto primary carrier and perform CLPC onto the carriers that convey uplink transmissions. In the multi-carrier operation, as shown in Fig. 1, MS firstly has to synchronize with the one of the primary carriers of a cell. During the synchronization procedure, the MS measures the downlink preamble, calculates the long-term fading and performs ranging with its serving cell. After that, it keeps calculating PL value and performs OLPC to compensate long-term fading by power control equation.

Figure 1: Initialization of primary carrier

Upon the activation of the second carrier, the MS performs ranging by using primary carrier’s current transmission power as initial value (Fig. 2). After the ranging adjustment, MS derives “carrier-offset” parameter for OLPC. The “carrier-offset” parameter indicates the power difference between primary and secondary carriers. It is not necessary to calculate PL for the secondary carrier. Every time the long-term fading value is changed in the primary carrier, it is directly applied to the secondary one. Since the primary and secondary carrier may belong to different FFR region, it is possible that the fractional compensation factors for the two carriers are different though the PL value is the same.

Figure 2: Activation of Secondary Carrier

When an uplink transmission link is established in primary or secondary carrier, the MS may start the CLPC operation and update its UL transmission power according to BS’ command. As mentioned, the short-term fading is frequency-selective; therefore, each carrier has to perform its own CLPC operation independently. The procedure is exemplified as Figure 3.

Primary carrier

Secondary carrier

Figure 3: example of multi-carrier power control

As for fast-switching scenario, MS stays in the secondary carrier most of the time and jumps back to primary carrier for DL broadcasting information while necessary; therefore, it is required to measure PL value and perform CLPC onto secondary carrier only. In this case, the multi-carrier power control is reduced to single-carrier one.

- Proposed Text for SDD

[In TGm SDD[1], ]

-------------------------------  Text Start  ----------------------------------------------------------------------------------

11. x Power Control

11.x.y multi-carrier power control

In the multi-carrier operation, MS firstly has to synchronize with the one of the primary carriers of a cell. During the synchronization procedure, the MS measures the downlink preamble, calculates the long-term fading and performs ranging with its serving cell. After that, it keeps calculating PL value and performs OLPC to compensate long-term fading by power control equation. Upon the activation of the second carrier, the MS performs ranging by using primary carrier’s current transmission power as initial value.

-------------------------------  Text End  ------------------------------------------------------------------------------------

[1] IEEE 802.16m-08/003r4, “The Draft IEEE 802.16m System Description Document”