| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Multi User MIMO HARQ in IEEE 802.16m |  |
| Date Submitted | 2008-09-12 |  |
| Source(s) | Bozo Cesar Joerg Schaepperle Andreas Rueegg Juergen Otterbach Alcatel-Lucent | Voice: +49 711 821 32266 E-mail: Bozo.Cesar@alcatel-lucent.de Joerg.Schaepperle@alcatel-lucent.de Andreas.Rueegg@alcatel-lucent.de Juergen.Otterbach@alcatel-lucent.de *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | SDD Session 56 Cleanup, Category MAC; in response to the TGm Call from Contributions and Comments 802.16m-08/033 for Session 57 |  |
| Abstract | Multi User MIMO HARQ in IEEE 802.16m |  |
| Purpose | To discuss and adopt the proposed text in the next revision of the 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Multi User MIMO HARQ in IEEE 802.16m

Bozo Cesar, Joerg Schaepperle, Andreas Rueegg, Juergen Otterbach

Alcatel-Lucent

# Introduction

As MIMO HARQ is in discussion in the PHY HARQ Rapporteur Group, also a MIMO HARQ chapter is required in the MAC HARQ part of the SDD.

In case of MU MIMO in the present SDD up to four data streams can be transmitted in the same resource units (RU).

With conventional SDMA techniques the bursts usually are at least approximately spatially orthogonal.

For Multi User MIMO special HARQ mechanism will be supported if required.

In case of MU MIMO using non-orthogonal superposition technique, two non orthogonal data streams, in the following called burst_1 and burst_2, are combined to a superposition burst. With this technique up to 8 data streams can be supported. In figure 1 only one superposition burst, containing burst_1 and burst_2, is transmitted – only the pair is relevant for the description of the proposed HARQ mechanism. Up to four superposition bursts can be present in the same resource unit.

For Multi User MIMO using non-orthogonal superposition technique HARQ mechanism may be supported to handle the interaction of the decoding processes of the burst pairs.

Figure 1: Example DL transmission

# DL HARQ transmission

### Initial transmission of the DL HARQ bursts

Two DL HARQ bursts, burst_1 and burst_2 occupying the same radio resource in the DL frame arrive at the MS in case of MU MIMO using non-orthogonal superposition technique. See figure 1 for example.

Each burst is characterized by a burst profile. The burst profile modulation, coding and transmission power.

One of the burst may be transmitted with higher mean power (e.g. burst_2), the other with lower mean power (e.g. burst_1).

MS2, which is located far away from the BS, will be able to decode burst_2 only. MS2 must generate the HARQ ACK/NACK for burst_2.

Even if the MS2 knows that there are two bursts located at the same resource, it shall not try to decode burst_1, and shall not transmit ACK/NACK for burst_1.

MS1, which is located close to the BS, must decode burst_2 to be able to decode burst_1.

For example a two step operation can be performed:

Step 1) MS1 has to decode burst_2 first, because it needs the results for step 2,

Step 2) MS1 has then to decode burst_1 and shall transmit ACK/NACK for burst_1. MS1 must transmit ACK/NACK for burst_1 even if it can not decode any of the bursts.

If MS1 shall also transmit ACK/NACK for burst_2 is FFS.

Summary:

The MS must be informed by signaling (DL control) which of the two bursts (burst_2 or burst_1) is addressed to it.

In addition it has to be clarified which of the two bursts it has to transmit ACK/NACK – because the MS must transmit ACK/NACK for the desired burst even if it can not decode any of them.

| Options: MSx generates ACK/NACK | MS1 | MS2 |
| --- | --- | --- |
| for Burst_2 | FFS | yes |
| for Burst_1 | yes | no |

### Retransmission of the DL HARQ bursts

Retransmission in case of synchronous HARQ scheme or in case of asynchronous HARQ scheme can be performed differently.

### Retransmission in case of synchronous non adaptive HARQ scheme

In case of synchronous HARQ scheme no signaling is assumed for the retransmission, i.e. no new information (new CID, new MCS, …) is available for the retransmission.

|  | MSx ACK/NACK response to initial transmission | retransmission |
| --- | --- | --- |
| Burst_2 Burst_1 | MS2 NACK MS1 NACK | Retransmit same combination of burst_1 and burst_2 |
| Burst_2 Burst_1 | MS2 NACK MS1 ACK | Retransmit burst_2 only. |
| Burst_2 Burst_1 | MS2 ACK MS1 NACK | Retransmit burst_1 only. |

### Retransmission in case of asynchronous HARQ scheme

In case of asynchronous HARQ scheme signaling is assumed for the retransmission, i.e. new burst combinations and new MCS can be signaled if necessary.

|  | MSx ACK/NACK response to initial transmission | retransmission |
| --- | --- | --- |
| Burst_2 Burst_1 | MS2 NACK MS1 NACK | burst_1 and burst_2 have to be retransmitted either together or in combination with other bursts |
| Burst_2 Burst_1 | MS2 NACK MS1 ACK | Retransmit burst_2 either alone or in combination with a new burst_1.. |
| Burst_2 Burst_1 | MS2 ACK MS1 NACK | Retransmit burst_1 either alone or in combination with a new burst_2. |

### Multi User MIMO HARQ signaling

For multi user MIMO using non-orthogonal superposition technique, the information may be provided to clearly identify the superimposed data streams and the sequence in which data stream 1 (burst_1) and data stream 2 (burst_2), which both are transported in the same radio resource, have to be decoded by the MS.

For multi user MIMO using non-orthogonal superposition technique the information may be provided to clearly assign the ACK/NACK channels for the superimposed data stream 1 (burst_1) and data stream 2 (burst_2), whereby both data streams are transported in the same radio resource.

### HARQ feedback

For the allocation of several HARQ feedback channels in the UL frame there are two options:

Option 1: allocate one resource unit per HARQ feedback channel

Option 2: share one resource unit for multiple HARQ feedback channel by orthogonal or non orthogonal superposition to minimize the allocated resources for HARQ signaling.

If MS1 shall also transmit ACK/NACK for burst_2 is FFS.

Option 1: MS1 shall also transmit ACK/NACK for burst_2.

Option 1A:Two resources in the UL frame are allocated for the ACK/NACK, one for burst_1 and one for burst_2.

| Resources in UL frame | MS1 generates | MS2 generates |
| --- | --- | --- |
| Resource 1 | ACK/NACK for burst_1 |  |
| Resource 2 | ACK/NACK for burst_2 | ACK/NACK for burst_2 |

Resource 2 is used to superimpose the ACK/NACK signals of two MSes

Option 1B:Three resources in the UL frame are allocated for the ACK/NACK as shown in the following table. Each resource is used to transmit one ACK/NACK only.

| Resources in UL frame | MS1 generates | MS2 generates |
| --- | --- | --- |
| Resource 1 | ACK/NACK for burst_1 |  |
| Resource 2 |  | ACK/NACK for burst_2 |
| Resource 3 | ACK/NACK for burst_2 |  |

Option 2: MS1 shall not transmit ACK/NACK for burst_2.

Option 2A:Two resources in the UL frame are allocated for the ACK/NACK, one for burst_1 and one for burst_2.

| Resources in UL frame | MS1 generates | MS2 generates |
| --- | --- | --- |
| Resource 1 | ACK/NACK for burst_1 |  |
| Resource 2 |  | ACK/NACK for burst_2 |

Resource 2 is used to superimpose the ACK/NACK signals of two MSes

Option 2B:Only one resource in the UL frame is allocated for the ACK/NACK.

The Resource is used to superimpose the ACK/NACK signals of two MSes

| Resources in UL frame | MS1 generates | MS2 generates |
| --- | --- | --- |
| Resource | ACK/NACK for burst_1 | ACK/NACK for burst_2 |

Selection of one of the options is FFS.

## Uplink HARQ transmission

### Initial transmission of the UL HARQ bursts

Two UL HARQ bursts, burst_1 and burst_2 occupying the same radio resource in the UL frame arrive at the BS in case of MU MIMO using non-orthogonal superposition technique. See figure 2 for example.

Figure 2: Example UL transmission

One of the burst may be received by the BS with higher power (e.g. burst_1), the other with lower power (e.g. burst_2).

BS must be able to decode burst_1 and burst_2, and must generate the HARQ ACK/NACK for burst_1 and burst_2.

### Retransmission of the UL HARQ bursts

Retransmission in case of synchronous HARQ scheme or in case of asynchronous HARQ scheme can be performed differently.

### Retransmission in case of synchronous non adaptive HARQ scheme

In case of synchronous HARQ scheme no signaling is assumed for the retransmission, i.e. no new information (new CID, new MCS, …) is available for the retransmission.

|  | MSx ACK/NACK response to initial transmission | retransmission |
| --- | --- | --- |
| Burst_1 Burst_2 | MS1 NACK MS2 NACK | Retransmit same combination of burst_1 and burst_2 |
| Burst_1 Burst_2 | MS1 NACK ( MS2 ACK ) | Can not occur because burst_2 can not be decoded without a valid burst_1. |
| Burst_1 Burst_2 | MS1 ACK MS2 NACK | Retransmit burst_2 only.. |

### Retransmission in case of asynchronous HARQ scheme

In case of asynchronous HARQ scheme signaling is assumed for the retransmission, i.e. new burst combinations and new MCS can be signaled if necessary.

|  | MSx ACK/NACK response to initial transmission | retransmission |
| --- | --- | --- |
| Burst_1 Burst_2 | MS1 NACK MS2 NACK | burst_1 and burst_2 have to be retransmitted either together or in combination with other bursts |
| Burst_1 Burst_2 | MS1 NACK ( MS2 ACK ) | Can not occur because burst_2 can not be decoded without a valid burst_1. |
| Burst_1 Burst_2 | MS1 ACK MS2 NACK | Retransmit burst_2 either alone or in combination with a new burst_1. |

# Proposed text for SDD for Multi User MIMO HARQ

Insert the following text into [IEEE 802.16m-08/003r4]

-----------------------------------------------------------Start of the Text---------------------------------------------------------

### 10.2.4 Multi User MIMO HARQ

For multi user MIMO special HARQ mechanism will be supported if required.

For multi user MIMO using non-orthogonal superposition technique, special HARQ mechanism may be supported to handle the interactions of the decoding processes of the two bursts (burst_1 and burst_2) in a pair.

In this case mechanisms for the following combinations will be supported:

Burst_1: NACK, 	Burst_2: NACK

Burst_1: NACK, 	Burst_2: ACK

Burst_1: ACK,   	Burst_2: NACK

------------------------------------------------------------End of the Text---------------------------------------------------------

# Proposed text for SDD for Multi User MIMO HARQ signaling

Insert the following text into [IEEE 802.16m-08/003r4]

-----------------------------------------------------------Start of the Text---------------------------------------------------------

### 10.2.4.1 Multi User MIMO HARQ signaling

For multi user MIMO technique (e.g. using non-orthogonal superposition), downlink control and signaling information have to be provided to MS to identify the superimposed data streams (both transported in the same radio resource) and the decoding sequence for these data streams.

Control and signaling information have to be provided to MS to clearly assign the ACK/NACK channels for the superimposed downlink data streams. E.g. the message ordering in the signaling path (e.g. MAP sequence), which is used to define the data streams, will define the ordering of the HARQ ACK/NACK channels.

------------------------------------------------------------End of the Text---------------------------------------------------------

# Proposed text for SDD for HARQ feedback

Insert the following text into [IEEE 802.16m-08/003r4]

-----------------------------------------------------------Start of the Text---------------------------------------------------------

### 11.x.1.2.6 HARQ feedback

Share one resource unit for multiple HARQ feedback channels by orthogonal or non orthogonal superposition to minimize the allocated resources for HARQ signaling is FFS.

------------------------------------------------------------End of the Text---------------------------------------------------------