| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Prioritized QoS Framework – Connection Establishment and Maintenance for IEEE 802.16m |  |
| Date Submitted | 2008-09-05 |  |
| Source(s) | An Nguyen DHS/NCS D. J. Shyy MITRE | An.P.Nguyen@dhs.gov djshyy@mitre.org |
| Re: | MAC: Connection Establishment and Maintenance In response to IEEE 802.16m-08/033, Call for Contributions and Comments on Project 802.16m System Description Document (SDD) (2008-08-01) |  |
| Abstract | The document describes prioritized QoS framework in connection establishment and maintenance used to support the priority access in 802.16m |  |
| Purpose | Request consideration of prioritized QoS framework in connection establishment and maintenance contained herein for the 802.16m SDD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Prioritized QoS Framework in Connection Establishment and Maintenance for IEEE 802.16m

# Introduction

Priority access is one of important requirements of Emergency Telecommunications Service (ETS) [1]. ETS is a telecommunications service to facilitate emergency recovery operations for restoring the community infrastructure and for returning the population to normal living conditions after serious disasters and events, such as floods, earthquakes, hurricanes, and terrorist attacks. The ETS will be provided through shared resources from the public telecommunications infrastructure, which includes wireline, wireless, satellite, broadband cable, and any hybrid networks. ETS traffic needs to access, traverse, and egress these networks. Voice, video and data services are supported by ETS.

IEEE 802.16m has requirements on priority for the government and public safety.  In Section 5.8 of IEEE 802.16m-07/002r4, it said “IEEE 802.16m shall be able to support public safety first responders, military and emergency services such as call-prioritization, preemption, push-to-talk”.  Priority access is also a SPWG requirement in Mobile WiMAX [2].  In Release 1.5, “R-[193] The WIMAX network SHALL provide high priority for special circumstances such as emergency services and deadlock situations. Such high priority SHALL be protected with appropriate additional levels of authentication.”  Priority access is required for National Security and Emergency Preparedness (NS/EP) users.

In this contribution, we propose prioritized QoS framework in connection establishment and maintenance to be considered by 802.16m.

  - Purpose

The major purpose of this document is to propose prioritized QoS framework in connection establishment and maintenance to support the priority access in 802.16m.  All the identified prioritized QoS framework in connection establishment and maintenance is our recommendations to be included in 802.16m SDD.

  - Document Organization

Section 2 presents prioritized QoS framework in connection establishment and maintenance to support priority access to be considered by 802.16m

Section 3 presents more detailed ETS requirements, especially in the area of priority access.

Section 4 is our SDD text proposal.

# 802.16m Prioritized QoS Framework in Connection Establishment and Maintenance to Support Priority Access

To support priority access in 802.16m, the following procedures are required

- User subscription

- Service priority invocation (service priority indication)

- User authentication (user identity; verifies the service indication is from priority user)

- Service authorization (specifies the service privileges of priority user)

- Service priority treatment (service differentiation)

This document discusses the techniques used for service priority treatment.  To provide service priority treatment using 802.16m networks, the following techniques are required:

- Prioritized network entry (initial ranging)

- Prioritized (initial) service flow setup and admission

- Prioritized session setup and maintenance

- Prioritized bandwidth request for admitted service flow

- Prioritized handover

The congestion points for radio access occur at contention-based and allocation-based transmission (shown in Figure 1).  The contention-based transmission is mainly used for initial ranging and bandwidth request.  The allocation-based is to allocate OFDMA capacity.  This contribution discusses the prioritized QoS framework to support priority access bandwidth request and allocation-based transmission.

Figure 1: Radio Access Congestion Points

.

  - Protocol Functions for Priority Access

The protocol functions for priority access are shown in Figure 2. The priority access capability is supported by different logic entities in the MAC Common Part Sublayer. Network Entry Management entity is responsible for priority access to the contention resource such as initial ranging. Connection Management entity is responsible for service flow setup between BS and MS. And scheduler is for dynamic radio bandwidth allocation. All these entities shall coordinate with Radio Resource Management entity about the resource availability. Local Radio Resource stores local radio bandwidth utilization. Radio Resource Management is the entity to access and update the real-time radio resource information.

Figure 2: 802.16m Protocol Functions for Priority Access

The 802.16e has the following network entry steps:

- Scanning

- Obtaining parameters

- Initial ranging

- Exchanging capabilities

- Authentication

- Registration

- IP connectivity

- Creating transport connection (Adding service, service flow admission)

- Bandwidth Request for admitted service flow

The priority treatments for initial ranging, creating transport connection and bandwidth request steps are discussed in the following sections.

  - Prioritized Contention Resolution

2.2.1 Introduction

This subsection was presented in Reference 3, and will not be repeated here.

2.2.2 Simulation Configurations

This subsection was presented in Reference 3, and will not be repeated here.

2.2.3 Guidelines

a. Assign one or more dedicated bandwidth request slot for regular user and one dedicated bandwidth request slot for priority user for a larger channel bandwidth (e.g. 10 MHz).  Also assign a set of dedicated codes for regular user and another dedicated set for priority user.

b. Assign one or more dedicated bandwidth request frames for regular user and one dedicated bandwidth request frame for priority user for a smaller channel bandwidth (e.g. 5 MHz).

  - Dedicated Codes for Bandwidth Request and Handover Ranging

- References 4 and 5 proposed to use dedicated codes for bandwidth request or handover ranging to speed up the process time. For example, the dedicated code may imply additional information such as capacity request size; as a result, one round trip delay time can be reduced during bandwidth request process.  The dedicated codes technique is proposed to be one of the techniques in the Prioritized QoS Framework.

  - Prioritized Service Flow Admission

The priority access indicator is passed to the network at the Adding Service step in network entry or network re-entry. Priority treatment is done first by mapping the priority access indicator to the proper pre-provisioned Service Flow ID (SFID) or dynamic Service Flow if supported. Out of the five 802.16e QoS classes, the following four are considered appropriate for priority access services: UGS, rtps, Ertps, and nrtps. Within each QoS class, authorized user can get non-preempted preferential treatment for their traffic in the network. One example is to perform priority treatment via queuing and scheduling; authorized user has their request in the front of the queue. Within the authorized users of the same QoS class, there are multiple sub-priority levels. The sub-priority level and the request arrival time determine the queue position.

Figure 7: Prioritized Service Flow Admission

Figure 7 shows the architecture of the prioritized service flow admission.  Service flow Authorization Module admits service flow request based on local resource availability, QoS parameters, request priority and request arrival time.  If the resource is currently unavailable, the prioritized request is queued based on its QoS type and priority.  If the resource is unavailable, a non-prioritized request will be discarded.  Once the resource is available, the queued requests are processed. The queuing and scheduling mechanism of Priority Queue is implementation specific. For admitted service flow, the priority is carried on for dynamic radio bandwidth allocation.

  - Air Interface Changes to Support Prioritized Service Flow Admission

We proposed to modify the existing 802.16e fields

- Traffic Priority in Service Flow TLV

- Confirmation code in DSx-RSP and DSx-ACK

The reasons for these changes are:

- To support the prioritized service flow, the priority information shall be carried in the service flow management messages and processed by Connection Management entity

- A mechanism to allow DSx to differentiate the following two scenarios:

  - Priority services requested by authorized users

  - Regular services requested by authorized users

- The timer and reply code are needed for preempted or delayed admission handling

The affected Service Flow management messages are:

- Addition: DSA-REQ, DSA-RSP, DSA-ACK

- Change: DSC-REQ, DSC-RSP, DSC-ACK

- Deletion: DSD-REQ, DSD-RSP

We also propose to add new timers

- Add a new timer for MS to handle request queuing

- Add a new timer for BS to handle request queuing

### Modify existing 802.16e fields

The following 802.16e fields are proposed to be modified:

- Traffic Priority in Service Flow TLV

- Confirmation code in DSx-RSP and DSx-ACK

The original traffic priority format is the 802.16e is listed below.

Our proposed 802.16m traffic priority format is listed below.

Two bits are reused in the 802.16e Traffic Priority field. Service flow admission bit can serve as priority access indicator. If it is set to prioritized, the service flow request will be queued even though the current radio resource availability doesn’t allow the admission of this service flow. If it is set to non-prioritized, the request will be discarded if radio resource is not available at request time. The pre-emption bit is used only when service flow admission bit is set to be prioritized. The queuing treatment for prioritized service flow request can be precedence only or pre-emption.

We propose to add new confirmation codes for prioritized request status.  The new confirmations codes are reject-temporary/queued and reject-permanent/queue-timeout.

Above changes will affect following Service Flow management messages

- Addition: DSA-REQ, DSA-RSP, DSA-ACK

- Change: DSC-REQ, DSC-RSP, DSC-ACK

- Deletion: DSD-REQ, DSD-RSP

### Add new timers

In order to handle queued service flow request, timers shall be added to MS and BS to prevent deadlock.

  - Prioritized Radio Resource Allocation

The scheduler is responsible for dynamic radio resource allocation. Within each QoS class, admitted/active service flows have different priorities for their traffic.  The queuing and scheduling in the scheduler would be implementation specific, but their behavior should be measurable.  It is envisioned that a priority resource allocation technique will be “recommended” for each of the QoS classes.  The “recommended” priority resource allocation technique will be presented in future sessions.

# Priority Access Requirements

  - Priority Access Service

ETS is a super set of all emergency services for an authorized select group of users. Wireless Priority Service (WPS) is one of ETS that is on wireless networks. The Wireless Priority Service should provide end-to-end treatment across the network (e.g., access, transport, and egress) so that higher communication session completion rates, as compared with public communication session completion rates, would be successfully achieved. Communication sessions in this context include voice, multimedia, and data.” [1]

DHS/NCS has worked with service providers as well as equipment vendors to get WPS implemented [6]:

- “Priority access service is intended to facilitate emergency response and recovery operations in response to natural and man-made disasters and events, such as floods, earthquakes, hurricanes, and terrorist attacks. Priority access service is also intended to support both national and international emergency communications.”

- “Priority access service is based upon, and complies with, the FCC Second Report and Order (R&O) 00-242 (Wireless Telecommunications (WT) Docket No. 96-86).

- “Priority access service is intended to allow qualified and authorized National Security and Emergency Preparedness (NS/EP) users to obtain priority access to radio traffic channels and core network resources during situations when Commercial Mobile Radio Service (CMRS) provider network congestion is blocking call attempts”.

WiMAX network should provide the Priority Service too. The end-to-end priority service needs to cover radio accessing, data transportation and interconnection with other networks. Priority access service only focuses on the radio accessing part of the priority service. The NS/EP authorized user shall have higher priority level to obtain radio resource to access WiMAX network to facilitate emergency recovery.

End-to-end ETS is across different telecommunication networks and different countries. There are ETS standardization activities around the globe. The following standards bodies have ETS activities:

- ITU (International Telecommunication Union)

- IETF (Internet Engineering Task Force)

- ETSI (European Telecommunications Standards Institute)

- ATIS (Alliance for Telecommunications Industry Solution)

- TIA (Telecommunications Industry Association)

- 3GPP (3rd Generation Partnership Project)

- 3GPP2 (3rd Generation Partnership Project 2)

  - Service Scenarios

We address two types of calling scenarios. Scenarios are presented to develop the requirements and technical solutions for priority access.

- Scenario 1: Mobile originating call.  It is for authorized user to initiate a voice, data or video call to any party using 802.16m technology.

- Scenario 2: Mobile terminating call.  It is for authorized user to call another authorized user reachable using 802.16m technology.

  - Requirements

The associated requirements for priority access service [1] include:

- Ubiquitous Services:

  - Able to make a call/session in times of disaster, national emergency, or for executive/governmental communications relating to National Security / Emergency Preparedness (NS/EP).

  - It applies to voice, video, and data services

- Availability:

  - It is available to NS/EP users at all times.

- QoS

  - End-to-end QoS should be provided based on 802.16e QoS classes and parameters

- Security

  - “Networks must have protection against corruption and intrusion such as unauthorized access, control and traffic”

- Priority treatment

  - “ETS communications should be provided preferential end-to-end treatment so that higher communication session completion rates, as compared with public communication session completion rates, would be successfully achieved”

# Text Proposal

[---------------------------------------------------Start of Text Proposal--------------------------------------------------]

Section 10 Medium Access Control Sub-Layer

10.x Connection Establishment and Maintenance

10.x.1 Bandwidth Request

Assign one or more dedicated bandwidth request slot for regular user and one dedicated bandwidth request slot for priority user for a larger channel bandwidth (e.g. 10 MHz).  Also assign a set of dedicated codes for regular user bandwidth request and another dedicated set for priority user bandwidth request.

Optionally, assign one or more dedicated bandwidth request frames for regular user and one dedicated bandwidth request frame for priority user for a smaller channel bandwidth (e.g. 5 MHz).

10.x.2 Service Flow Admission

The following 802.16e fields are proposed to be modified:

- Traffic Priority in Service Flow TLV

- Confirmation code in DSx-RSP and DSx-ACK

The original traffic priority format is the 802.16e is listed below.

Our proposed 802.16m traffic priority format is listed below.

Two bits are reused in the 802.16e Traffic Priority field. Service flow admission bit can serve as priority access indicator. If it is set to prioritized, the service flow request will be queued even though the current radio resource availability doesn’t allow the admission of this service flow. If it is set to non-prioritized, the request will be discarded if radio resource is not available at request time. The pre-emption bit is used only when service flow admission bit is set to be prioritized. The queuing treatment for prioritized service flow request can be precedence only or pre-emption.

We propose to add new confirmation codes for prioritized request status.  The new confirmations codes are reject-temporary/queued and reject-permanent/queue-timeout.

Above changes will affect following Service Flow management messages

- Addition: DSA-REQ, DSA-RSP, DSA-ACK

- Change: DSC-REQ, DSC-RSP, DSC-ACK

- Deletion: DSD-REQ, DSD-RSP

In order to handle queued service flow request, timers shall be added to MS and BS to prevent deadlock.

[---------------------------------------------------End of Text Proposal--------------------------------------------------]

## 5. References

[1] “Overview of Standards in Support of Emergency Telecommunication Service (ETS),” ATIS-pp-0100009, November 2006.

[2]”Recommendations and Requirements for Networks based on WiMAX Forum Certified Products,” WiMAX Forum Release 1.5, Jan. 2007

[3] An Nguyen and D. J. Shyy, “Prioritized QoS Framework in Network Entry”, C802.16m-08/1046r1, September 2008.

[4] “Proposed Text on BW-REQ Channel for Baseline Content of Uplink Control Structure for IEEE 802.16m SDD based on Contribution C802.16m-08/725”, IEEE C802.16m-UL_ctrl-08/866r3

[5] “Handover using Exclusive HO Ranging Channel”, IEEE C802.16m-08/714

[6] Wireless Priority Service (WPS) Industry Requirements for UMTS – Phase 1 – Redirection to GSM,” DHS/NCS IR Document, Version 0.3, April 2005

[7] Quality of Service Attributes for Diameter,” draft-ietf-dime-qos-attributes-02.txt

[8] “Quality of Service Parameters for Usage with the AAA Framework,” draft-ietf-dime-qos-parameters-01.txt

[9] “RSVP Extensions for Emergency Services,” draft-ietf-tsvwg-emergency-rsvp-03.txt

[10] “QoS NSLP QSPEC Template,” draft-ietf-nsis-qspec-18.txt

[11] Report ITU-R.[PPDR]: “Radiocommunication objectives and requirements for public protection and disaster relief (PPDR)”

[12] Recommendation H.460.4 – Call priority designation for H.323 calls

[13] Draft recommendation H.SETS – security for ETS (H.235)

[14] E.106 – Description of an international emergency preference scheme (IEPS)

[15] Draft Recommendation U.roec – network requirements and capabilities to support ETS

[16] RFC3523: “Internet Emergency Preparedness (IEPREP) Telephony Topology Terminology”

[17] TETRA: “Mobile narrowband and wideband communications for public safety applications”

[18] TR41.4 TSB146: “Telecommunications – IP Telephony Infrastructure – IP Telephony Support for Emergency Calling Service)

[19] 3GPP TS 22.153, “Technical Specification Group Services and System Aspects; Multimedia priority service”, Release 8.

[20] 3GPP S.R0017-0, “Multimedia Priority Service (MMPS) for MMD-based Networks – Stage 1 Requirements”