| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Device Classes for IEEE 802.16m Mobile Stations |  |
| Date Submitted | 2008-09-11 |  |
| Source(s) | Yuval Lomnitz Tom Harel Sassan Ahmadi Intel Corporation | Yuval.Lomnitz@intel.com Tom.Harel@intel.com Sassan.ahmadi@intel.com |
| Re: | PHY: Text In response to IEEE 802.16m-08/003r4 Call for contributions on IEEE 802.16m PHY |  |
| Abstract | Proposal for IEEE 802.16m MS classes |  |
| Purpose | This contribution defines mobile station device classes for IEEE 802.16m Discussion and Approval (to be included in section 11.10 of IEEE 802.16m SDD) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Device Classes for IEEE 802.16m Mobile Stations

Yuval Lomnitz, Tom Harel, Sassan Ahmadi

Intel Corporation

# Introduction

In IEEE 802.16e and Mobile WiMAX Release 1.0 mobile system profile (i.e., the legacy system) there is no explicit limitation on the maximum throughput supported by the MS. As a result, the MS must support the maximum throughput determined by the supported bandwidth, modulation, MIMO mode, etc, which is in some cases more than the throughput actually required. In IEEE 802.16m higher bandwidths, multi-carrier support, and various MIMO configurations are considered, which adds to the severity of the issue. This contribution proposes to define explicit limitations on the maximum processing capabilities required from the MS, and to keep a proportion between the processing requirements at different levels (data bits in subframe, H-ARQ memory, etc), in order to allow more cost effective hardware solutions.

In addition to maximum throughput we propose to define the minimum bandwidth, minimum antenna configuration and multicarrier support.

# Proposed IEEE 802.16m Device Classes

### Scaling between Mobile WiMAX Release 1.0 and IEEE 802.16m

A Mobile WiMAX Release 1.0 device is required to have 2 receive antennas and operate at maximum BW of 10 MHz. A Mobile 802.16m device may have up to 4-8 antennas and operate at BW of 20MHz, therefore if all requirements scale accordingly the processing requirements are between x4 and x8. Since the processing requirements are high to start with, and require parallelization, an increase in the processing requirements directly translates to a proportional increase in the HW (silicon) size which affects cost as well as power consumption.

This necessitates upper limits on the processing requirements, which will enable on the one hand to have a spectrally efficient network and on the other hand to build cost-efficient devices.

In principle, processing requirements are decoupled from BW support requirements: a low-throughput device may support large BW (which is required for network efficiency) with small processing requirements. However to minimize the number of different classes we suggest that a class will define simultaneously the processing, BW and minimal antenna configuration requirements.

### The imbalance in Mobile WiMAX Release 1.0

In Mobile WiMAX Release 1.0 some limitation on the throughput exists implicitly because of H-ARQ buffer size, however this limitation doesn’t necessarily help reduce processing requirements. For example, a H-ARQ CAT 2 device is required to buffer 128 Kbits, and assuming coding rate 5/6 and RTT=3 frames (15ms), the H-ARQ traffic (without errors and buffer overflows) is 128Kbit/15ms5/6=7 Mbps. However, the maximum DL data rate in Mobile WiMAX Release 1.0 profile (64QAM 5/6, 2x2 spatial multiplexing, PUSC, 10MHz) is 10 bit/subcarrier  720 subcarriers/symbol / 102.85s = 70 Mbps. When operating in non-HARQ mode, the device has to be able to decode information at up to 70 Mbps (burst rate), while the actual throughput is approximately 7 Mbps. The conclusion from this is that some coarse alignment between the requirements is needed.

### Analysis of factors affecting complexity

The following table summarizes the factors that affect the processing at the MS.

| Factor | Main effect on complexity | Note |
| --- | --- | --- |
| Number of bits stored for H-ARQ combining (H-ARQ buffer size) | DL: memory (~1B/bit) UL: memory (1 bit/bit) | DL stores metrics, UL stores bits |
| Number of data bits to decode per symbol/subframe | DL: Channel decoder (e.g. CTC) complexity | Channel decoder complexity is significant in Mobile WiMAX Release 1.0. |
| Peak throughput supported by the MAC per frame | UL and DL complexity (encryption/decryption, QoS, packing, etc) Depends also on average packet size |  |
| Maximum channel bandwidth supported | RF filtering requirements, ADC/DAC sampling rates, digital time-domain processing and FFT |  |
| Multi carrier support – adjacent carriers | May require (pending on specific details): More severe PA and filtration requirements stemming from reduced guard frequency; multiple FFT; additional filtration to separate carriers; etc |  |
| Multi carrier support – disjoint carriers | Separate RF chains per carrier (LO, mixer, filters) |  |

### Considerations for minimum bandwidth

We recommend setting the minimum bandwidth supported by MS to 20 MHz (i.e., the maximum contiguous bandwidth considered for 802.16m) for all device classes. The alternative is that some MS supports minimum bandwidth of 5 MHz, and the system enables 5 MHz device in 20MHz carrier, however this will create additional complexity in the system (and hence also in the devices). The cost implication of supporting larger bandwidth (if no larger processing is required) is relatively small.

### Considerations for antenna configurations

Various antenna configurations can be considered: 2 RX/1 TX (baseline), 2 RX / 2 TX, 3 RX / 3 TX, 4 RX / 2 TX, 4 RX / 4 TX, 8 RX / 4 TX and many more. We cannot, and need not, define each configuration as a class. Therefore we considered three minimum configurations: the 2 RX / 1 TX which is the minimum (or baseline antenna configuration)  required for 802.16m, the 4 RX / 2 TX which defined as the target configuration specified by the 802.16m SRD, and the 3 RX / 1 TX which is an efficient compromise for mobile devices. Thus, classes define the minimum antenna configuration, and any other antenna configuration that expands the minimum configuration is considered within the class.

### Considerations for determining the processing related factors

Most of these factors can be determined by using some proportion of the throughput supported by the MS. It’s important to stress that this throughput is just a key and doesn’t define the actual throughput that the MS can support in different scenarios, and the assumptions are not necessarily correct in each system, they are just used as a way to determine the ratio between different resources. The idea behind these factors is that the MS should be able to support the designated throughput in a reasonably good (but not extreme) scenario.

We suggest using the following key, with respect to the throughput Thp:

Number of bits stored for H-ARQ combining (H-ARQ buffer size): assume no-error and no-overflow transmission with RTT=2 frames (10 ms) and code rate ½ then Thp = Buffer / 10 ms  ½, thus

Buffer (bits) = 20ms  Thp

Number of data bits to decode per symbol/subframe: Assume traffic condensation of 1:4 (i.e. a user that needs maximum throughput has traffic on at least 2 sub-frames per frame on the average), then

Bits Per symbol = Thp  4  SymbolDuration = 400s  Thp

Number of bits supported by the MAC per frame: assume condensation of 1:4 (i.e. at least 1 forthfourth of the frames are used for a maximum throughput user), therefore

Bits = Thp  5ms  4 = 20ms  Thp

In all factors we didn’t separate between UL and DL. The reason is that DL processing incurs more complexity and the requirements on DL throughput are usually higher, so the UL processing is not the limiting factor, and for simplicity the requirements can be symmetrical.

Note that for 2 RX over 20Mhz with code rate of 1 and with 11% pilot overhead the instantaneous throughput is 1728(16/18)62/102.8 = 180 Mbps, so according to our assumption of 1:4 compression, the maximum throughput key is 45Mbps.

### Minimum performance requirements

802.16m devices are required to be compliant with Mobile WiMAX Release 1.0 (and to perform better).  Therefore, they cannot benefit from requirements that are lower than the legacy system (about 70 Mbps peak throughput).

# SDD text proposal

[Add the following text at the end of section 11.10]

- - - - - - - - - - - TEXT START - - - - - - - - -

11.10 Device Classes

A number of device classes are defined in order to reduce the cost and implementation complexity of mobile stations and to allow a range of device capabilities (from a baseline/basic class to an advanced/premium class) suited to the market needs and usage models. All device classes in the following table shall support a minimum bandwidth of 20 MHz and the support of eMBS is optional.

| Class | Average throughput key* | Max instantaneous DL throughput [Mbps] | H-ARQ buffer size [Kbit] | Max bits per symbol | Max data in frame [K byte] | Minimum antenna configuration | Highest Modulation | Multi-carrier Support |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class 1 | 20 | 80 | 400 | 8244 | 50 | 1TX 2RX | DL: 64QAM UL: 16QAM | No |
| Class 2 | Unlimited | Unlimited | PeakThp 10ms | Unlimited | Unlimited | 1TX 2RX | DL: 64QAM UL: 64QAM | Any |
| Class 3 | 40 | 160 | 800 | 16452 | 100 | 1TX 3RX | DL: 64QAM UL: 64QAM | No |
| Class 4 | Unlimited | Unlimited | PeakThp 10ms | Unlimited | Unlimited | 1TX 3RX | DL: 64QAM UL: 64QAM | Any |
| Class 5 | 50 | 200 | 1000 | 20592 | 125 | 2TX 4RX | DL: 64QAM UL: 64QAM | Yes |
| Class 6 | Unlimited | Unlimited | PeakThp 10ms | Unlimited | Unlimited | 2TX 4RX | DL: 64QAM UL: 64QAM | Any |
| Class 7 | 100 | 400 | 2000 | 41148 | 250 | 4TX 4RX | DL: 64QAM UL: 64QAM | Yes |
| Class 8 | Unlimited | Unlimited | PeakThp 10ms | Unlimited | Unlimited | 4TX 4RX | DL: 64QAM UL: 64QAM | Any |
| Factors |  | 4 | 20ms | 4102.8s | 20ms |  |  |  |

Where:

- Average throughput key is an informative parameter used to calculate the maximum values according to the factors (informative) in the last row. It is not a maximum throughput.

- H-ARQ buffer size: number of coded bits (RX) or transmitted bits (TX) stored for H-ARQ combining

- Max bits per symbol: max number of information bits to decode/encode per symbol (assuming symbol duration is 102.8 μs)

- Max bits per frame: maximum number of bits contained in all PDUs (in DL and UL separately).

- - - - - - - - - - - TEXT END - - - - - - - - -

Another limitation that cannot help reduce PHY processing is the “Maximum amount of MAC level data per DL/UL frame”, since the PHY processing can include large amount of data for other destinations.