| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Paging Interval Design & Mobile Battery Level |  |
| Date Submitted | 2008-09-05 |  |
| Source(s) | John Harris, Stavros Tzavidas, Hua Xu Motorola | Voice: [+1 847-632-4042] , [+1 847-632-4313], [+1 847-632-2176] E-mail:[John.Harris@Motorola.com] [stavros.tzavidas@motorola.com] [Hua.Xu@motorola.com] *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | MAC: Idle Mode; in response to the TGm Call for Contributions and Comments 802.16m-08/003r4 for Session 57 |  |
| Abstract | Proposal for IEEE 802.16m Paging Interval Design/Mobile Battery Level Reporting, and Mobile Thermal Status Processing Limitation |  |
| Purpose | Discussion and Approval |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Paging Interval Design/Mobile Battery Level, & mobile thermal processing limitation

John M. Harris, Stavros Tzavidas, Hua Xu

# Introduction and Background

This contribution addresses situations where the system performance can be enhanced by better adapting the behavior of the system to the mobiles current operating conditions.

# Battery Life and Paging Interval

In idle mode there is a trade-off between the paging interval length and the mobile battery life.  If the paging interval is short, then this results in less latency, and possibly less paging load.

For example, when mobile is plugged into a fixed power source a shorter paging interval is appropriate.  the shorter paging reduces latency when reaching that mobile, and reduces paging load when paging that mobile.

This latency reduction follows from the reduce delay when the network is paging the mobile. This reduced paging interval also enables the system to use an iterative paging technique where the system may attempt to reduce the number of cells which must be paged by attempting to guess which cell the mobile is in and only initially paging a subset of the cells in the paging group.  This iterative paging approach of can result in additional delay when the mobile station is not in the initial subset which is paged if a shorter paging interval was not used. However, by using this iterative approach and reducing the paging interval the overall delay can be reduced while also decreasing the number of cells which must be paged when the network attempts to reach that mobile.

On the other hand, a short paging interval will also result in a shorter battery life. When the mobile’s battery life is particularly low in the mobile is not plugged into a fixed power source -- the battery life becomes a relatively important topic.  In this situation is more appropriate to have a longer paging interval.

# Proposed Paging Design Considerations

This contribution proposes that the paging interval may be adapted to the mobile's battery situation.  This can be achieved through the use of Mac management messages, which feedback the battery life from the mobile.

# Thermal Status

Processing limitations & thermal status at the mobile station represent useful information at the base station.  This information can be used by the base station to schedule transmissions, anticipate mobile responses, & transmission rates.  This enables the base station to reduce the impact of thermal state on mobile station, & to adapt the system performance around the mobile station status -- avoiding and efficient resource allocation.

# Proposed Paging Design Considerations

This contribution proposes that the mobile station reports its thermal status, which affects its processing capabilities.

# Proposed Text for SDD

Insert the following text into Idle Mode Paging Design sub-clause (i.e. Chapter 6.4 in [3]):

-------------------------------  Text Start  ---------------------------------------------------

6.4.x.x  Paging Interval Design

The mobile station can report its battery life, e.g. when the battery life.  When the network receives this mobile battery life report it should set the paging interval appropriately based on this battery life information.

This avoids higher battery drain rate when the MSS is low on battery, and not plugged into a fixed power source.  This also avoids additional latency (and system paging load) when the mobile has plenty of battery life or is plugged into a fixed power source.

The mobile station reports its thermal status, which affects its processing capabilities to enable the base station to appropriately adapted its messaging exchanges with the mobile station.

-------------------------------  Text End  ---------------------------------------------------

# References

[1] IEEE 802.16m-08/003r1, “The Draft IEEE 802.16m System Description Document”