| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Paging Group Design |  |
| Date Submitted | 2008-09-05 |  |
| Source(s) | John Harris, Stavros Tzavidas, Hua Xu, Pranav Joshi Motorola | Voice: +1 847-632-4042 , +1 847-632-4313, +1 847-632-2176 E-mail: John.Harris@Motorola.com stavros.tzavidas@motorola.com Hua.Xu@motorola.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | MAC: Idle Mode; in response to the TGm Call for Contributions and Comments 802.16m-08/003r4 for Session 57 |  |
| Abstract | Proposal for IEEE 802.16m support of paging group design . |  |
| Purpose | Review and adopt. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Paging Group Design

John Harris, Stavros Tzavidas, Hua Xu, Pranav Joshi

Motorola

# Introduction & Background

The paging wake-up time (paging interval and paging offset) and paging area should be set in a way that reduces paging delay, & paging load, while also benefiting battery life.

Mobiles are typically found with a high probability at the last known cell or sub paging area, where it performed a routing area update or ended its last call.  From field statistics, a mobile can be found in the same cell as was last observed in, approximately 50% to 90% of the time.

Based on this, it is desirable to page the mobile in a sequential fashion. However, normal sequential paging may result in a longer paging delay when the mobile is not present in the area in which it is paged first. As a result, it is important to design a paging scheme that provides the benefits of reduced paging load offered by sequential paging without introducing considerable delay. However, this delay can be avoided by configuring a Primary and Secondary Paging Group. This contribution introduces such a scheme.

# Proposal

When a call arrives for the mobile, the mobile is initially paged only in its Primary Paging Group (e.g., near its last known cell) as shown in Figure 1.  If there is no response, the mobile is paged on the second paging group in the remaining cells of the paging area.  The Secondary Paging Group offset is configured to occur a finite time duration T (e.g., 100 ms) after the Primary Paging Group offset. The value of T includes the timeout required at the last known cell to figure out that the mobile has not responded.

Fig. 1: Illustration of cell coverage by Primary & Secondary Paging Groups

This contribution proposes that it be possible for the system to assign a mobile into a Primary Paging Group (HighPriorityGroup) and a Secondary Paging Group (LowPriorityGroup). If the mobile is under a cell within either or both of these paging groups then it does not need to perform a routing area update.

Generally, the network will configure the Primary Paging Group to have the same paging interval is the Secondary Paging Group, but the Primary Paging Group will have a paging offset just prior to the paging offset for the Secondary Paging Group. When the network attempts to page the mobile, it will begin by paging the Primary Paging Group.  If no response is received, then it can immediately page the Secondary Paging Group. This enables avoiding page messages in the cells covered by the Secondary Paging Group, in the case where the mobile is under a cell where the Primary Paging Group is configured.

The mobile will monitor the highest priority group, which is configured in the cell in which it resides.  If the mobile is under a cell with the Secondary Paging Group, but not the Primary Paging Group, then it should monitor the Secondary Paging Group for pages. If the mobile is under a cell within the Primary Paging Group, then it should monitor the Primary Paging Group for pages to it.

Because the value of offset between the Primary Paging Group and the Secondary Paging Group is expected to be small, the paging delay introduced due to this sequential paging is not significant.

From field statistics, at the time of a page to the mobile, the mobile can be found in the same cell where the mobile was last observed, between ~50% to 90% of the time.  This yields that the paging load is reduced by ~5x; or 1 / (P*S +(1-P)*1) where:

- P = probability mobile is still in the last known area

- S =  number of cells in last known area / number of cells in overall paging area

Insert the following text into Idle Mode Paging Design sub-clause (i.e. Chapter 6.4 in [3]):

-------------------------------  Text Start  ---------------------------------------------------

6.4.x.x  Paging Group Design

The mobile can be configured into both a Primary Paging Group (HighPriorityGroup) and a Secondary Paging Group (LowPriorityGroup). If the mobile is under a cell within either or both of these paging groups then it does not need to perform a routing area update. The mobile will monitor the highest priority group, which is configured in the cell in which it resides.

-------------------------------  Text End  ---------------------------------------------------

# References

[1] IEEE 802.16m-08/003r1, “The Draft IEEE 802.16m System Description Document”